3GPP TSG-RAN WG4 Meeting #116	R4-R4-2509623

Bengaluru, India, August 25 – 29, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for on-demand SSB SCell operation

Agenda item:	7.23.3.1

Document for:	Discussion

- Introduction

At RAN4#115 meeting, the R19 WI on network energy saving was further discussed and the agreements were captured in the WF [1]. In this contribution, we continue the discussion on the RRM impact from on-demand SSB SCell operation.

- OD-SSB based deactivated SCell measurement (Case 1)

In previous meetings, the UE measurement behaviour on the deactivated OD-SSB SCell was concluded. In general, a UE is expected to perform a fast measurement based on OD-SSB periodicity within a fast mode window (FMW) when OD-SSB is activated. Afterwards, UE would fall back to legacy deactivated SCell measurement for UE power saving. We will continue discussing the leftover issues in this section.

  - OD-SSB based deactivated SCell measurement in FMW

One open issue to be discussed is how to handle the collision between OD-SSBs in FMW and measurement gaps.

Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

In our understanding, measurement gaps are configured as a static pattern and are not interrupted by other measurement procedures. To minimize the impact on UE implementation, UE can be allowed to skip the OD-SSB measurement if FMW collides with MG occasions.

- Prefer Option 1, UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

As the OD-SSB measurement may be skipped due to conflict with MG occasions, the fast measurement window needs to be extended to make sure UE is able to obtain a valid measurement result based on OD-SSB. In existing measurement delay requirement, the impact from measurement gaps is reflected by adding a scaling factor Kp as cited below. It is defined based on the measurement periodicity i.e. SMTC and MGRP. However, for the fast measurement window, it does not occur periodically. Instead it is an one-shot measurement window once OD-SSB is activated hence Kp cannot be used to scale the FWM. To account the impact, we can extend the FWM by the number of OD-SSBs that are overlapping with the MG occasions. Since the OD-SSB and MG configurations are up to network, both network and UE are able to know the length of FMW after extension.

TS 38.133 clause 9.2.5.1:

When intra-frequency SMTC is fully non overlapping with measurement gaps or NCSG, or intra-frequency SMTC is fully overlapping with MGs or NCSG, Kp=1

When intra-frequency SMTC is partially overlapping with measurement gaps, Kp = 1/(1- (SMTC period /MGRP)), where SMTC period < MGRP. When intra-frequency SMTC is partially overlapping with NCSG, Kp = 1/(1- (SMTC period /VIRP)), where SMTC period < VIRP.

- FMW shall be extended due to collision with MGs to make sure UE is able to obtain a valid measurement result based on OD-SSB.

Observation 1: The scaling factor Kp cannot be used to scale the FWM as the measurement in FWM is an one-shot measurement window once OD-SSB is activated.

- To extend the FWM by the number of OD-SSBs that are overlapping with the MG occasions.

  - OD-SSB based deactivated SCell measurement - procedure

Regarding to the OD-SSB reactivation, there is only one open issue i.e.  if the UE shall start FMW after T2 if OD-SSB is further activated (i.e. reactivated) within T2.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure - Background - The deactivated SCell measurement requirement in FR1 is as follow. - DRX cycle; - T SSB_measurement_period_intra. No DRX; Ceil(5 x Kp) x measCycleSCell x CSSFintra. DRX cycle≤ 320ms; Ceil(5 x Kp) x max(measCycleSCell, 1.5xDRX cycle) x CSSFintra. DRX cycle> 320ms; Ceil(5 x Kp) x max(measCycleSCell, DRX cycle) x CSSFintra. - The OD-SSB based deactivated SCell measurement procedure is shown as follow. Sub-issue 1: The start point of T2 Agreement: - The time point of OD-SSB deactivation. Sub-issue 2: The minimum interval(T2) between two FMWs Agreement: - For FR1, 5*max (measCycleSCell, DRX cycles) - For FR2, 4s for power class 1/5 and 3s for power class 2/3/4 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Agreement: - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

The reason for introducing T2 is to allow UE power saving so that UE is not required to perform fast measurements if OD-SSB is again activated within T2 after OD-SSB was deactivated. As the UE has acquired a valid measurement result when OD-SSB was previously activated, and T2 ensures the measurement result/report remains valid when OD-SSB is reactivated, there is no urgency for UE to perform fast measurement after T2. In addition, the measurement result from fast measurement would not make difference from the measurement result derived from legacy deactivated measurement. On the other hand, if UE triggers fast measurement after T2, both network and UE would have to maintain the timing how long OD-SSB has been deactivated and when T2 is supposed to expire. This would increase the implementation efforts for OD-SSB SCell. Hence, we see it sufficient for UE to follow legacy deactivated SCell measurement after T2, if OD-SSB reactivation/reconfiguration occurs within T2.

Observation 2: The UE has acquired a valid measurement result when OD-SSB was previously activated, and T2 ensures the measurement result/report remains valid when OD-SSB is reactivated.

Observation 3: After T2, the measurement result from fast measurement would not make difference from the measurement result derived from legacy deactivated measurement.

- If OD-SSB reactivation/reconfiguration occurs within T2, UE performs legacy deactivated SCell measurement after T2.

- OD-SSB based deactivated SCell measurement (Case 2)

Referring to Fig.1, RAN1 further agreed to support Alt Time-C2 and scenario 3 in addition to Alt Time-C1. It is still open whether and how to define the measurement requirements for these scenarios.

Fig. 1 OD-SSB SCell operation in Case 2 and Scenario #2/#2A

Measurement requirement in FMW for scenario Alt Time – C2

Regarding to the scenario Alt Time-C2, there were different options as cited below. In our view, it differs from Alt Time-C1 in that the union of AO-SSB transmission and OD-SSB transmission has an unevenly distributed time domain pattern. During the discussion, some UE vendors argued they may not be able to handle the different time distances hence not all the SSBs can be measured. In this case, we can consider relaxing the measurement requirement in FMW by using the OD-SSB periodicity instead of the effective measurement periodicity considering both AO-SSB and OD-SSB. It is up to UE to determine which SSB to be measured but the measurement requirement is defined based on OD-SSB periodicity. Hence, Option 1 is preferred.

Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion.

- For the measurement in FMW in scenario Alt Time-C2, RAN4 to define the delay requirement only based on the OD-SSB periodicity.

Measurement requirement in FMW for Scenario 3 – AO-SSB and OD-SSB on different frequencies

In last meeting, RAN2 confirmed to introduce a new servingCellMO for OD-SSB (i.e. servingCellMO-OD) in addition to the legacy servingCellMO for AO-SSB. They further concluded that UE uses servingCellMO-OD to measure serving cell when OD-SSB is activated and uses servingCellMO-AO otherwise.

RAN2#130 agreements: - When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.​ - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.​ - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.​ - RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.​ - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.​ RAN1 reply LS (R1-2501633): RAN1 discussed the relation in terms of time location between the always-on SSB and on-demand SSB and agreed the following in RAN1#120. Agreement - For the case when the center frequency locations of always-on SSB and on-demand SSB are different, - Alt Time-C: RAN1 specification has no restriction with regards to overlapping - UE assumes that frequency resources of always-on SSB are not overlapped with those of on-demand SSB in frequency domain. - AO-SSB and OD-SSB are located in the same BWP - FFS: PBCH payload for the same SSB index (other than SFN index, half frame index) should be the same for AO-SSB and OD-SSB - NOTE: AO-SSB periodicity is not adapted RAN1#120 Agreement: Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability Note: UE is not required to measure both AO-SSB and OD-SSB

Following RAN2 agreements, we understood that UE is assumed to perform measurement on one of the servingCellMOs for measuring the serving Cell. Besides, RAN1 agreed UE is not required to measure both AO-SSB and OD-SSB when OD-SSB is activated. To simplify the discussion, we can assume UE is required to measure only OD-SSB corresponding to the servingCellMO-OD when OD-SSB is activated in FMW. Hence for the deactivated SCell measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

Observation 4: UE is assumed to perform measurement on one of the servingCellMOs for measuring the serving Cell.

- For the deactivated SCell measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

Meanwhile, there was discussion if UE shall perform neighbor cell measurements on both carriers indicated by servingCellMO-OD and servingCellMO-AO. In previous RAN4 meetings, it was agreed to follow legacy neighbor cell measurements for Case 1. In Scenario 3, RAN1 has clarified that AO-SSB is assumed to be CD-SSB and OD-SSB is NCD-SSB. As AO-SSB is on sync raster, UE shall perform legacy measurements on the carrier. Hence UE shall perform neighbor cell measurements at least on the carrier indicated by servingCellMO-AO. And the measurement requirement for neighbor cell measurement is defined based on the SMTC associated with the corresponding MO as in legacy. While for the carrier indicated by servingCellMO-OD, UE will perform serving cell measurement on this carrier only if OD-SSB is activated, hence it may not have to perform neighbor cell measurement on the carrier i.e. UE is not required to perform neighbor cell measurement on carrier indicated by servingCellMO-OD. We are open to discuss this issue.

- For scenario 3, UE shall perform neighbor cell measurements at least on the carrier indicated by servingCellMO-AO.

- For scenario 3, UE is not required to perform neighbor cell measurement on carrier indicated by servingCellMO-OD.

- OD-SSB based SCell activation

Known/unknown condition

On SCell activation delay, the following was captured in the way forward. It is FFS if the above agreement also applies to Alt Time -C2 and scenario 3.

Issue 1-3-1: The known cell condition in Case 2 Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. - FFS whether apply to Alt Time-C2, if the requirement is defined. - FFS whether apply to scenario 3, if the requirement is defined. - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

As discussed before, for the scenario where AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, as the known condition is based on the measurement reporting for the SCell, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB and the SCell shall be activated as a known SCell.

Observation 5: If AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, the known condition is based on the measurement reporting for the SCell on the servingMO, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB.

Even if the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, UE is not required to measure both SSBs according to RAN1. Since they are within the same BWP, we can assume there is not much difference between the measurements based on AO-SSBs and OD-SSBs. Although separate measurement reportings are configured for AO-SSB and OD-SSB, both of the measurement reports can be used to determine the known condition to facilitate the SCell activation. Hence the known condition can be determined based on either AO-SSB based measurement reporting or OD-SSB based measurement reporting.

Observation 6: If the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, separate measurement reportings may be triggered for AO-SSB and OD-SSB, but both of the measurement reports can be used to determine the known condition to facilitate the SCell activation.

In summary, for the OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

- For OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

- General OD-SSB requirements

  - Multiple OD-SSB based SCell scenarios

In last meeting, the following was agreed to simply the multiple SCell scenario. It was agreed OD-SSB is activated on one of the multiple OD-SSB SCells in deactivated SCell stage. Hence, for the deactivated SCell measurement, the measurement requirement shall be the same as the measurement requirement for single OD-SSB based SCell.

Issue 1-4-1: OD-SSB based multiple SCells activation Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

Observation 7: In case of multiple OD-SSB SCells, it is assumed OD-SSB is activated only on one of the multiple OD-SSB SCells in deactivated SCell stage.

- For deactivated SCell measurement, the measurement requirement shall be the same as the measurement requirement for single OD-SSB based SCell.

Under this assumption, OD-SSB will be activated only on one of the multiple OD-SSB SCells when UE receives SCell activation to activate multiple OD-SSB SCells. This is conflicting with the tentative agreements “OD-SSB is assumed to be activated on all multiple SCells to be activated before SCell activation command” in the brackets. That is, under the agreement, it is not possible to activate multiple OD-SSB SCells where OD-SSB are all activated. To be aligned with the agreed scope, we can assume only one OD-SSB is activated when activating the SCell hence do not consider the case of multiple SCell activation.

Observation 8: The tentative agreements in brackets “OD-SSB is assumed to be activated on all multiple SCells to be activated before SCell activation command” is conflicting with the agreed scope.

- To assume only one OD-SSB is activated when activating the SCell hence do not consider multiple SCell activation.

  - L3 measurement after SCell is activated

Alt. Time-C1 scenario:

In this scenario, RAN4 agreed to define the measurement requirements based on the effective measurement periodicity (EMP) which is half of the OD-SSB periodicity. But this only applies to the deactivated SCell measurements in FMW. It is FFS how to define the L3 measurement requirement after the SCell is activated.

RAN2#130 agreements: - When the AO-SSB and OD-SSB have the same frequency, they share the same servingCellMO.​ - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured.​ - RAN2 confirms the working consumption that when AO-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB, as agreement.​ - RAN2 understands multiple OD-SSBs with the different frequencies for a given SCell is not supported.​ - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.​ - RAN2 will leave whether to introduce OD-SSB MAC CE triggered MR or not to RAN4. RAN4#114: Issue 2-3: Combine OD-SSB and AO-SSB in measurement Agreement: - In scenario 4, the measurement requirement is defined based on the effective measurement periodicity. - The requirement is only applied in the FMW which includes both OD-SSB and AO-SSB, and - The effective measurement periodicity is half of OD-SSB periodicity - AO-SSB and OD-SSB are on the same frequency with the same spatial relation RAN4#114bis: Issue 1-2-2: Requirement for RAN1 Alt Time-C1 Agreement: - RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario. - The EMP is the periodicity of the union of AO-SSB and OD-SSB and equals one of legacy SSB periodicities defined in the specification. - FFS for other procedures.

In last RAN2 meeting, RAN2 agreed the UE applies OD-SSB specific SMTC when OD-SSB is activated and SCell is activated. In our view, after SCell is activated, UE has likely acquired a valid measurement result based on either OD-SSB or AO-SSB. There is no urgency for UE to perform fast measurement after SCell is activated. In addition, the OD-SSB specific SMTC is up to network configuration, which can be configured to allow UE measure both AO-SSB and OD-SSB if needed. Hence it is sufficient to define the L3 measurement requirement in line with RAN2 agreement i.e. After SCell is activated. the L3 measurement requirement is defined based on OD-SSB specific SMTC when OD-SSB is activated and based on AO-SSB specific SMTC if OD-SSB is deactivated.

Observation 9: RAN4 agreed to define the measurement requirements based on the effective measurement periodicity (EMP) which only applies to the deactivated SCell measurements in FMW.

- After SCell is activated, the L3 measurement requirement is defined based on OD-SSB specific SMTC when OD-SSB is activated and based on AO-SSB specific SMTC if OD-SSB is deactivated.

  - Processing time

Additional processing time for OD-SSB parameter update

In last meeting, an additional processing time was agreed with values of 2ms or 5ms based on UE capability. We believe the same value can be applied to other procedures where additional processing time is needed. Hence option 1 is preferred.

Issue 1-4-4: Additional Processing time for OD-SSB parameter update Agreement - Option 1: RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates. - The additional processing time is 5ms or 2ms based on UE capability reporting. - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Option 2: When NW updates the OD-SSB parameters for an active Scell by MAC-CE, additional 5ms processing time is applied on top of THARQ+3ms after UE receiving the MAC-CE. - The additional processing time is 5ms - Note 1: The additional processing time is only applied to L1 and L3 measurement transition period [after the SCell is activated]. - Note 2: No capability is needed

- RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates i.e. based on the 2ms or 5ms processing time up to UE capability indication.

Processing time for OD-SSB deactivation

In last meeting, RAN1 sent LS to RAN4 asking for confirmation on their working assumption regarding to the timeline when MAC-based OD-SSB deactivation is received. This issue was also raised by other companies in RAN4.

Issue 1-4-5: RAN1 LS - OD-SSB deactivation timeline Agreement - RAN4 to confirm RAN1’s working assumption about OD-SSB deactivation processing time. Further discuss the wording in the reply LS. - Further discuss in RAN4 the UE additional processing time for OD-SSB deactivation. - Option 1: No UE additional processing time for OD-SSB deactivation - Option 2: Follow the same values as UE additional processing time for OD-SSB activation

Different from the OD-SSB activation, we don’t think additional processing time is needed upon OD-SSB deactivation. The reason why additional processing time was introduced is the time for UE to get ready to receive the OD-SSB which are activated irregularly. However, this is not required when receiving OD-SSB deactivation. UE would be expected to release the resources for monitoring OD-SSB which should not count additional time. Therefore, the additional processing time is not needed upon OD-SSB deactivation.

- Additional processing time is not needed upon OD-SSB deactivation.

  - Interruption

In last meeting, it was agreed to introduce new interruption at OD-SSB activation/deactivation. Another aspect to be discussed is impact on existing interruption requirement due to OD-SSB.

In existing specification, interruption is allowed on active serving cells due to RF tuning during some RRM procedures e.g.

- 8.2.2.2.1 Interruptions at SCell addition/release: This defines the maximum allowed interruption during the RRC reconfiguration procedure for SCell addition/release. As UE needs to monitor SSB

- 8.2.2.2.3 Interruptions during measurements on deactivated SCC:  This defines the maximum allowed interruption due to measurements when an SCell is deactivated

In both clauses above, UE needs to tune on its RF in order to monitor SSBs for identifying or measuring the SCell. However, if the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted. Hence no interruption is allowed before OD-SSB is indicated when UE is commanded to add/release an OD-SSB SCell.

If the SCell is in OD-SSB operation in Case 2 i.e. with always-on SSB, UE needs to tune the RF in order to receive at least always-on SSB. Existing interruption requirement can still apply. In any case, the interruption requirements during SCell addition/release need to be updated at least for OD-SSB SCell in Case 1.

Observation 10: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

Similarly, interruption shall not be allowed during the measurements on deactivated SCC before OD-SSB is indicated. This applies if the SCell is in OD-SSB operation in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

  - L1 measurement in OD-SSB SCell

Another issue is L1 measurement in OD-SSB SCell in Case 2 and the scheduling restriction and measurement restriction. The following has been agreed pending on the RAN1 agreement on L1 measurement in Case 2.

ssue 1-2-1: Requirement for RAN1 Alt Time-C1 Agreement Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation. Issue 1-2-5: Scheduling restriction for L1 measurement after SCell activation Agreement - In Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions if UE only supports OD-SSB based L1-RSRP measurement. - The scheduling restriction shall be applied to both AO-SSB and OD-SSB if the requirement is defined based on both of them.

We understand this depends on how the UE is expected to perform L1 measurement after SCell activation. If AO-SSB and OD-SSB are on the same frequency, RAN1 agreed which SSB to measure is up to UE implementation as long as RAN4 requirements are fulfilled without differentiating Alt Time-C1 from Alt Time-C2. As we agreed to follow legacy requirement for the L1 measurement in Case 1, the same principle can apply to Case 2. The question in Case 2 is which SSB periodicity is used to determine the L1 measurement period in addition to DRX cycle. In our view, we can define the L1 measurement period in the similar way as L3 measurement in respective scenarios. That is,

- Alt Time-C1: define the L1 measurement requirements based on union of AO-SSB and OD-SSB and DRX cycle (agreed in RAN4#115 meeting).

- Alt Time-C2: define the L1 measurement requirements based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: define the L1 measurement requirements based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- For L1 measurement in OD-SSB SCell in Case 2, the L1 measurement requirement is defined as the following:

- Alt Time-C2: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

With the measurement requirement above, no scheduling restriction or measurement restriction is allowed for AO-SSB occasions if UE is only required to measure OD-SSB based L1-RSRP measurement when OD-SSB is activated. Hence for Alt Time-C2 and Scenario 3 in Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions when OD-SSB is activated.

- For Alt Time-C2 and Scenario 3 in Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions when OD-SSB is activated.

- Conclusion

In this contribution, we discussed the RRM requirements for OD-SSB SCell operation based on RAN1/2 progress and presented our views on the potential RAN4 impact. The following observations and proposals are made:

- Prefer Option 1, UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion.

- FMW shall be extended due to collision with MGs to make sure UE is able to obtain a valid measurement result based on OD-SSB.

Observation 1: The scaling factor Kp cannot be used to scale the FWM as the measurement in FWM is an one-shot measurement window once OD-SSB is activated.

- To extend the FWM by the number of OD-SSBs that are overlapping with the MG occasions

Observation 2: The UE has acquired a valid measurement result when OD-SSB was previously activated, and T2 ensures the measurement result/report remains valid when OD-SSB is reactivated.

Observation 3: After T2, the measurement result from fast measurement would not make difference from the measurement result derived from legacy deactivated measurement.

- If OD-SSB reactivation/reconfiguration occurs within T2, UE performs legacy deactivated SCell measurement after T2.

- For the measurement in FMW in scenario Alt Time-C2, RAN4 to define the delay requirement only based on the OD-SSB periodicity.

Observation 4: UE is assumed to perform measurement on one of the servingCellMOs for measuring the serving Cell.

- For the deactivated SCell measurement in FMW in scenario 3, the measurement requirement is defined based on OD-SSB periodicity.

- For scenario 3, UE shall perform neighbor cell measurements at least on the carrier indicated by servingCellMO-AO.

- For scenario 3, UE is not required to perform neighbor cell measurement on carrier indicated by servingCellMO-OD.

Observation 5: If AO-SSB and OD-SSB are on the same frequency i.e. Alt Time-C1 and Alt Time-C2, the known condition is based on the measurement reporting for the SCell on the servingMO, it does not matter the known condition is determined based on always-on SSB and/or OD-SSB.

Observation 6: If the AO-SSB and OD-SSB are on different frequencies i.e. scenario 3, separate measurement reportings may be triggered for AO-SSB and OD-SSB, but both of the measurement reports can be used to determine the known condition to facilitate the SCell activation.

- For OD-SSB based SCell activation in all the Case 2 scenarios, the known condition is determined based on the measurement reporting for the SCell irrespective of it is measured on AO-SSB or OD-SSB.

Observation 7: In case of multiple OD-SSB SCells, it is assumed OD-SSB is activated only on one of the multiple OD-SSB SCells in deactivated SCell stage.

- For deactivated SCell measurement, the measurement requirement shall be the same as the measurement requirement for single OD-SSB based SCell.

Observation 8: The tentative agreements in brackets “OD-SSB is assumed to be activated on all multiple SCells to be activated before SCell activation command” is conflicting with the agreed scope.

- To assume only one OD-SSB is activated when activating the SCell hence do not consider multiple SCell activation.

Observation 9: RAN4 agreed to define the measurement requirements based on the effective measurement periodicity (EMP) which only applies to the deactivated SCell measurements in FMW.

- After SCell is activated, the L3 measurement requirement is defined based on OD-SSB specific SMTC when OD-SSB is activated and based on AO-SSB specific SMTC if OD-SSB is deactivated.

- RAN4 to apply the same timeline for MAC CE based OD-SSB activation and MAC CE based OD-SSB transmission parameter updates i.e. based on the 2ms or 5ms processing time up to UE capability indication.

- Additional processing time is not needed upon OD-SSB deactivation.

Observation 10: If the SCell is in OD-SSB operation in Case 1 i.e. without always-on SSB, UE may not need to switch on the RF immediately as there is no SSB being transmitted.

- No interruption is allowed during the RRC reconfiguration procedure for SCell addition/release before OD-SSB is indicated in Case 1.

- No interruption is allowed during measurements on deactivated SCC before OD-SSB is indicated in Case 1.

- For L1 measurement in OD-SSB SCell in Case 2, the L1 measurement requirement is defined as the following:

- Alt Time-C2: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- Scenario 3: based on OD-SSB and DRX cycle if OD-SSB is activated, otherwise based on AO-SSB and DRX cycle.

- For Alt Time-C2 and Scenario 3 in Case 2, no scheduling restriction and measurement restriction is expected for AO-SSB occasions when OD-SSB is activated.

- References

[1]	R4-2508277, “WF on Netw_Energy_NR_enh”, Ericsson