3GPP TSG-RAN4 Meeting # 116	 R4-2509631

# Bangaluru, India,  25th – 29rd August, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft CR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for minimum requirement for L1 indication for event trigger Reporting based on L1-RSRP measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MIMO_Ph5-Core |  |  |  |  |  | Date: |  |  | 2025-08-14 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The requirements for minimum requirement for L1 indication for event trigger reporting based on L1-RSRP measurements was not defined. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The requirements for minimum requirement for L1 indication for event trigger reporting based on L1-RSRP measurements applicable when eventDetectionTimeWindowLength is configured were added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The core requirements for MIMO phase 5 will be missed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.533 |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

-------------- Start of Change <1> --------------

8.x.x	Minimum requirement for L1 indication for event trigger Reporting based on L1-RSRP measurements

The requirement is applicable when a UE is configured with a CSI-ReportConfig with eventType for periodic reference signals with the same periodicity configured by the newBeamResourceSet, with eventDetectionTimeWindowLength, and with dl-OrJointTCI-StateList.

When the event instance is determined as specified in clause 5.2.1.5.4 in TS 38.213[3] layer 1 of the UE shall send an event instance indication to the higher layers.

The event instance evaluation for the RS resources shall be performed as specified in clause 5.2.1.5.4 in TS 38.213[3]. Two successive indications from layer 1 shall be separated by at least TIndication_interval_UEIBR.

When DRX is not used, TIndication_interval_UEIBR is max(1ms, β*TSSB-RS,UEIBR) ) or max(1ms, β*TCSI-RS,UEIBR), where TSSB-RS,UEIBR and TCSI-RS,UEIBR is the periodicity of the SSB or CSI-RS for event instance determination, and β=2 if current beam and new beams are overlapped on the same symbol for eventType of ‘event2’ or ‘event7’, otherwise β=1.

When DRX is used, for SSB based event instance determination,

-	TIndication_interval_UEIBR = Max(1.5 × DRX_cycle_length, K*1.5 × TSSB-RS,UEIBR), if DRX_cycle_length ≤ 320 ms,

-	TIndication_interval_UEIBR = DRX_cycle_length, if DRX_cycle_length > 320 ms.

When DRX is used, for CSI-RS based event instance determination,

-	TIndication_interval_UEIBR = Max(1.5 × DRX_cycle_length, K*1.5 × TCSI-RS,UEIBR), if DRX_cycle_length ≤ 320 ms,

-	TIndication_interval_UEIBR = DRX_cycle_length, if DRX_cycle_length > 320 ms.

For deactivated PSCell,

when DRX is not used, TIndication_interval_UEIBR is Max (1 ms, measCyclePSCell).

when DRX is used, for SSB based event instance determination,

-	TIndication_interval_ UEIBR = Max (1.5 × DRX_cycle_length, 1.5 × measCyclePSCell), if DRX_cycle_length ≤ 320 ms,

-	TIndication_interval_ UEIBR = Max (DRX_cycle_length, measCyclePSCell), if DRX_cycle_length > 320 ms.

When DRX is used, for CSI-RS based event instance determination,

-	TIndication_interval_ UEIBR = Max (1.5 × DRX_cycle_length, 1.5 × measCyclePSCell), if DRX_cycle_length ≤ 320 ms,

-	TIndication_interval_ UEIBR = Max (DRX_cycle_length, measCyclePSCell), if DRX_cycle_length > 320 ms.

-------------- End of Change <1> --------------