3GPP TSG-RAN WG4 Meeting #116	R4-2509637

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM requirements for FR2-1 L3 measurement delay reduction

Agenda item:	7.14.3.1

Document for:	Discussion

Introduction

In this contritubtion, we provide our views on remaining open issue on L3 measurement delay reduction with multi-Rx simultaneous reception based on [1].

Discussion

Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”

In the last meeting, RAN4 agreed two quitting conditions for FBS below, but still open whether to define re-evaluation mechanism.

Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Way forward: For re-evaluation mechanism for scenario 1/2/3/4: - Check whether the following mechanism is agreeable: - UE re-evaluates the activation condition after the timer expires, where the timer starts after UE quits from Multi-Rx mode.

For time-based quitting FBS, UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition. Since the UE needs to perform at least one complete FBS based measurement period, T1 should be considered at least based on Tidentify time, and Tidentify could be measurement period or cell identification period.

- Proposal 1: T1 for timer-based deactivation should be at least Tidentify time, and Tidentify could be measurement period or cell identification period.

For the re-evaluation for FBS re-actiation, UE re-evalutes the activation condition after time T2 following the completion of T1. In our understanding for overall procedure of FBS operation, even if the UE fall back to normal measurement behavior after T1, the UE continuously measures serving cell’s signal quality. If the signal quality has not changed significantly, the UE does not need to re-acitve FBS mode. Otherwise, the UE can re-active FBS mode as long as the activation condition is met. And even if the re-evaluation mechanism is introduced, it is difficult to define T2 since T2 would depends on UE status (e.g., mobility). Therefore, it is better to leave UE implementation whether to re-active FBS mode.

- Proposal 2: Prefer not to define re-evaluation mechanism for FBS re-activation

Figure 1 FBS activation, deactivation, and re-evaluation

Conclusion

In this contribution, we provide views on remaining open issues for multi-Rx simultaneous reception, and we propose

- Proposal 1: T1 for timer-based deactivation should be at least Tidentify time, and Tidentify could be measurement period or cell identification period.

- Proposal 2: Prefer not to define re-evaluation mechanism for FBS re-activation

Reference

[1] R4-2508266, “WF on RRM requirements for NR_RRM_Ph5_Part2,” CATT.