3GPP TSG-RAN WG4 Meeting #116	R4-2509645

Bengaluru, India, August 25th – 29th, 2025

Source:	Tejas Networks

Title:	Discussion on testability and interoperability for AI/ML mobility

Agenda item:	7.19.4

Document for:	Discussion

Introduction

The discussions on AI/ML mobility in NR were continued in RAN4#115 in the scope of Rel-19 SI. Agreements achieved during this meeting are summarized in the WF [1].

Discussion

During the RAN4 #112bis meeting, RAN4 agreed to study aspects related to NR AIML mobility requirements. In this paper, we provide our views on testability and interoperability issues in RAN4 for AI/ML based mobility.

Testing setup for inter-frequency prediction

RAN4#114bis agreements: - RAN4 to consider two cells: serving cell and another/target cell for intra-cell RRM measurement prediction/event prediction. - FFS: RAN4 to study if more than 2 cells are needed for inter-cell RRM measurement prediction/event prediction. - RAN4 to study the factors potentially impacting RSRP prediction accuracy for inter-frequency prediction scenario considering at least the following aspects: - the side condition of frequency prediction (e.g., EPRE difference) - the impact of correlation coefficient between measured and predicted frequency layers - impact of cluster approach, e.g., - When measurement from single cell in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency. - When measurement from a group of cells in one carrier frequency is used by the UE as an input to predict the RRM measurement for the intra-FR and co-located cell in another carrier frequency.

In the FR1-to-FR1 inter-frequency prediction scenario (Scenario 4), the AIML model is expected to predict the measurements for one cell using measurements from another cell. Based on the RAN2 agreement, the target cell for prediction should be an inter-frequency cell that is co-located with one of the input cells. It is essential to ensure that both measurement and prediction reflect comparable radio conditions. Synchronization between the timing of the measurement in cell 1 and prediction for cell 2 may be required. Furthermore, it would avoid temporal variations caused by the user mobility and channel variations, enabling a fair evaluation of prediction accuracy.

Observation 1: Aligning the time of measurement and prediction is required to avoid temporal variations and ensure fair prediction accuracy.

Proposal 1: For scenario 4, it should be clarified whether synchronization between the timing of measurement and prediction is required for testing.

As indicated in the existing RAN2 study, prediction accuracy is influenced by the correlation between the frequency layers. Therefore, when developing the channel model for RAN4 testing, it is important to ensure that the setup reflects realistic deployment scenarios.

Proposal 2: The test setup should closely reflect the deployment conditions and inter-frequency correlation characteristics.

Testing goal

Agreement: - Discuss whether to further clarify the testing goal for measurement prediction use case, e.g,, - The testing goal for measurement prediction is to verify: - The reported measured results meet certain requirements under certain conditions - The reported predicted results meet certain requirements under certain conditions - Note: the requirements include core and performance requirements - FFS the relative requirements for the difference between reported measured results and reported predicted results

RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

The study of the use cases is driven mainly by two study goals:

- The 1st study goal is to reduce measurement efforts in temporal, spatial or frequency domain by using predicted measurements.
The following metrics are considered:

  - MRRS	Measurement reduction rate in spatial domain

  - MRRT	Measurement reduction rate in temporal domain

- The 2nd study goal is to improve the handover performance (e.g., Ping-pong HO, HOF/RLF, short time of stay, Handover interruption).

RAN2 prioritized the following scenarios for the RRM measurement prediction study:

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |

For scenarios 2 and 3, the target study goal is to reduce the number of measurements in the temporal and frequency domains. In scenario 2 (temporal domain case B), the number of measurements is reduced by skipping measurements during the observation window. In the inter-frequency case (scenario 3), measurements for a co-located target cell at a different frequency are skipped. In both cases, the accuracy of the predicted measurements for the skipped duration or frequency should be verified. RAN4 should define accuracy requirements to determine whether the predicted measurement results meet the minimum acceptable levels.

For scenario 4 (temporal domain case A), continuous measurement results collected over a period of time (prediction window) are used to predict measurement results for a future interval of time (observation window). In this case, the network can know the future measurements in advance through prediction, potentially reducing the delay associated with the handover procedure. Since the actual measurements for the future time interval are not available to verify accuracy, one of the key testing goals can be to assess the potential reduction in handover delay.

Observation 2: For scenario 2 and 3, the measurements are skipped; accuracy of the skipped measurement should be verified.

Observation 3: For scenario 4, the network knows the future measurements in advance.

Proposal 3: For scenario 4, one of the testing goals should be handover delay.

Ground truth derivation in FR1

Agreement: - Agreement: - Alt 1: Ground truth for FR1 will be based on the transmitted or reception power - Alt 2: Ground truth for FR1 will be based on the reported measurement value under certain conditions - Further discuss the advantages and disadvantages for Alt 1 and Alt 2 considering the aspects such as: - The information at the TE side, on the timing UE performs and/or finishes the measurement and/or prediction - Impact of L3 filtering - Number of samples for L1 filtering - Timing window, if it exists, where UE can’t measure the ground truth in Alt2 - channel condition - Avoid UE cheat in the test - S(I)NR variations in the test at different time instances (for Alt2) - Other aspects are not precluded - Companies are encouraged to provide analysis on all or some of the aspects in the list Agreement: During the SI, RAN4 to provide analysis on potential issue and possible solution to the two Alternatives. If RAN4 cannot make decision during the SI phase, RAN4 will discuss in the WI phase. - At least capture the definition of the two Alternatives in the TR.

During the RAN4 #112bis meeting, the issue of potential UE manipulation in AI-based beam management (BM) tests was discussed. It was observed that since both the predicted measurement and the ground truth are reported by the UE to the test equipment, there is a risk of the UE aligning these values to pass the test.

- Proposal 4: Safeguards should be implemented to prevent UEs from manipulating prediction results to align with ground truth reports, ensuring reliable test outcomes.

Conclusion

This section summarizes our discussion by listing all the observations and proposals discussed in this contribution.

Observation 1: Aligning the time of measurement and prediction is required to avoid temporal variations and ensure fair prediction accuracy.

Observation 2: For scenario 2 and 3, the measurements are skipped; accuracy of the skipped measurement should be verified.

Observation 3: For scenario 4, the network knows the future measurements in advance.

Proposal 1: For scenario 4, it should be clarified whether synchronization between the timing of measurement and prediction is required for testing.

Proposal 2: The test setup should closely reflect the deployment conditions and inter-frequency correlation characteristics.

Proposal 3: For scenario 4, one of the testing goals should be handover delay.

- Proposal 4: Safeguards should be implemented to prevent UEs from manipulating prediction results to align with ground truth reports, ensuring reliable test outcomes.

- References

- R4-2508269, WF on FS_NR_AIML_Mob_Part2 (OPPO), Malta, Malta, May 19th – May 23rd, 2025.

- RP-250970, Status Report to TSG, Prague, Czech Republic, June 9-13, 2025.

- R4-2415968, Work Plan for RAN4 Discussions on AI/ML for Mobility in NR, Nokia, OPPO, MediaTek, Hefei, China, Oct 14 – Oct 18, 2024