3GPP TSG-RAN WG4 Meeting #116	R4-2509653

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.12.3

Source:	MediaTek Inc.

Title:	Simulation results of Spatial Channel Modelling

Document for:	Discussion

# Introduction

In RAN4#115, RAN4 agreed set of alignment simulations for Spatial Channel Modelling [1], [3]. In this contribution, we provide additional simulation results for 32TX-4RX with PMI feedback.

# Simulation results

## 2.1 Simulation parameters

In this section, we provide our simulation setup for Spatial Channel Modelling with 32TX-4RX. In RAN4#113, RAN4 agreed that general simulation configuration can follow existing 38.101-4 8RX TDD requirement configuration specified in Chapter 5.2.4.2.1 [2]. Common simulation parameters are listed in Table 1. We have assumed 0% TxEVM and LMMSE MIMO demodulation.

Table 1: Common test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 for rank <= 4 2 for rank > 4 |
| Codebook configuration | CodebookType |  | typeI-SinglePanel, typeII-r16 |
|  | CodebookMode |  | 1 (for typeI) |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,2) for 32Tx |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |
| CSI-RS for tracking | First OFDM symbol in the PRB used for CSI-RS |  | l0 = 5 for CSI-RS resource 1 and 3 l0 = 9 for CSI-RS resource 2 and 4 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |

Test specific MCS with common system bandwidth, subcarrier spacing, and TDD UL-DL pattern are listed in Table 2. Note that PDSCH data is allocated only in full slots, i.e. not in special slots in our simulations.

Table 2: BW and MCS parameters

| Test | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Test type |
| --- | --- | --- | --- | --- |
| Rank 2 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | PMI |
| Rank 4 | 40 / 30 | 16QAM, 0.48 (MCS13) | FR1.30-1 | PMI |

We have simulated CDL model A4 (truncated) as agreed after RAN#114bis. Our CDL channel model implementation follows TR38.827 randomness reduction. The CDL channel parameters follow [3] and are listed in Table 3.

Table 3: CDL channel parameters

| Carrier frequency | 3.5 GHz |
| --- | --- |
| UE speed | 3 km/h |
| UE travelling direction (fv, qv) | (65°, 90o) |
| Channel profile | 827 UMa CDL-C (A1) |
| Antenna array virtualization for 32TX | AAV Option 3 (2, 8, 2, 1, 1) (Direct passthrough) |
| BS (α, β, γ) | (0°, 10°, 0°) |
| BS polarization slant | (45°, -45o) |
| UE (α, β, γ) | (180°, 0°, 0°) |
| UE polarization slant | (0°, 90o) |
| BS antenna radiation pattern | As defined in TR38.901 Table 7.3-1 |
| UE antenna radiation pattern | Omnidirectional |

## 2.2 Follow-PMI testing in CDL-C

In Figure 1 we show the performance of different PMI reporting options (Type1 WB-PMI, eType2 SB-PMI, and random Type1 with WB-PMI) with MCS13 and 3 km/h UE speed for AAV option 3 and 32TX-4RX. PMI feedback delay is 6.5 ms. For eType2, subband size 8 PRBs and paramCombination-r16 = 6 are used.

As a performance upper bound, per-subband SVD precoding with ideal conditions (ideal DMRS and CSIRS channel knowledge, minimal PMI feedback delay) was also simulated, and the results are plotted in Figure 1 for reference.

Figure 1: 32TX-4RX PMI reporting in CDL-C A4 (left: rank 2, right: rank 4)

In Table 4 we list the required SNR to achieve 90% relative throughput in PMI tests with different PMI reporting options and ranks. As can be seen, eType2 outperforms Type1 by 1dB with rank2 and by 2dB with rank4. Compared to the performance upper bound (ideal SB-SVD), eType2 loses by 2dB with rank2 and by 3dB with rank4.

In Table 5 we list gammas (throughput ratio between follow and random PMI) for both Type1 WB-PMI and eType2 SB-PMI.

Table 4: SNR [dB] at 90% relative tput of 32TX-4RX PMI reporting in CDL-C A4 AAV 3

| CDL-C | Type1 follow WB-PMI | eType2 follow SB-PMI | Type1 random WB-PMI | Ideal SB-SVD |
| --- | --- | --- | --- | --- |
| Rank 2 | -0.7 | -1.7 | 22.8 | -3.8 |
| Rank 4 | 9.1 | 7.1 | 30.5 | 4.1 |

Table 5: Gammas of 32TX-4RX PMI reporting in CDL-C A4 AAV 3

| CDL-C | Type1 WB-PMI | eType2 SB-PMI |
| --- | --- | --- |
| Rank 2 | 11.8 | 16.6 |
| Rank 4 | 3.7 | 5.3 |

# Conclusion

In this paper we provided our simulation setup and results for 32TX-4RX PMI testing in CDL-C A4 channel.

# Reference

- R4-2508776, “Way Forward for [115][321] NR_SCM”, Nokia

- 3GPP TS 38.101-4 V18.8.0 (2025-06), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements

- R4-2508622, “Simulation assumptions for alignment and comparison for NR SCM”, Huawei, MediaTek