3GPP TSG-RAN WG4 Meeting #116	R4-2509658

Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda item:	7.20.6

Source:	MediaTek Inc.

Title:	Simulation results of NR MIMO Phase 5 demodulation and CSI requirements

Document for:	Discussion

# Introduction

In RAN4#115, RAN4 agreed basic parameters for simulations of typeI-SinglePanel-r19 with 64 ports [1]. In this contribution, we provide our simulation results for NR MIMO Phase 5 demodulation and CSI requirements.

# Simulation results

In this Section we will present our simulation results.

## 2.1 Single PMI with 64TX Type I-Single Panel-r19 Codebook

In this section, we present our simulation setup for NR MIMO Phase 5 demodulation and CSI requirements. At RAN4#115, it was agreed that the general simulation configuration can follow the existing 38.101-4 Single PMI with 32TX Type I-Single Panel Codebook requirement configuration, as specified in Sections 6.3.3.1.4 and 6.3.3.2.4 [2]. The test parameters for FDD Single PMI with 64TX Type I-Single Panel-r19 Codebook are listed in Table 1, and the corresponding parameters for TDD are provided in Table 2. Note that the CQI/RI/PMI delays are inherited from the baseline as a starting point.

Table 1: Test parameters of FDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests
(baseline Table 6.3.3.1.4-1 in [2])

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |
| Subcarrier spacing |  | kHz | 15 |  |
| Duplex Mode |  |  | FDD |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) and (16,2) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 5/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0, l1) |  | Row 17,(2, 5) | Row 17,(9, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 8 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 5 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 5) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) and (16,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 9 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.1-6.3 FDD (MCS20) and MCS22 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (1 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

Table 2: Test parameters of TDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests
(baseline Table 6.3.3.2.4-1 in [2])

| Parameter |  | Unit | Test 1 |  |
| --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 40 |  |
| Subcarrier spacing |  | kHz | 30 |  |
| Duplex Mode |  |  | TDD |  |
| TDD DL-UL configurations |  |  | FR1.30-1 as specified in Annex A |  |
| Propagation channel |  |  | TDLA30-5 |  |
| Antenna configuration |  |  | High XP 64 x numRX (N1,N2) = (8,4) and (16,2) |  |
| Beamforming Model |  |  | As specified in Annex B.4.1 |  |
| ZP CSI-RS configuration | CSI-RS resource Type |  | Periodic |  |
|  | Number of CSI-RS ports (X) |  | 4 |  |
|  | CDM Type |  | FD-CDM2 |  |
|  | Density (ρ) |  | 1 |  |
|  | First subcarrier index in the PRB used for CSI-RS (k0) |  | Row 5,(4) |  |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 5,(7) |  |
|  | CSI-RS periodicity and offset | slot | 10/1 |  |
| NZP CSI-RS for CSI acquisition | CSI-RS resource ID |  | Resource #1 | Resource #2 |
|  | CSI-RS resource Type |  | Aperiodic | Aperiodic |
|  | Number of CSI-RS ports (X) |  | 32 | 32 |
|  | CDM Type |  | CDM4 (FD2, TD2) | CDM4 (FD2, TD2) |
|  | Density (ρ) |  | 1 | 1 |
|  | First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3) |  | Row 17,(2, 4, 6, 8) | Row 17,(2, 4, 6, 8) |
|  | First OFDM symbol in the PRB used for CSI-RS (l0) |  | Row 17,(2, 5) | Row 17,(9, 12) |
|  | CSI-RS interval and offset | slot | Not configured | Not configured |
|  | aperiodicTriggeringOffset |  | 0 | 0 |
| CSI-IM configuration | CSI-IM resource Type |  | Aperiodic |  |
|  | CSI-IM RE pattern |  | Pattern 0 |  |
|  | CSI-IM Resource Mapping (kCSI-IM,lCSI-IM) |  | (4,7) |  |
|  | CSI-IM timeConfig interval and offset | slot | Not configured |  |
| ReportConfigType |  |  | Aperiodic |  |
| CQI-table |  |  | Table 1 |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |
| timeRestrictionForChannnelMeasurements |  |  | Not configured |  |
| timeRestrictionForInterferenceMeasurements |  |  | Not configured |  |
| cqi-FormatIndicator |  |  | Wideband |  |
| pmi-FormatIndicator |  |  | Wideband |  |
| Sub-band Size |  | RB | 16 |  |
| csi-ReportingBand |  |  | 1111111 |  |
| CSI-Report interval and offset |  | slot | Not configured |  |
| Aperiodic Report Slot Offset |  |  | 8 |  |
| CSI request |  |  | 1 in slots i, where mod(i, 10) = 1, otherwise it is equal to 0 |  |
| reportTriggerSize |  |  | 1 |  |
| CSI-AperiodicTriggerStateList |  |  | One State with one Associated Report Configuration Associated Report Configuration contains pointers to NZP CSI-RS and CSI-IM |  |
| Codebook configuration | Codebook Type |  | typeI-SinglePanel-r19 |  |
|  | Codebook Mode |  | 1 |  |
|  | (CodebookConfig-N1,CodebookConfig-N2) |  | (8,4) and (16,2) |  |
|  | (CodebookConfig-O1,CodebookConfig-O2) |  | (4,4) |  |
|  | CodebookSubsetRestriction |  | 0x FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF FFFF |  |
|  | RI Restriction |  | 00000010 |  |
| Physical channel for CSI report |  |  | PUSCH |  |
| CQI/RI/PMI delay |  | ms | 7 |  |
| Maximum number of HARQ transmission |  |  | 4 |  |
| Measurement channel |  |  | R.PDSCH.2-8.3 TDD (MCS20) and MCS22 |  |
| PDSCH & PDSCH DMRS Precoding configuration for random Precoding |  |  | Single Panel Type I, Random precoder selection updated per slot, with equal probability of each applicable i1, i2 combination, and with Wideband granularity |  |
| Note 1: When Throughput is measured using random precoder selection, the precoder shall be updated in each slot (0.5 ms granularity) with equal probability of each applicable i1, i2 combination. Note 2: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). Note 3: Randomization of the principle beam direction shall be used as specified in Annex B.2.3.2.3. |  |  |  |  |

In the following Figures 1-4 and Tables 3-6 simulation results of FDD 4RX, FDD 2RX, TDD 4RX, and TDD 2RX are presented with MCS20 and MCS22.

Figure 1: Simulation results of FDD 4RX Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests.

Figure 2: Simulation results of FDD 2RX Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests.

Figure 3: Simulation results of TDD 4RX Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests.

Figure 4: Simulation results of TDD 2RX Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests.

Table 3: Simulation results of FDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests with MCS20

| FDD MCS20 | 4-Rx, 8x4 | 4-Rx, 16x2 | 2-Rx, 8x4 | 2-Rx, 16x2 |
| --- | --- | --- | --- | --- |
| SNR (90%) [dB] | 2.32 | 2.27 | 7.22 | 7.45 |
| Gamma | 22.50 | 30.66 | 15.55 | 19.33 |

Table 4: Simulation results of FDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests with MCS22

| FDD MCS22 | 4-Rx, 8x4 | 4-Rx, 16x2 | 2-Rx, 8x4 | 2-Rx, 16x2 |
| --- | --- | --- | --- | --- |
| SNR (90%) [dB] | 4.80 | 4.77 | 9.95 | 10.08 |
| Gamma | 18.03 | 24.06 | 12.24 | 16.42 |

Table 5: Simulation results of TDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests with MCS20

| TDD MCS20 | 4-Rx, 8x4 | 4-Rx, 16x2 | 2-Rx, 8x4 | 2-Rx, 16x2 |
| --- | --- | --- | --- | --- |
| SNR (90%) [dB] | 2.44 | 2.36 | 7.08 | 7.20 |
| Gamma | 22.48 | 27.07 | 17.54 | 21.05 |

Table 6: Simulation results of TDD Single PMI with 64TX TypeI-SinglePanel-r19 Codebook tests with MCS22

| TDD MCS22 | 4-Rx, 8x4 | 4-Rx, 16x2 | 2-Rx, 8x4 | 2-Rx, 16x2 |
| --- | --- | --- | --- | --- |
| SNR (90%) [dB] | 4.72 | 4.62 | 9.63 | 9.74 |
| Gamma | 18.05 | 24.23 | 13.64 | 18.53 |

# Conclusion

In this paper we provided our simulation setup and results for NR MIMO Phase 5 demodulation and CSI requirements.

# Reference

- R4-2508627, Way Forward for [115][329] NR_MIMO_demod, Samsung

- 3GPP TS 38.101-4 V18.8.0 (2025-06), User Equipment (UE) radio transmission and reception, Part 4: Performance requirements