3GPP TSG-RAN4 Meeting #116	R4-2509671

# Bangalore, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on OTA reference sensitivity level for the Ku band SAN

Agenda item:	7.7.6

Document for:	Discussion and Approval

- Introduction

In this contribution, we want to share some views about OTA reference sensitivity level for the Ku band SAN, which does not have agreement at way forward R4-2504720[1] and R4-2508778 [2].

In designing a communication system, it is possible to specify the required performance of the receive system (e.g., EIS) based on the transmit system parameters (e.g., EIRP) along with the path loss and other environmental losses. Conversely, the required transmit system performance (e.g., minimum EIRP) can also be derived from the receive system characteristics (e.g., EIS), taking into account the same propagation and environmental factors.

Typically, the transmit system is an active component capable of adjusting its EIRP through input power control, whereas the receive system is a passive component whose EIS is generally fixed and not configurable. Therefore, it is common practice to define a realizable receive system first, and then determine the required transmit EIRP accordingly to ensure link feasibility.

It is noted that the GEO Ku-band satellite system parameters specified in 3GPP are defined incorrectly in several aspects. As a result, in R4-2506293, a proposal was made to update the parameters as illustrated in the table 1 below.  In addition, R4-2506020 menthioned issue of the incorrect GEO Ku-band satellite system parameters at the NR_NTN_Ku Band_ Co-existence Session.   Ultimately, it was decided in R4-2507845 that the NR_NTN_Ku_Band_Coexistence Session would proceed with the incorrect parameters as they were without correction.  But, fortunately, NR_NTN_Ku_Band_UE_SAN RF session was able to identify the flawed parameters and has the opportunity to determin a reasonable SAN Reference Sensitiviy spec[2]. Table 2 presents an analysis of the implications of the incorrect parameters versus the appropriate parameters, viewed from both the antenna efficiency perspective and the system noise temperature perspective.

Table 1: Satellite Parameters

| Satellite orbit |  | GEO | LEO-600 |
| --- | --- | --- | --- |
| Satellite altitude |  | 35786 km | 600 km |
| Payload characteristics for UL transmissions |  | Set1 |  |
| Equivalent satellite antenna aperture (Note1) | S-band | 22 m | 2 m |
| G/T (dB/K) | (i.e. 2 GHz for UL) | 19 | 1.1 |
| Satellite RX max Gain (dBi) |  | 51 | 30 |
| Equivalent satellite antenna aperture (Note1) | Ku-band | 4 m | 0.714 m |
| G/T (dB/K) | (i.e. 14 GHz for UL) | 16.5 21.9 | 10.9 |
| Satellite RX max Gain (dBi) |  | 44 53 | 38.5 |
| Equivalent satellite antenna aperture (Note1) | KA-band | 3.33 m | 0.33 m |
| G/T (dB/K) | (i.e. 30 GHz for UL) | 28 | 13 |
| Satellite RX max Gain (dBi) |  | 58.5 | 38.5 |

Table 2: Incorrect Parameter Effect

| Parameters | Satellite RX max Gain (dBi) : 44 dBi G/T = 16.5 dB/k | Satellite RX max Gain (dBi) : 53 dBi G/T = 21.9 dB/k |
| --- | --- | --- |
| Influenced Factors |  |  |
| Antenna Efficiency | Extremely Low (7.17%) (Ka band (65%) and S band (59%). | More Reasonable (62.5%) |
| System Noise Temperature | Extremely Low (562.3 K) (Ka band (1122K) and S band (1584.9K). | More Reasonable (1288 K) |

- Discussion

In R4-2508766 WF, the following definitions were established for the minimum peak Effective Isotropic Radiated Power (EIRP) for Ku-band VSAT (Very Small Aperture Terminal) communications:

- For Ku-band VSAT communicating with a Geostationary (GEO) satellite: The minimum peak EIRP is 70 dBm.

- For Ku-band VSAT communicating with a Low-Earth Orbit (LEO) satellite:The minimum peak EIRP is 60 dBm.

Table 3: Received Power of SAN when Min Peak EIRP of VSAT applied

| SATELLITE | Min Peak EIRP of VSAT | Elevation Angle / Distance | Pathloss | Received Power at SAN |
| --- | --- | --- | --- | --- |
| GEO | 70 dBm | 90° / 35786 km | 206.7 dB | -136.7 dBm |
| GEO | 70 dBm | 30° / 38612 km | 207.4 dB | -137.4 dBm |
| LEO 1200 | 60 dBm | 90° / 1200 km | 177.3 dB | -117.3 dBm |
| LEO 1200 | 60 dBm | 30° / 1999 km | 181.7 dB | -121.7 dBm |
| LEO 600 | 60 dBm | 90° / 600 km | 171.2 dB | -111.2 dBm |
| LEO 600 | 60 dBm | 30° / 1075 km | 176.3 dB | -116.3 dBm |

This summary provides a link budget analysis for Ku-band VSAT communications with both GEO and LEO satellites, ensuring that the received power of SAN meets the required sensitivity threshold.

Given a specified G/T, the effective Isotropic Sensitivity (EIS) can be calculated using the following equations (4,5).

EIS1=KB+10LOG10BW-G/T+Target SNR					(4)

EIS2=PkT+10LOG10BW+NF+IM+Target SNR-Grx				(5)

Where KB : the thermal noise power spectral density, referred to 1 K and 1 Hz bandwidth, is -198.6 dBm/K/Hz (=10LOG101.38*10-23+30), BW :channel BW, and G/T : Antenna gain-to-noise-temperature (dB/K), PKT  : The thermal noise power spectral density, referred to 290 K and 1 Hz bandwidth, is –174 dBm/Hz , IM : Implementation Margin,

Target SNR =-1 dB.

Table 4: G/T-Inclusive vs. Explicit NF-Based EIS Parameterization for GEO

| GEO |  | EIS Calcuation Parmeters using G/T | = | EIS Calcuation Parmeters using Grx |  |
| --- | --- | --- | --- | --- | --- |
|  |  | G/T | = | Grx | NF + Implementation Margin |
| Ku band | Prev. | 16.5 dB/K (7.17% antenna efficiency) | = | 44 dBi | 3dB + (-0.1 dB) = 2.9 dB |
|  | corrected | 21.9 dB/K (62.5% antenna efficiency) | = | 53 dBi | 3dB + 3.5 dB = 6.5 dB |
| S band |  | 19 dB/K (59% antenna efficiency) | = | 51 dBi | 3dB + 4.4 dB = 7.4 dB |
| KA band |  | 28 dB/K (65% antenna efficiency) | = | 58.5 dBi | 3dB + 2.9 dB = 5.9 dB |
| The G/T values implicitly include the effects of noise figure and implementation losses, yielding the same EIS outcome as the explicit Grx-based approach. |  |  |  |  |  |

Table 5: G/T-Inclusive vs. Explicit NF-Based EIS Parameterization for LEO

| LEO | EIS Calcuation Parmeters using G/T | = | EIS Calcuation Parmeters using Grx |  |
| --- | --- | --- | --- | --- |
|  | G/T | = | Grx | NF + Implementation Margin |
| Ku band_original | 10.9 dB/K | = | 38.5 dBi | 3dB + (0) dB = 3 dB |
| Ku band_IM_2dB | 10.9 dB/K w/ 2 dB additional IM | = | 38.5 dBi | 3dB + (2) dB = 5 dB |
| Ku band_IM_5dB | 10.9 dB/K w/ 5 dB additional IM | = | 38.5 dBi | 3dB + (5) dB =8 dB |
| The G/T values implicitly include the effects of noise figure and implementation losses, yielding the same EIS outcome as the explicit Grx-based approach. |  |  |  |  |

G/T is a measured performance parameter. It inherently includes the effects of implementation margin.

GEO "G/T of 21.9 dB/K" represents an antenna gain of 53 dBi, noise figure of 3 dB, and an implementation margin (IM) of 3.5 dB. It implies antenna efficiency 62.5%, while GEO "G/T of 16.5 dB/K" represents an antenna gain of 44 dBi, noise figure of 3 dB, and an implementation margin (IM) of -0.1 dB, and It implies antenna efficiency 7.17%.

Therefore, we consider the scenario where the antenna efficiency is 62.5%, yielding an antenna gain of 53 dBi with a Noise Figure (NF) of 3 dB and an Intermodulation (IM) of 3.5 dB, resulting in a G/T value of 21.9 dB/K.

The original G/T of LEO (10.9 dB/K) is set somewhat tightly, as it includes the antenna gain (38.5 dBi), noise figure (3 dB), and implementation margin (0 dB). Based on R4-2508766 WF, we must consider an implementation margin of at least 2 dB.  I propose using total implementation margin of 5 dB because the mobility of LEO is likely to cause beam misalignment, which will directly impact reception performance.  This is the same approach as the Upperbound of EISREFSENS_50M for KA band of LEO was defined as  129 dBm, Where G/T is 13 dB/k w/ 5.0 dB additional IM.

Tables 6 and 7 provide the logarithmic scale of bandwidth values based on PRB allocation for FR1 and FR2, respectively.  For FR1, 50 MHz is suitable as a common reference for both FR1 and FR2, enabling consistent evaluation across frequency ranges.

Bandwidth offsets are defined as 10·log₁₀(BW) – 10·log₁₀(BW_ref), and are used to normalize performance metrics such as EIS.

Table 6 Bandwidth in Logarithmic Scale Based on PRB Allocation for FR1,

with Offsets 50 MHz References

| SCS (kHz) | 10 | 15 | 20 | 25 | 35 | 50 | 70 | 100 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz | MHz | MHz | MHz | MHz |
|  | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) | 10*LOG10 (BW) |
| 15 | 69.71 | 71.53 |  | 73.79 | 75.29 | 76.87 |  |  |
| 30 | 69.37 |  | 72.64 | 73.69 | 75.20 | 76.80 | 78.33 | 79.92 |
| Offset (ref. 50 MHz) | -7 | -5 | -4 | -3 | -1.5 | 0 | 1.5 | 3 |

Table 7 Bandwidth in Logarithmic Scale Based on PRB Allocation for FR2,

with Offset 50 MHz References

| SCS (kHz) | 50 | 100 | 200 | 400 |
| --- | --- | --- | --- | --- |
|  | MHz | MHz | MHz | MHz |
|  | 10*LOG10(BW) | 10*LOG10(BW) | 10*LOG10(BW) | 10*LOG10(BW) |
| 120 | 76.64 | 79.78 | 82.79 | 85.80 |
| Offset (ref. 50 MHz) | 0 | 3 | 6 | 9 |

Table 8 Upperbound Reference Sensitivity of 50MHz Ku band of GEO/LEO for FR1 & FR2

| 50 MHz Channel Bandwidth | 15 KHz | 30KHz | 120KHz | Representative Integer Value |
| --- | --- | --- | --- | --- |
| GEO | -144.63 | -144.7 | -144.86 | -145 dBm |
| LEO | -128.63 | -128.7 | -128.86 | -129 dBm |
| GEO condition : Grx = 53 dBi w/ NF=3Db & IM = 3.5 dB or G/T = 21.9 dB/K LEO condition : Grx = 38.5 dBi w/ NF=3Db & IM = 5 dB or G/T = 10.9 dB/K w/ 5.0 dB additional IM Representative Integer value is int(-144.X) = -145 where X is 1 to 9 |  |  |  |  |

As I understand, Although the Saltllite Rx max gain is fixed, the Reference sensitivity of 50MHz BW can vary by up to 9 dB across different vendors due to implementation-dependent factors for KA band.

Reference Sensitivity of 50MHz _upperbound=KB+10LOG10BW-G/T+Target SNR

=PkT+10LOG10BW+NF+IM+Target SNR-Grx

Reference Sensitivity of 50MHz _lowerbound=Reference Sensitivity of 50MHz _upperbound+Vendor _margin

Where KB : the thermal noise power spectral density, referred to 1 K and 1 Hz bandwidth, is -198.6 dBm/K/Hz (=10LOG101.38*10-23+30), BW :channel BW, and G/T : Antenna gain-to-noise-temperature (dB/K), PKT  : The thermal noise power spectral density, referred to 290 K and 1 Hz bandwidth, is –174 dBm/Hz , IM : Implementation Margin, Target SNR =-1 dB.

Example)

Reference Sensitivity of 50MHzupperbound for GEO=-198.6+76.64-21.9+-1 = -144.86

=-174+76.64+3+3.5+-1-53= -144.86

In Table 1, the received power for GEO is estimated at -137.4 dBm w/ min Peak EIRP 70 dBm of VSAT. The reference sensitivity for the 50 MHz upper bound is calculated to be -144.86 dBm.  Since the reference sensitivity of 50 MHz_upper bound is lower than the received power, stable communication is ensured.

Therefore, we propose the vendor Margin =  9dB (same as Ka band Vendor margin)

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -136 to -145 dBm.

For LEO class SAN, EISREFSENS_50M for Ku band is an integer value in the range -120 to -129 dBm.

The difference between BS type 1-O and BS type 2-O in OTA Reference Sensitivity test requirements lies in their operating frequency ranges. Specifically, the frequency range of BS type 1-O is below 7.125 GHz, while that of BS type 2-O exceeds 24.25 GHz. Consequently, the OTA Reference Sensitivity for BS type 1-O is characterized by conductive sensitivity, expressed as, for example, [-92.4 – ΔOTAREFSENS], where ΔOTAREFSENS represents antenna gain, and -92.4 is one of the example of concutive sensitivity. This is because microwave coaxial connectors can support frequencies below 6 GHz, 9 GHz, or 11 GHz (with recent advancements in technology). Therefore, even when using FR1-NTN Ku band (which operates over 10 GHz), we must adhere to the format of BS type 2-O, as coaxial connectors above 10 GHz do not exist.  Fig. 1 shows microwave coaxial connector for measuring conducted reference sensitivity and spec for most promising mobile connector [6].

Fig.1 Microwave Coaxial Connector for measureing conducted ref sensitivity and spec for most promising microwave coaxial connetor [6].

Therefore, For OTA reference sensitivity level for FR1-NTN Ku-band and FR2-NTN Ku-band, it is necessary to follow the method for existing OTA reference sensitivity format of SAN(BS) type 2-O.

Table 9: FR2-NTN OTA reference sensitivity requirement for Ku band

| SAN channel Bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 50 | 120 | G-FR2-NTN-A1-2 | EISREFSENS_50M + ΔFR2_REFSENS |
| 100, 200, 400 | 120 | G-FR2-NTN-A1-3 | EISREFSENS_50M + 3 + ΔFR2_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

Table 10: FR1-NTN above 10 GHz OTA reference sensitivity requirement for Ku band

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 (Note 1) | EISREFSENS_50M + (-10) + ΔFR1_REFSENS |
| 10 | 30 | G-FR1-NTN-A1-2 (Note 1) | EISREFSENS_50M + (-10) + ΔFR1_REFSENS |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 (Note 1) | EISREFSENS_50M + (-4) + ΔFR1_REFSENS |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 (Note 1) | EISREFSENS_50M + (-4) + ΔFR1_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

=====================================================================================

- Conclusion

In this contribution, we want to share some views on SAN RF requirements (OTA ref. sensitivity level) for NR NTN ku band and proposals are made as following

Observation 1. From a link budget perspective, assuming the minimum peak EIRP of VSAT (70 dBm : GEO / 60 dBm : LEO), the received power at the SAN is -137.4 dBm for GEO and -121.7 dBm for LEO.  For stable communication, the received power at SAN shall be greater than or equal to the EIS reference sensitivity.

Observation 2. GEO "G/T of 21.9 dB/K" represents an antenna gain of 53 dBi, noise figure of 3 dB, and an implementation margin (IM) of 3.5 dB. It implies antenna efficiency 62.5%, while GEO "G/T of 16.5 dB/K" represents an antenna gain of 44 dBi, noise figure of 3 dB, and an implementation margin (IM) of -0.1 dB, and It implies antenna efficiency 7.17%.

Proposal 1:

For GEO class SAN, EISREFSENS_50M  for Ku band is an integer value in the range -136 to -145 dBm.

For LEO class SAN, EISREFSENS_50M for Ku band is an integer value in the range -120 to -129 dBm.

Observation 3. The difference between BS type 1-O and BS type 2-O in OTA Reference Sensitivity test requirements lies in their operating frequency ranges. Specifically, the frequency range of BS type 1-O is below 7.125 GHz, while that of BS type 2-O exceeds 24.25 GHz. Consequently, the OTA Reference Sensitivity for BS type 1-O is stated by conductive sensitivity, expressed as, for example, [-92.4 – ΔOTAREFSENS], where ΔOTAREFSENS represents antenna gain and -92.4 is one of the example value of concutive sensitivity. This is because microwave coaxial connectors can support frequencies below 6 GHz, 9 GHz, or 11 GHz (with recent advancements in technology). Therefore, even when using FR1-NTN Ku band (which operates over 10 GHz), we must adhere to the format of BS type 2-O, as coaxial connectors above 10 GHz do not exist due to manufacturing limitation.

Proposal 2: FR1-NTN above 10 GHz OTA reference sensitivity requirement for Ku band shows below table 10.

Table 10: FR1-NTN above 10 GHz OTA reference sensitivity requirement for Ku band

| SAN channel bandwidth (MHz) | Sub-carrier spacing (kHz) | Reference measurement channel | OTA reference sensitivity level, EISREFSENS (dBm) |
| --- | --- | --- | --- |
| 10, 15 | 15 | G-FR1-NTN-A1-1 | EISREFSENS_50M + (-10) + ΔFR1_REFSENS |
| 10 | 30 | G-FR1-NTN-A1-2 | EISREFSENS_50M + (-10) + ΔFR1_REFSENS |
| 25, 35, 50 | 15 | G-FR1-NTN-A1-4 | EISREFSENS_50M + (-4) + ΔFR1_REFSENS |
| 20, 25, 35, 50, 70, 100 | 30 | G-FR1-NTN-A1-5 | EISREFSENS_50M + (-4) + ΔFR1_REFSENS |
| NOTE 1: EISREFSENS is the power level of a single instance of the reference measurement channel. This requirement shall be met for each consecutive application of a single instance of the reference measurement channel mapped to disjoint frequency ranges with a width corresponding to the number of resource blocks of the reference measurement channel each, except for one instance that might overlap one other instance to cover the full SAN channel bandwidth. NOTE 2: The declared EISREFSENS_50M shall be within the range specified above. |  |  |  |

====================================================

- References

[1] R4-2504720, Way Forward for [114bis][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL, TSG-RAN WG4 Meeting #114bis, April 2025

[2] R4-2508778 Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, CHTTL, TSG-RAN WG4 Meeting #115, May 2025

[3] R4-2506293 for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, LGE, TSG-RAN WG4 Meeting #115, May 2025

[4] R4-2506020 for [115][313] NR_NTN_Ku_Band_Coexistence, CATT, TSG-RAN WG4 Meeting #115, May 2025

[5] R4-2507845 Topic Summary for [115][313] NR_NTN_Ku_Band_ Coexistence, Intelsat, TSG-RAN WG4 Meeting #115, May 2025

[6]  https://www.murata.com/-/media/webrenewal/support/library/catalog/products/connector/o30e.ashx?la=en&cvid=20230202042145000000