# 3GPP TSG-RAN WG4 Meeting #116	R4-2509688

# Bengaluru, India, Aug 25th – 29th, 2025

Title: 	On optimizing CSSF for FR2-1 L3 measurement delay

Source: 	OPPO

Agenda item:	7.14.3.2

Document for:	Discussion

Introduction

In last meeting, a WF [1] was approved on FR2-1 SSB based L3 measurement delay reduction.

In this contribution, we will provide discussion on FR2-1 SSB based L3 measurement delay reduction by optimizing CSSFoutside_gap.

Discussion

CSSF solution 1

Issue 1-2-1: revision on solution 1 Continue discussion in next meeting. Options of this meeting are captured in topic summary R4-2503618. - Proposal 1 (OPPO, QC, Apple): For solution 1, remove the sub-case and update the agreements as follows: - Follow the network indication to measure a SCC, if - neither PCC nor PSCC is in band, and - if there is no SCCs on which UE is configured to report SSB based measurements in band, or - if there are multiple SCCs on which UE is configured to report SSB based measurements in band. - Network indication of a specific SCC per-band for measurement. - if network indicates which of the SCC to be measured in a band, UE will perform both serving and neighbour cell measurements on the indicated SCC in a band. - Otherwise, UE will perform serving and neighbor cell measurements on one SCC per band, which SCC to measure is up to UE implementation. - Proposal 2 (OPPO): RAN4 to discuss the potential revision on network indication requests of solution 1, and if agreed, send LS to RAN2 for update.

In last meeting, we propose to revise the agreement of solution 1, because measurement on SCC without any report configuration is not valid to both network and UE. If there is no SCCs on which UE is configured to report SSB based measurements in band, how UE perform the measurement and report is totally up to UE, without any specific RRM requirements. We support to remove the invalid case that there is no SCCs on which UE is configured to report SSB based measurements in band.

Proposal 1: For solution 1, remove the remove the invalid case that there is no SCCs on which UE is configured to report SSB based measurements in band.

On top of UE specific RRC configuration (per UE) to enable/disable the CSSF optimization feature,  RAN2 also discussed whether to introduce additional RRC signaling as indication of a specific SCC per band for measurement.

RAN2 have discussed the RRC configuration to support the CSSF optimization and reached the following agreements: Introduce the UE specific RRC configuration (per UE) to enable/disable the CSSF optimization feature. About the network indication of a specific SCC per-band for measurement, RAN2 discussed the following two alternatives: - Alternative 1: Introduce new RRC configuration to indicate which SCC to be measured. - When CSSF optimization feature is enabled, UE only performs the serving and intra-frequency neighbor measurement on the indicated SCC per band. - Alternative 2: No additional RRC signaling is needed. - When CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement. - One of the SCCs on which UE is configured to report SSB based measurements when neither PCC nor PSCC is in the same band, so that the selected SCC shall be an SCC where the UE is configured with SS-RSRP measurement reporting if such SCC exists, otherwise the selected SCC is determined by UE implementation.. Majority’s view in RAN2 is Alternative 2, but RAN2 did not reach a consensus. Therefore, RAN2 would like to ask RAN4 to decide which alternative to adopt for SCC indication and provide feedback during August meeting so that RAN2 can prepare the RRC CR for approval in August meeting.

The network can always reconfigure and adapt the RRC configuration for measurement and report, in replace of an additional indication of a specific SCC to be selected. The benefit of such network indication seems quite limited. And the majority’s view in RAN2 is Alternative 2 that no additional RRC signaling is needed. With Alternative 2, the feature in RAN4 is not broken as well. We also agree that when CSSF optimization feature is enabled, UE follows the existing RAN4 rule (defined in clause 9.2.3.2 of TS 38.133) below to select the SCC to perform both serving and intra-frequency neighbor measurement.

Furthermore, we think the SCC decided by UE can be more accurate than that indicated by the network, and such carrier can represent the quality of carriers in the same band. From UE side, UE can select one suitable SCC based on both network configuration and real-time channel conditions, which leave some flexibility to UE implementation.

Proposal 2: Support alternative 2 that no additional RRC signaling is needed, and send LS to RAN2.

Issue 1-2-2: Expand solution 1 to FR1 case - FFS: - Option 1 (Nokia, Apple, CTC): Expand the solution 1 to FR1 band. - Option 1a (Nokia): - Expand the solution 1 to at least FR1 intra-band contiguous CA. - Details: - Network indicates to enable or disable this CSSF enhancement i.e. measuring one serving CC per band for FR1 and FR2 separately. - When network indicates to enable Tentative solution 1 for FR1 bands, UE is required to measure serving and neighbor cells on one serving CC per band when configured with FR1 intra-band contiguous CA. - Option 2 (Huawei): Not to expand solution 1 to FR1 band. - Option 3 (moderator): Expand the solution 1 to FR1 intra-band contiguous CA, i.e., measure one serving CC per FR1 band if all to-be-measured serving CCs in this FR1 band are contiguous.

RAN4 agreed to discuss whether to extend solution 1 to FR1 after FR2-1 L3 measurement delay reduction by optimizing CSSF is concluded. However, compared to FR2’s antenna panels and beam design, FR1’s RF and antenna design are quite different, e.g., FR1 omnidirectional antenna, LB, MB and HB RF architecture. We share the similar view as option 2. Even within the same band, different CCs may experience varying levels of interference in FR1, regardless of contiguous or non-contiguous CCs in the FR1 band. Therefore, one CC in FR1 band cannot accurately represent the measurement results for the entire band.

Proposal 3: Not to expand solution 1 to FR1 band.

CSSF solution 3

Editor’s note: For UE supports [FR1+FR2 CA (PCell in FR1) 3-searcher capability] the applicability for inter-RAT measurement with no measurement gap can be revisited.

For solution 3, we support to differentiate the CSSF values for UE supporting CSSF enhancement on inter-frequency MO without gap and E-UTRA inter-RAT MO without gap for SA mode. For inter-RAT E-UTRA MO, only the legacy searcher is used. The additional searcher can apply for measurements on SCC and inter-frequency MO without gap. For UE supporting [new 3-searcher capability] the Enhanced CSSF scaling factor does not apply for the E-UTRA inter-RAT MOs without MG that are being measured outside of MG.

Proposal 4: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG that are being measured outside of MG.

conclusions

In this paper we provided our views on CSSF optimization.

Proposal 1: For solution 1, remove the remove the invalid case that there is no SCCs on which UE is configured to report SSB based measurements in band.

Proposal 2: Support alternative 2 that no additional RRC signaling is needed, and send LS to RAN2.

Proposal 3: Not to expand solution 1 to FR1 band.

Proposal 4: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG that are being measured outside of MG.

Reference

- R4-2504899, WF on RRM requirements for NR_RRM_Ph5_Part1

- R2-2504728, Reply LS on CSSF optimization for NR RRM Phase 5