# 3GPP TSG-RAN WG4 Meeting #116	R4-2509692

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On accuracy of CSI-RS based L1 measurement for SBFD operation

Source: 	OPPO

Agenda item:	7.21.5

Document for:	Discussion

Introduction

In last meeting, a WF on RRM requirements for NR_duplex_evo was approved [1].

In this contribution, we will provide our views on RRM impacts for SBFD operation.

Discussion

Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain • Background: o Different cases for DUD - Case 1: X1 >= X3 and X2 >= X3 - Case 2: X1 >= X3 and X2 < X3 - Case 3: X1 < X3 and X2 < X3, (X1 + X2) >= X3 - where X1 and X2 are CSI-RS BW in DL sub-band #1 and sub-band #2, and X3 is the min BW with which the existing accuracy requirements apply (X3=48 for L1-RSRP/L1-SINR and X3=24 for RLM/BFD/CBD). Agreements (adhoc session on Thursday with additions) o Note: the following are for UE that can support simultaneous PDSCH reception on the two DL subbands for DUD o Existing delay (number of samples) requirements and accuracy requirements apply for Case 1 and Case 2 o If UE indicates that CSI processing time is scaled by 2, further discuss whether and how to update the delay requirements - Option 1: Add a lower bound of 8ms on top of existing delay requirements. - Option 2: other o For Case 3 - For L1-RSRP and L1-SINR, existing delay requirements apply and accuracy requirements are relaxed provided that X1 > X4 or X2 > X4, where the accuracy requirements depend on the X4 = 24 PRBs • In the test case, not model the interfering UE.

For CSI-RS L1 measurement with puncturing in frequency domain, RAN4 agreed to define relaxed accuracy requirements based on 24 RBs for case 3. Based on the agreed simulation assumption, the L1 measurements for both 24RBs and 48 RBs are simulated and the accuracy results are summarized in Table 1 below. About 0.6~0.8 dB performance loss is observed for 24 RBs compared with 48 RBs.

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

Table 1. Simulation results for CSI-RS L1-RSRP accuracy in Case 3

|  | SCS | CSI-RS (RB) | 5% | 95% | max (5%, 95%) |
| --- | --- | --- | --- | --- | --- |
| FR1 | 15kHz | 24 | -1.73 | 1.30 | 1.73 |
|  |  | 48 | -1.15 | 0.91 | 1.15 |
|  | 30kHz | 24 | -1.77 | 1.32 | 1.77 |
|  |  | 48 | -1.07 | 0.91 | 1.07 |
| FR2 | 120kHz | 24 | -1.81 | 1.32 | 1.81 |
|  |  | 48 | -1.09 | 0.98 | 1.09 |

To be conservative, we propose to introduce 1dB margin for the relaxed accuracy requirement, based on  legacy CSI-RS based L1-RSRP accuracy requirements defined in clause 10.1.19.2 in TS38.133.

Proposal 1: For the relaxed accuracy requirement for CSI-RS L1 measurement with 24 RBs for case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Conclusions

In this paper we provided our views on RRM impact for SBFD operation.

Observation 1: For CSI-RS L1 measurement, 0.6~0.8 dB performance loss is observed for 24 RBs compared to 48 RBs.

Proposal 1: For the relaxed accuracy requirement for CSI-RS L1 measurement with 24 RBs for case 3, consider 1dB relaxation compared to the legacy accuracy with 48 RBs.

Reference

- R4-2508274, WF on RRM requirements for NR_duplex_evo