3GPP TSG-RAN WG4 Meeting #116															R4-2509698

Bengaluru, India, August 25th – 29th, 2025

Title: 	On event triggered L1 measurement reporting

Source: 	OPPO

Agenda item:	7.25.3.1

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on remaining issues of measurements related enhancements for LTM, including event trigger L1 measurement reporting.

Discussion

Event triggered L1 measurement reporting

- Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

We support to update the requirements for L1 event triggered reporting criteria per measurement category. Whether to define new Ecat capability for LTM L1 measurements or share Ecat capability with L3 measurements can be candidate options.

Observation 1: Defining new Ecat capability for LTM L1 measurements, or sharing the Ecat capability for L1 and L3 measurements are candidate options.

We support to include L1 reporting criteria per measurement category into current Ecat. UE can share the UE capability between L1 and L3 reporting. We suggest to use the same value of Ecat for L1+L3 intra-frequency and inter-frequency measurement category, and update the note for each of events description, as the table below for example:

| Measurement category | Ecat | Note |
| --- | --- | --- |
| Intra-frequency Note 1,2,3,4,5 | 9 | Events for any one or a combination of intra-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN intra-frequency cells |
| Inter-frequency Note 2,3,4,5 | 10 | Events for any one or a combination of inter-frequency SS-RSRP, SSB based L1-RSRP, CSI-RS based L1 RSRP, SS-RSRQ, SS-SINR, CSI-RSRP, CSI-RSRQ, and CSI-SINR for NG-RAN inter-frequency cells |

Proposal 1: Support to share the UE capability between L1 and L3 reporting, and update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Conclusion

In the contribution we provide the following proposals:

Observation 1: Defining new Ecat capability for LTM L1 measurements, or sharing the Ecat capability for L1 and L3 measurements are candidate options.

Proposal 1: Support to share the UE capability between L1 and L3 reporting, and update intra-frequency and inter-frequency measurement reporting criteria requirements for SSB and CSI-RS based L1-RSRP with the same Ecat.

Reference

- R4-2408282, WF on NR mobility enhancements Phase 4_part1