# 3GPP TSG-RAN WG4 Meeting #116	R4-2509701

# Bengaluru, India, August 25th – 29th, 2025

Title: 	On RRM performance requirements of R19 mobility

Source: 	OPPO

Agenda item:	7.25.4

Document for:	Discussion

Introduction

In last meeting, a WF was approved on RRM requirements for R19 mobility [1].

In this contribution, we provide our views on RRM perf part.

Discussion

Event-triggered L1 reporting

- Issue 4-1-1: test case list for event-triggered L1 reporting Candidate options: - Option 1: Nokia - FR1 and FR2 Intra-frequency SSB-based event-triggered L1 reporting (without measurement gaps) - FR1 and FR2 Inter-frequency SSB-based event-triggered L1 reporting (with and without measurement gaps) - FR1 and FR2 Intra-frequency CSI-RS-based event-triggered L1 reporting (without measurement gaps) - At least 1 test case for event-triggered periodic reporting. - Option 2: Apple - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR1 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR1 - Event-triggered Reporting based on SSB based L1 RSRP measurement in FR2 - Event-triggered Reporting based on CSI-RS based L1 RSRP measurement in FR2 - Option 3: CATT - The new test cases for event triggered L1 measurement reporting need to defined in the clauses defined for R18 LTM. - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX - TC for LTM Inter-frequency event triggered SSB based L1 measurement reporting tests with gaps under non-DRX - TC for LTM Intra-frequency event triggered SSB based L1 measurement reporting tests without gap under non-DRX with SSB index reading - For event triggered CSI-RS based L1 measurement reporting, only new TC for LTM Intra-frequency event triggered reporting tests without gap under non-DRX should be defined. - Option 4: cover both SSB and CSI-RS based, in FR1 and FR2, repsectively (vivo) - Option 5: follow aspects should be considered (ZTE) - With gap &without gap, intra-f & inter-f - non-DRX & DRX - With SSB index reading & without SSB index reading - Other aspects are not precluded. - Option 6: CMCC - Both FR1 and FR2-1 are considered. - For SSB based event triggered L1 measurement reporting, intra-frequency measurement, inter-frequency measurement without gap, gap-based inter-frequency measurement need to be considered. - For CSI-RS based event triggered L1 measurement reporting, measurement without gap need to be considered. - Option 7: RAN4 to consider the list of test cases agreed in Release 18 to perform event-triggered L1-RSRP testing, thereby defining the same list of test cases for event-triggered L1-RSRP. Adopt the following list of test cases for Release 19 SSB-based event-triggered reporting. (E///) - Intra-f L1-RSRP measurement for LTM - Intra-f L1-RSRP measurement in FR1 - Intra-f L1-RSRP measurement in FR2 - 1 serving cell, 2 neighbor cells, none of neighbor cells’ TCI state activated - Intra-f L1-RSRP measurement in FR2 - 1 serving cell, 2 neighbor cells, one of neighbor cells’ TCI state activated - Inter-f L1-RSRP measurement with MG - Inter-f L1-RSRP measurement with MG in FR1 - Inter-f L1-RSRP measurement with MG in FR2 - Inter-f L1-RSRP measurement without gap for LTM - Inter-f L1-RSRP measurement without gap in FR1 - Inter-f L1-RSRP measurement without gap in FR2 - 2 neighbor cells, none of neighbor cells’ TCI state activated - Option 8: HW - LTM Intra-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - LTM Intra-frequency event triggered reporting tests without gap (CSI-RS based L1-RSRP measurement) - LTM Inter-frequency event triggered reporting tests with gap (SSB based L1-RSRP measurement) - LTM Inter-frequency event triggered reporting tests without gap (SSB based L1-RSRP measurement) - FFS on LTM Inter-frequency event triggered reporting tests for CSI-RS based L1 measurement

For the test cases for event triggered reporting, we agree that the following aspects should be considered, like SSB or CSI-RS based, intra-frequency or inter-frequency, measurement with gap or without gap, FR1 or FR2, non-DRX or DRX, with SSB index reading or without SSB index reading. However, compared to legacy tests for LTM, the new tests should cover the delta parts mainly about event triggered reproting. To reduce the amount of test cases, we prefer to define 2 cases to cover different configurations in different scenarios.

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

Proposal 1: For the test cases for event triggered reporting, we propose 4 tests:

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

CSI-RS based L1 measurement

- Issue 4-2-1: CSI-RS based L1 measurement accuracy Candidate options: - Option 1: CSI-RS based L1-RSRP accuracy of the serving cell can be reused for neighbour cell. (Apple, Nokia, CATT, vivo, MTK, ZTE, CMCC, E///) - Option 1a: reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the condition of CSI-RS resources with 48 PRBs and density 3. (vivo) - Option 1b: (HW) - The SINR of the CSI-RS L1-RSRP measurement on candidate cell is above -3dB. The accuracy of CSI-RS based L1-RSRP on serving cell can be used as a baseline. The conditions of CSI-RS L1-RSRP measurement on candidate cells are: - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified: - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2 - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2 - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell. - Option 1c: (CATT) - RAN4 to define new clauses for CSI-RS based L1-RSRP requirements for neighbor cell in clauses 10.1.19D, 10.1.19E, 10.1.20A and 10.1.20B as R18 SSB based L1-RSRP requirements for neighbor cell. Recommended WF: - Reuse legacy accuracy requirements for CSI-RS based L1 measurements for serving cell under the following conditions: - The bandwidth of CSI-RS is 48 PRBs and the density is 3. - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements. - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP. - The following accuracy requirements for CSI-RS based L1 RSRP on candidate cells shall be specified: - Absolute and relative CSI-RS based intra-frequency L1-RSRP accuracy requirements for FR1 and FR2 - Absolute and relative CSI-RS based inter-frequency L1-RSRP accuracy requirements for FR1 and FR2 - Legacy SS/CSI-RSRP measurement L1 report mapping specified in clause 10.1.6 (including measured quantity reporting and differential reporting) can be reused for SS/CSI-RS based L1 measurement reporting on neighbor cell.

We agree with the recommended WF above.

Proposal 2: For CSI-RS based L1-RSRP measurement on neighbour cell, we propose to

- Reuse legacy accuracy requirements of CSI-RS based L1 measurements for serving cell under the following conditions:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Specify absolute and relative intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

- Reuse the legacy L1 report mapping specified in clause 10.1.6

- Issue 4-2-2: test case list for CSI-RS based L1 measurement Candidate options - Option 1: Nokia - FR1 intra-frequency without gaps test case. - FR2 intra-frequency without gaps test case with CSI-RS beam sweeping and without FR2 SSB measurements. - FR2 intra-frequency without gaps test case without CSI-RS sweeping and with FR2 SSB measurements. - Option 2: Apple - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1 - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2 - Option 3: CATT - CSI-RS based Intra-frequency L1-RSRP measurement in FR1. (CATT) - Option 4: cover both delay and accuracy (vivo) - Option 5: for CSI-RS based L1-RSRP measurement on LTM candidate cell(s), it is proposed to define test cases for measurement delay. And both the case with step 2 and the case without step 2 are tested. (CMCC) - Option 6: E/// - Intra-f L1-RSRP measurement in FR1 - Intra-f L1-RSRP measurement in FR2 - Inter-f L1-RSRP measurement without gap in FR1 - Inter-f L1-RSRP measurement without gap in FR2

We support to only consider tests for intra-frequency CSI-RS based L1-RSRP measurement.

Proposal 3: Support to define the following test cases:

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Conditional LTM

About CLTM test scope, we are ok to consider the different configuration including frequency range, intra or inter-frequency, L1 or L3 based CLTM, RACH-less or RACH-based CLTM, with or without early DL sync, PCell swith or PSCell switch. To reduce the test amount, we prefer to choose one configuration for the similar requirements.

For example, only L1 based CLTM with early DL sync will be considered. Other configuration like intra-f or inter-f, RACH-less or RACH-based CLTM, PCell or PSCell switch can be tested in different cases.

Proposal 4: For Conditional LTM tests, only L1 based CLTM with early DL sync will be considered.

Proposal 5: Support to define the following tests for conditional LTM

  - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1

  - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2

  - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2

  - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1

Conclusion

In the contribution we provide the following proposals:

Proposal 1: For the test cases for event triggered reporting, we propose 4 tests:

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR1

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR1

  - event triggered reporting tests for intra-frequency CSI-RS based L1-RSRP measurement without gap under non-DRX for FR2

  - event triggered reporting tests for inter-frequency SSB based L1-RSRP measurement with gap under non-DRX for FR2

Proposal 2: For CSI-RS based L1-RSRP measurement on neighbour cell, we propose to

- Reuse legacy accuracy requirements of CSI-RS based L1 measurements for serving cell under the following conditions:

  - The bandwidth of CSI-RS is 48 PRBs and the density is 3.

  - The performance with larger bandwidth of CSI-RS is equal to or better than the accuracy requirements.

  - The timing offset between the reference measurement timing and the target CSI-RS in one layer is no larger than CP.

- Specify absolute and relative intra-frequency L1-RSRP accuracy requirements for FR1 and FR2

- Reuse the legacy L1 report mapping specified in clause 10.1.6

Proposal 3: Support to define the following test cases:

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP measurement for neighbour cell in FR2

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR1

  - Intra-frequency CSI-RS based L1-RSRP accuracy requirements for neighbour cell in FR2

Proposal 4: For Conditional LTM tests, only L1 based CLTM with early DL sync will be considered.

Proposal 5: Support to define the following tests for conditional LTM

  - (SA) RACH based intra-frequency subsequent CLTM for PCell switch from FR1 to FR1

  - (SA) RACH less inter-frequency subsequent CLTM for PCell switch from FR2 to FR2

  - (NR-DC) RACH based intra-frequency subsequent CLTM for PSCell switch from FR1 to FR2

  - (NR-DC) RACH less inter-frequency subsequent CLTM for PSCell switch from FR2 to FR1

Reference

- R4-2508282, WF on NR mobility enhancements Phase 4_part1