# 3GPP TSG-RAN WG4 Meeting #116	R4-2509702

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.26.3

Source: 	OPPO

Title: 		RRM requirements for XR enhancements

Document for:	Discussion

Introduction

RAN4 achieved multiple agreements over the last few meetings on DCI based MG cancellation mechanism introduced by RAN1. In this contribution paper, we further discuss the remaining issues identified in the WF [1] from the last meeting.

Impact on RRM requirements

- Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter - Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms CSSFinter, Ceil(8 1.5) Max(MGRP, SMTC period, DRX cycle) CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 DRX cycle CSSFinter + Nskip DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) 1.5) Max(MGRP, SMTC period, DRX cycle)) CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) DRX cycle CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

RAN4 agreed that the MG cancellation applies only to L3 measurements and to follow NR-U-like approach for designing the formulas for measurement delay extension. We prefer to reuse NR-U approach and add the number of skipped MGs on top of the legacy number of SSB samples for each requirement.

Proposal 1: For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period.

As an example, Table 9.2.6.2-1 can be modified as below:

Table 9.2.6.2-1: Time period for PSS/SSS detection (FR1)

| DRX cycle | TPSS/SSS_sync_intra |
| --- | --- |
| No DRX | max(600 ms, (5+Nskip) x Kgap x max(MGRP, SMTC period)) x CSSFintra |
| DRX cycle≤ 320 ms | max(600 ms, ceil(M2Note 1x (5+Nskip) x Kgap) x max(MGRP, SMTC period,DRX cycle)) x CSSFintra |
| DRX cycle>320 ms | Ceil((5+Nskip) x Kgap ) x max(MGRP, DRX cycle) x CSSFintra |

Issue 3-4: Conditions for measurement skipping <Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

Considering the different UE implementation of handling gap skipping, the time margin between two shots of DCI based gap skipping should be set for UE to re-arrange the measurement/processing timeline. We support to specify the limit on maximum number of skipped MGs within a measurement period and maximum acceptable time separation between two non-cancelled MGs as a part of the requirements, i.e., option 3.

Proposal 2: RAN4 to define maximum time separation between two non-skipped gaps for measurement.

Conclusion

Proposal 1: For measurement delay extension, RAN4 to extend the L3 delays by adding the number of skipped MG occasions on top of the legacy number of measurement samples in the measurement period.

Proposal 2: RAN4 to define maximum time separation between two non-skipped gaps for measurement.

- References

[1] R4-2504911, “WF on RRM requirements for XR_Ph3”, Nokia, RAN4#114bis