# 3GPP TSG-RAN WG4 Meeting #116	R4-2509715			R4-250xxxx

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.22.3.3

Source: 	CMCC

Title: 	Discussion on A-IoT CW RF requirements

Document for:	Discussion

# 1. Introduction

In RAN plenary #106 meeting, a new WI [1] of A-IoT is approved with following RAN4 lead RF part objectives:

- RAN4 scope:

  - Specify RF requirements for Ambient-IoT BS, device 1, and CW

    - RF requirements for Type 1-C Ambient-IoT BS

    - RF requirements for device 1

    - RF requirements for CW

  - Specify RRM core requirements for device 1, if necessary

  - Study and develop OTA test methodology for A-IoT device 1

    - Consider test methods specified in TR 38.870 as starting point. Take test system reuse, test system complexity and test time into account, when developing test methods suitable for Ambient IoT.

    - Develop the preliminary Measurement Uncertainty (MU) assessment for the test system

  - Use band n8 as an example band

In this contribution, we discuss the RF requirements for A-IoT CW node.

- Discussion

In last meeting, the remaining issues for CW is listed as below: the OFF requirements, the phase noise requirements, the SEM requirements.

- OFF power requirements

- Issue 3-2-1: OFF power - Agreement: - OFF power is -50dBm, RBW is FFS

OFF power requirements is still FSS in last meeting. Current scope only contains the HD FDD operation between R2D and CW2D/D2R, CW is OFF when reader is transmitting. One motivation of OFF power is to avoid the interference from closed CW to device receiver. The proponent view of OFF requirements is that the OFF power will influence the noise floor at device side especially when many CW nodes are deployed in the same room. But usually the received SNR at device side is much higher because link budget is the bottleneck. The impact of OFF power at device side is small. Therefore, the UE OFF requirements of -50dBm is enough. One another issue is the measurement bandwidth. In theory, it is equal to CW bandwidth, if there is no such requirement, we can reuse the same requirements as UE, i.e./MHz.

Proposal 1: OFF power is suggested as -50dBm/MHz.

- Phase noise for outside topology

- Issue 3-4: Phase noise - Agreement: - For BS with phase noise cancellation capability, no phase noise requirement is specified. - For BS without phase noise cancellation capability, to define Phase noise offset [x]kHz from CW frequency, X value is FFS.

- As discussed in our A-IoT BS contribution, to meet REFSENSE requirement, the residual phase noise is much strict. Detailed analysis is listed as below:

at BS side, CW cancellation mechanism could achieve about 100 dB using anolog and digital algorithm. But it’s hard for CW node to achieve 100dBc integral phase noise. This means CW’s phase noise is the key factor that dominate final REFSENSE performance if no additional phase noise cancellation algorithm is implemented.

Observation 1: the residual phase noise is the key factor that will decrease REFSENSE performance.

Reader REFSENSE performance during testing depends on CW node performance which is unfair for reader when worse CW node is used in testing especially when BS without phase noise cancellation capability. For this case, the phase noise from CW is the key factor that contribute to desense. The phase noise should be defined together with REFSENSE. We suggest to use 6dB desense which means 70-80dBc phase noise as shown below table.

| D2R CBW | desense | wanted signal | noise floor | phase noise interference | integral phase noise/dBc | received CW signal | CW power | MCL between CW and BS | Distance Assume 6dB atenna gain at reader side |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 200 | 6 | -109 | -111 | -106 | -50 | -56 | 33 | 89 | 1542 |
|  | 6 | -109 | -111 | -106 | -60 | -46 | 33 | 79 | 488 |
|  | 6 | -109 | -111 | -106 | -70 | -36 | 33 | 69 | 154 |
|  | 6 | -109 | -111 | -106 | -80 | -26 | 33 | 59 | 49 |
|  | 6 | -109 | -111 | -106 | -90 | -16 | 33 | 49 | 15 |
|  | 6 | -109 | -111 | -106 | -100 | -6 | 33 | 39 | 5 |
| 3520 | 6 | -97 | -99 | -94 | -50 | -44 | 33 | 77 | 368 |
|  | 6 | -97 | -99 | -94 | -60 | -34 | 33 | 67 | 116 |
|  | 6 | -97 | -99 | -94 | -70 | -24 | 33 | 57 | 37 |
|  | 6 | -97 | -99 | -94 | -80 | -14 | 33 | 47 | 12 |
|  | 6 | -97 | -99 | -94 | -90 | -4 | 33 | 37 | 4 |
|  | 6 | -97 | -99 | -94 | -100 | 6 | 33 | 27 | 1 |

| CW power | PL:CW to device/20m | backscatter loss | PL: device to reader | REFSENSE |
| --- | --- | --- | --- | --- |
| 33 | 58 | 10 | 74 | -109 |

This is much strict requirement for CW performance. If vendors think it is challenging, we can only consider the case that BS with phase noise cancellation capability to not delay this WI.

Observation 2: the phase noise requirement is about 70-80dBc for the case when BW without phase noise cancellation capability. If companies think such requirement is much challeging, we can only consider BS with phase noise cancellation capability in R19.

- Unwanted emission requirements

FFS on unwanted emission boundary and corresponding values

As for the boundary of unwanted emisison requirements, we suggestet to use D2R channel bandwidth edge as boundary. Usually the CW node and reader node would belongs to the same vendor to help cancel CW interference at reader side. No need for explicit unwanted emission inside the D2R bandwidth which will be implicitly reflected into reader REFSENSE definition. Based on the discussion of device CBW, there may be many candidate CBW values due to defferent combinations of device date rate, R factor. It’s no need to defing such complex CW requirements depending on variable device CBW. To avoid interference to other system, we can only consider device’s minimum CBW case.

Proposal 2: use minimum D2R CBW as the boundary for CW unwanted emission requirements. i.e. inside the minimum D2R CBW, we define phase noise if needed, while outside the CBW, we define unwanted emission requirements.

As for the detailed value, the same value as NB-IoT BS is suggested.

Proposal 3: the same NB-IoT unwanted emission requirement with 33dBm output power is suggested for CW node.

# 3. Conclusions

In this contribution, RF requirements for gNB like reader are discussed with following observations and proposals.

Proposal 1: OFF power is suggested as -50dBm/MHz.

Observation 1: the residual phase noise is the key factor that will decrease REFSENSE performance.

Observation 2: the phase noise requirement is about 70-80dBc for the case when BW without phase noise cancellation capability. If companies think such requirement is much challeging, we can only consider BS with phase noise cancellation capability in R19.

Proposal 2: use minimum D2R CBW as the boundary for CW unwanted emission requirements. i.e. inside the minimum D2R CBW, we define phase noise if needed, while outside the CBW, we define unwanted emission requirements.

Proposal 3: the same NB-IoT unwanted emission requirement with 33dBm output power is suggested for CW node.

# 4. Reference

- RP-243326, “New Work Item: Solutions for Ambient IoT (Internet of Things) in NR”, RAN1 Vice-chair (Huawei), 3GPP TSG RAN Meeting #106, Dec. 2024.