3GPP TSG-RAN4 Meeting #116	R4-2509720

Bengaluru, India, Aug.25th – 29th, 2025

# Source:	CMCC

# Title:	TP for TR 38.191 section 5.3 Channel bandwidth and 5.4 Channel Arrangement

# Agenda Item:	7.22.3.2

# Document for:	Approval

- Introduction

This TP will focus on section 5.3 Channel bandwidth and 5.4 Channel Arrangement.

- Reference

[1]

- Text Proposal

----- Start of TP -----

## 5.3	Channel bandwidth

5.3.1	R2D Channel bandwidth

### 5.3.1.1	General

The R2D channel bandwidth supports a single reader RF carrier in R2D link at the reader. [The placement of the UE channel bandwidth is flexible but can only be partially within the reader channel bandwidth].

The relationship between the R2D channel bandwidth, the guardband and the transmission bandwidth is shown in figure 5.3.1-1.

Figure 5.3.1.1-1: Definition of channel bandwidth and transmission bandwidth for one reader channel

### 5.3.1.2	R2D Transmission bandwidth

The transmission bandwidth NRB for each reader channel bandwidth and subcarrier spacing is specified in table 5.3.2.-1 .

Table 5.3.1.2-1: R2D Transmission bandwidth NRB for FR1

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB |
| 15 | 1 | 2 | 3 | 4 |

NOTE:	All reader Tx and device Rx requirements are defined based on transmission bandwidth specified in table 5.3.1.2-1.

### 5.3.1.3	Minimum guardband and R2D transmission bandwidth

The minimum guardband for each reader channel bandwidth and SCS is specified in table 5.3.3-1.

Table 5.3.1.3-1: Minimum guardband (kHz) (FR1)

| SCS (kHz) | 200 kHz | 400 kHz | 600 kHz | 800 kHz |
| --- | --- | --- | --- | --- |
| 15 | 10 | 20 | 30 | 40 |

The number of RBs configured in any reader channel bandwidth shall ensure that the minimum guardband specified in this clause is met.

Figure 5.3.1.3-1: reader PRB utilization

### 5.3.1.4	RB alignment

For each reader channel bandwidth, BS transmission bandwidth must fulfil the minimum guardband requirement specified in clause 5.3.3.

[For each numerology, its common resource blocks are specified in clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.]

### 5.3.1.5	BS channel bandwidth per operating band

The requirements in this specification apply to the combination of BS channel bandwidths, SCS and operating bands shown in table 5.3.5-1 for FR1. The transmission bandwidth configuration in table 5.3.2-1 shall be supported for each of the BS channel bandwidths within the BS capability. The BS channel bandwidths are specified for the Tx path.

Table 5.3.5-1: Reader channel bandwidths and SCS per operating band

| NR Band | SCS (kHz) | Reader channel bandwidth (kHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 200 | 400 | 600 | 800 |
| n8 | 15 | 200 | 400 | 600 | 800 |

5.3.2	D2R Channel bandwidth

### 5.3.2.1	General

The D2R channel bandwidth supports a single NR RF carrier in the uplink at device or downlink at the BS. From a BS perspective, different device channel bandwidths may be supported within the same spectrum for transmitting to and backscattering from devices connected to the BS.

From a device perspective, the device is configured with its own device channel bandwidth. The device does not need to be aware of the BS transmission channel bandwidth.

The placement of the D2R channel bandwidth for each device carrier is flexible.

The relationship between the channel bandwidth, the guardband and the transmission bandwidth  is shown in Figure 5.3.2.1-1.

Figure 5.3.2.1-1: Definition of channel bandwidth and transmission bandwidth for one D2R channel bandwidth at BS side

### 5.3.2.2	D2R Transmission bandwidth

The transmission bandwidth for D2R transmission bandwidth at device side is listed as in table 5.3.2.2-1.

Table 5.3.2.2-1: D2R Transmission bandwidth at device side

NOTE:	D2R Transmission bandwidth at device side is calculated using following equation:

D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift

=(2000*(1+R)/Tb)* (1+∣SFO∣)

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)

The transmission bandwidth for D2R transmission bandwidth at BS side is listed as in table 5.3.2.2-2.

Table 5.3.2.2-2: D2R Transmission bandwidth at BS side

NOTE:	D2R Transmission bandwidth at BS side is calculated using following equation:

D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9

=(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9

=(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9

NOTE:	All reader Tx and device Rx requirements are defined based on transmission bandwidth specified in table 5.3.2.2-1 and 5.3.2.2-2.

### 5.3.2.3	Minimum guardband and D2R transmission bandwidth

The minimum guardband for each D2R channel bandwidth at BS side is specified as D2R Transmission bandwidth at BS side * 0.05 and are symetric at each side.

### 5.3.2.4	transmission bandwidth alignment

For each D2R channel bandwidth, D2R transmission bandwidth at device side and BS side must fulfil the minimum guardband requirement specified in clause 5.3.2.3.

[For each numerology, its common resource blocks are specified in clause 4.4.4.3 in [9], and the starting point of its transmission bandwidth configuration on the common resource block grid for a given channel bandwidth is indicated by an offset to “Reference point A” in the unit of the numerology.]

### 5.3.2.5	D2R channel bandwidth per operating band

The requirements in this specification only apply to the operating band n8 with combination of D2R channel bandwidths at device side and BS side. [All the D2R transmission bandwidth at device side in table 5.3.2.2-1 and at BS side at table 5.3.2.2-2 shall be supported within the BS capability].

5.4	Channel arrangement

5.4.1	Channel raster

5.4.1.1	NR-ARFCN and channel raster

The global frequency raster defines a set of RF reference frequencies FREF. The RF reference frequency is used in signalling to identify the position of RF channels and other elements. The granularity of the global frequency raster is ΔFGlobal.

RF reference frequencies are designated by an NR Absolute Radio Frequency Channel Number (NR-ARFCN) in the range [0…3279165] on the global frequency raster. The relation between the NR-ARFCN and the RF reference frequency FREF in MHz is given by the following equation, where FREF-Offs and NRef-Offs are given in table 5.4.1.1-1 and NREF is the NR-ARFCN.

FREF = FREF-Offs + ΔFGlobal (NREF – NREF-Offs)

Table 5.4.1.1-1: NR-ARFCN parameters for the global frequency raster

| Range of frequencies (MHz) | ΔFGlobal (kHz) | FREF-Offs (MHz) | NREF-Offs | Range of NREF |
| --- | --- | --- | --- | --- |
| 0 – 3000 | 5 | 0 | 0 | 0 – 599999 |

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The RF reference frequency for an RF channel maps to a resource element on the carrier. For each operating band, a subset of frequencies from the global frequency raster are applicable for that band and forms a channel raster with a granularity ΔFRaster, which may be equal to or larger than ΔFGlobal.

The mapping between the channel raster and corresponding resource element is given in clause 5.4.1.2. The applicable entries for each operating band are defined in clause 5.4.1.3.

5.4.1.2	Channel raster to resource element mapping

The mapping between the RF reference frequency on the channel raster and the corresponding resource element is given in table 5.4.1.2-1 and can be used to identify the RF channel position. The mapping depends on the total number of RBs that are allocated in the channel and applies to both UL and DL. The mapping must apply to at least one numerology supported by the BS.

Table 5.4.1.2-1: Channel Raster to Resource Element Mapping

|  | NRBmod2=0 | NRBmod2=1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

k,  and NRB are as defined in TS 38.211 [9].

5.4.1.3	Channel raster entries for each operating band

The RF channel positions on the channel raster in each NR operating band are given through the applicable NR-ARFCN in table 5.4.1.3-1, using the channel raster to resource element mapping in clause 5.4.1.2.

Channel raster is defined with ΔFRaster = 2 × ΔFGlobal. In this case every 2th NR-ARFCN within the operating band are applicable for the channel raster within the operating band and the step size for the channel raster in Table 5.4.1.3-1 is given as <2>.

Table 5.4.1.3-1: Applicable NR-ARFCN per operating band for enhanced channel raster

| NR operating band | ΔFRaster (kHz) | Uplink Range of NREF (First – <Step size> – Last) | Downlink Range of NREF (First – <Step size> – Last) |
| --- | --- | --- | --- |
| n8 | 10 | 176000 – <2> – 183000 | 185000 – <2> – 192000 |
| NOTE 1: The channel numbers that designate carrier frequencies so close to the operating band edges that the carrier extends beyond the operating band edge shall not be used. These channel numbers shall also be such that the minimum guard band for each channel bandwidth and SCS specified in Table 5.3.3-1 are met for carriers located at the upper or lower edge of an operating band. |  |  |  |

##### ---End of changes---