3GPP TSG-RAN WG4 Meeting # 116	R4- 2509736

# Bengaluru, India, 25th – 29th August, 2025

Source:	LG Electronics

Title:	Discussion on RRM performance requirements of Rel-19 SBFD

Agenda item:	7.21.5

Document for:	Discussion

Introduction

From last RAN4#115 meeting, RAN4 started to discuss on RRM performance part of Rel-19 SBFD. In this contribution, we provide our views on some RRM performance part issues described in WF[1] and last meeting’s moderator summary[2] as below:

- Issue 3-2-5: L1-CLI-RSSI – accuracy requirements

- Issue 3-3-1: Absolute report mapping

- Issue 3-3-2: Differential report mapping

Discussion

Sub-topic 3-2: Accuracy requirements

Issue 3-2-5: L1-CLI-RSSI – accuracy requirements

- Proposals - Proposal 1a (Samsung, CATT, HW): - Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Proposal 1b (Nokia, CTC): - Same accuracy requirements are defined for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Proposal 2 (Nokia): - RAN4 to introduce performance requirements for CLI measurements for L1-CLI-RSSI measurements. - Proposal 3 (MTK): - Further discuss the side conditions and accuracy requirements of L1-CLI-RSSI. - Recommended WF - Suggest to agree the following: - RAN4 to define accuracy requirements for L1-CLI-RSSI measurements. - FFS on how to define the requirements - Option 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3. - Option 2: other

The performance of RSSI measurement metric is impacted by number of RB or SINR. So, if the minimum RB is guaranteed and SINR environment is similar, then the RSSI measurement accuracy can be similar. Based on this, the R16 accuracy requirements for CLI-RSSI can be re-used for L1-CLI-RSSI.

Proposal 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Sub-topic 3-3: Report mapping

Issue 3-3-1: Absolute report mapping

- Proposals - Proposal 1 (HW): - Re-use Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively. - Recommended WF - Suggest to agree on P1 based on RAN4#114-bis agreements.

In the RAN4#114bis meeting, RAN4 agreed that the LS from RAN1 R1-2501620 is reasonable.

So, the Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 can be re-used for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 2: Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 can be re-used for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Issue 3-3-2: Differential report mapping

- Proposals - Proposal 1 (HW): - Use Table 2 and Table 3 (based on Table 10.1.6.1-2) for reporting mapping of differential L1-SRS-RSRP and L1-CLI-RSSI, respectively Table 2: Reporting mapping of differential L1-SRS-RSRP Reported value; Measured quantity value (difference in measured SRS-RSRP from strongest SRS-RSRP); Unit. DIFF_SRS-RSRP_0; 0≥ΔSRS-RSRP>-2; dB. DIFF_SRS-RSRP_1; -2≥ΔSRS-RSRP>-4; dB. DIFF_SRS-RSRP_2; -4≥ΔSRS-RSRP>-6; dB. DIFF_SRS-RSRP_3; -6≥ΔSRS-RSRP>-8; dB. DIFF_SRS-RSRP_4; -8≥ΔSRS-RSRP>-10; dB. DIFF_SRS-RSRP_5; -10≥ΔSRS-RSRP>-12; dB. DIFF_SRS-RSRP_6; -12≥ΔSRS-RSRP>-14; dB. DIFF_SRS-RSRP_7; -14≥ΔSRS-RSRP>-16; dB. DIFF_SRS-RSRP_8; -16≥ΔSRS-RSRP>-18; dB. DIFF_SRS-RSRP_9; -18≥ΔSRS-RSRP>-20; dB. DIFF_SRS-RSRP_10; -20≥ΔSRS-RSRP>-22; dB. DIFF_SRS-RSRP_11; -22≥ΔSRS-RSRP>-24; dB. DIFF_SRS-RSRP_12; -24≥ΔSRS-RSRP>-26; dB. DIFF_SRS-RSRP_13; -26≥ΔSRS-RSRP>-28; dB. DIFF_SRS-RSRP_14; -28≥ΔSRS-RSRP>-30; dB. DIFF_SRS-RSRP_15; -30≥ΔSRS-RSRP; dB. Table 3: Reporting mapping of differential L1-CLI-RSSI Reported value; Measured quantity value (difference in measured CLI-RSSI from strongest CLI-RSSI); Unit. DIFF_CLI-RSSI_0; 0≥ΔCLI-RSSI>-2; dB. DIFF_CLI-RSSI_1; -2≥ΔCLI-RSSI>-4; dB. DIFF_CLI-RSSI_2; -4≥ΔCLI-RSSI>-6; dB. DIFF_CLI-RSSI_3; -6≥ΔCLI-RSSI>-8; dB. DIFF_CLI-RSSI_4; -8≥ΔCLI-RSSI>-10; dB. DIFF_CLI-RSSI_5; -10≥ΔCLI-RSSI>-12; dB. DIFF_CLI-RSSI_6; -12≥ΔCLI-RSSI>-14; dB. DIFF_CLI-RSSI_7; -14≥ΔCLI-RSSI>-16; dB. DIFF_CLI-RSSI_8; -16≥ΔCLI-RSSI>-18; dB. DIFF_CLI-RSSI_9; -18≥ΔCLI-RSSI>-20; dB. DIFF_CLI-RSSI_10; -20≥ΔCLI-RSSI>-22; dB. DIFF_CLI-RSSI_11; -22≥ΔCLI-RSSI>-24; dB. DIFF_CLI-RSSI_12; -24≥ΔCLI-RSSI>-26; dB. DIFF_CLI-RSSI_13; -26≥ΔCLI-RSSI>-28; dB. DIFF_CLI-RSSI_14; -28≥ΔCLI-RSSI>-30; dB. DIFF_CLI-RSSI_15; -30≥ΔCLI-RSSI; dB. - Recommended WF - Suggest to agree on P1 based on RAN4#114-bis agreements.

According to the agreement regarding the differential report mapping, for differential L1 SRS-RSRP and L1 CLI RSSI, 4-bit resources are allocated and defined 2dB step size with a reference to the largest measured value. So, the suggested report mapping tables of Proposal 1 are reasonable.

Proposal 3: Differential L1-SRS-RSRP and L1-CLI RSSI report mapping tables of P1 are reasonable.

Summary

In this contribution, we provide our views on the remain open issues on RRM impacts of SBFD operation described in WF [1] and more detail in last meeting’s moderator summary [2]. Our proposals are as below.

Proposal 1: Re-use R16 accuracy requirements for CLI-RSSI for L1-CLI-RSSI, for Method#1 with resource type#1 and resource type#2, as well as for Method#3.

Proposal 2: Table 10.1.22.1.2-1 and Table 10.1.22.2.2-1 can be re-used for reporting mapping of absolute L1-SRS-RSRP and L1-CLI-RSSI, respectively.

Proposal 3: Differential L1-SRS-RSRP and L1-CLI RSSI report mapping tables of P1 are reasonable.

Reference

[1] R4-2508274, “WF on RRM requirements for NR_duplex_evo”, Huawei, HiSilicon

[2] R4-2505568, “Topic summary for [115][219] NR_duplex_evo”, Moderator (Huawei, HiSilicon)