3GPP TSG-RAN WG4 Meeting # 116	R4-2509767

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on UE-initiated/event-driven beam management on Rel-19 MIMO phase 5

Agenda Item:	7.20.4.1

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the RRM requirement of UE-initiated/event-driven beam management for Rel-19 MIMO phase 5.

- Discussion

2.1 Measurement reporting delay

Issue 1-2: Components of Event triggered measurement reporting delay? - Agreement: If the measurement periods are different for current beam and new beam, the TL1_measurement period is not larger than the maximum of the L1 measurement periods to measure both current beam and new beams; Note: The agreement does not imply that all measurements on individual reference signals are after the event is met over air; UE shall perform L1-RSRP measurement based on current beam and new beams L1-RSRP measurement period by assuming TReport =0

RAN1 agreement is as below:

Agreement On cross-CC beam report measurement for UE-initiated/event-driven beam reporting, regarding the evaluation periodicity for determining event instance, the periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s). Above applies for all events.

Observation 1: From RAN1, the same evaluation periodicity is defined for all events.

Even with a common evaluation periodicity, the L1 measurement period per beam can still differ (e.g., due to different sharing). Therefore, RAN4’s rule—bounding TL1_measurement by the max of the per-beam L1 measurement periods—still applies.

Proposal 1: The RAN1 agreement on a common evaluation periodicity does not affect RAN4’s existing agreement that the L1 measurement delay component is bounded by the maximum of the per-beam measurement periods.

2.2 Whether to consider M>1

#### Issue 1-3: Whether RAN4 to specify requirements for no time window based or cover both no time window based and with time window based with eventInstanceCount-r19? - Proposals - Option 1: (Qualcomm, Apple, LGE) - Do not define requirements for the window-based approach whether the event has occurred M times. - Option 1a: (Qualcomm) - RAN4 to discuss whether to send LS to RAN1 summarizing the RAN4 agreements for the measurement periods and asking for further clarity of the RAN1 agreement - Option 1b: (Apple) - Add note longer delay is expected if eventInstanceCount-r19 >1 is configured - Option 2: (China Telecom, Nokia, Xiaomi, Samsung, CMCC, Huawei, Ericsson, ZTE, MTK) - Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19 - Option 2a: (Nokia) - The delay takes into account that the UE shall send the report after M event occasions have occurred within the window/timer - Option 2b: (Huawei) - When the conditions is met over the air and it is kept fulfilled, UE shall be able to trigger the event TL1-RSRP_event triggered_measurement_Period * C + Tfirst UL channel, where C is configured counter - For time window-based event triggering, one event instance shall be evaluated based on latest results from both new beam and current beam. i.e. one result shall not be used to evaluate whether event instance is met for multiple instance. - Option 2c: (Ericsson) - After the first event entering condition is met, UE starts the timer with length T_window and after the T_window+ TFirst UL channel, UE should send the indication on the first UL channel if the event has been met M number of times in the T_window - Option 2d: (MTK) - Alt1: Measurement reporting delay can re-use the same principle as TTT for the configurable time window. - Alt2: Measurement reporting delay is TL1_measurement period * C + Tfirst UL channel given that measurement reporting delay ≤ Ttime_window, where, - TL1_measurement period: L1-RSRP measurement period in event triggered measurement reporting for either SSB or CSI-RS. - C: total number of measurement period used for event triggered condition is met (At least event instances is greater than or equal to an M (event instances threshold) configured by NW. - Ttime_window: time window is configured by NW. - Tfirst UL channel: the uncertainty time to send beam reporting notification on 1st PUCCH. - Recommended WF - First decide from two options: - Option 1: No requirements for with time window based with eventInstanceCount-r19>1 - Option 2: Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19

In the context of event-triggered L1-RSRP reporting, the 3GPP RAN1 has introduced a new reporting quantity: the Condition Met Indicator (CMI). This 1-bit indicator is designed to provide an efficient and reliable mechanism for the UE to inform the gNodeB about whether a specific beam has met the event-triggering conditions within a defined time window. The interpretation of the CMI is as follows:

Agreement On UE-initiated/event-driven beam reporting, support the following interpretation on each of the codepoints of the ‘1-bit’ condition met indicator as follows: - ‘0’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window doesn’t reach the configured number M. - ‘1’ – indicating that the Event-2 instances for corresponding CRI/SSBRI within the time window reach the configured number M. FFS: whether/how to introduce the ‘1-bit’ condition met indicator for Event 7.

In the current meeting, multiple companies have further elaborated on this behavior with proposals that define explicit delay models for time window-based triggering. These discussions confirm that time-window-based triggering is no longer hypothetical—it is actively shaping UE behavior and performance expectations.

Therefore, to maintain alignment with RAN1 framework and provide clear, testable RAN4 behavior, it is essential to specify requirements that cover both:

- Legacy no-window behavior, and

- Time-window-based behavior with configurable eventInstanceCount-r19.

This enables consistent interpretation of reporting delay, supports conformance testing, and reflects real-world deployment configurations.

Proposal 2: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

2.3 Reduction of beam application delay

- Issue 1-5: Whether to specify new unified TCI switching delay requirements? - Agreement: The existing requirements for TCI state switch delay are applicable for UE-initiated/even-triggered beam management; Further discuss additional enhancements in the next meeting, and the discussion does not impact the WI core part completion

Since RAN1 has closed the topic without conclusion, RAN4 will not introduce new delay requirements and considers the item closed for this WI.

Proposal 3: Don’t specify new unified TCI switching delay requirements.

2.4 Indicated TCI states is changed

#### Issue 1-6: Whether/How to specify RAN4 requirements if indicated TCI states is changed during the L1 measurement time? - Proposals - Option 1: (Nokia, Ericsson) - No need to capture this scenario in RAN4 requirements - Option 2: (Xiaomi, Samsung) - The valid L1 evaluation of event-triggered L1-RSRP should be after the MAC-CE command or DCI. If the event has been triggered, the L1 evaluation time should be restart. - Option 3: (Apple, MTK) - Add the applicability rule of event triggered L1 measurement: - The event trigger condition shall be maintained from when condition is met over the air until end of measurement period at the UE. - The reporting delay requirements are applicable when indicated TCI state remains the same through the measurement period

The RAN1 agreement is as below, it states that if measured current beam RS is updated based on indicated TCI state, the counting for all new beams will be reset.

Agreement Regarding triggering event determination for Event 2, on resetting the counting, the following modification on the agreed Candidate #2 in RAN1#120 is supported. - Candidate#2: [The measured current beam RS is updated based on] indicated TCI state is updated; - In such case, the UE needs to reset the counting for all new beams. - FFS: Further details on the update (if necessary).

Observation: The measured current beam RS is updated based on indicated TCI state, in such case, the UE needs to reset the counting for all new beams.

Proposal 4: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart.

- Conclusion

In this contribution, we provide the following proposals：

Observation 1: From RAN1, the same evaluation periodicity is defined for all events.

Proposal 1: The RAN1 agreement on a common evaluation periodicity does not affect RAN4’s existing agreement that the L1 measurement delay component is bounded by the maximum of the per-beam measurement periods.

Proposal 2: RAN4 to Specify requirements to cover both cases of no time window based and with time window based with configurable eventInstanceCount-r19.

Proposal 3: Don’t specify new unified TCI switching delay requirements.

Proposal 4: If indicated TCI states is changed during the L1 measurement time, the L1 evaluation time should restart.

- Reference