3GPP TSG-RAN WG4 Meeting # 116	 R4-2509768

Bengaluru, India, August 25th – 29th, 2025

Source: 	           Xiaomi

Title: 	Discussion on other RRM requirement on Rel-19 MIMO phase 5

Agenda Item:	7.20.4.2

Document for:	Discussion

# 1 Introduction

In this contribution, we will mainly discuss the following issues for Rel-19 MIMO phase 5.

- Two TAs in Asymmetric DL sTRP/UL mTRP

- Discussion

The agreement in last meeting is as below:

For asymmetric DL sTRP/UL mTRP deployment with two TAs, for the scenario that UE is not configured with PL offset in joint/UL TCI state(s) and UE may expect to receive SSB from UL TRP(s), for the case UE is configured with SSB-MTC-additionalPCI, the reference point for PRACH transmission triggered by PDCCH order, - if “PRACH association indicator” in DCI format 1_0 is 0, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL TCI state(s) of the reference cell associated with the first TAG. - if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or [UL] TCI state(s) of the reference cell associated with the second TAG. For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState [or UL TCI states] having the same TAG as the uplink signal. For each TAG the uplink transmission timing takes place (NTA + NTA offset) x Tc before the reception of the downlink reference point

SSB-based reference limitation:

- In some deployments, especially in UL-only TRP configurations, the only downlink reference signal available is the SSB. However, SSB cannot be configured in a DL TCI state (only used as a source RS), which makes it impossible to meet the current requirement if only DL TCI states are allowed.

UL TCI states can reference valid RS:

- UL TCI state(s) can include reference signals as source RS for the purpose of uplink operations. Therefore, when a UE is not configured with DL TCI state (especially for UL-only TRPs), it still has valid RS information via UL TCI state(s) to derive the timing reference.

Unified treatment of DL and UL TCI states improves flexibility:

- Allowing both DL and UL TCI states provides a unified and consistent timing reference framework across different TRP configurations (DL-only, UL-only, or asymmetric), thereby enabling better support for inter-cell multi-TRP and asymmetric deployments.

Backward compatibility and alignment with deployment scenarios:

- This proposal maintains compatibility with current mechanisms while enabling practical deployment flexibility—especially critical in asymmetric DL/UL TRP scenarios or where SSB is the only available reference.

Proposal 1: Update the specification to remove the bracket around "UL TCI state(s)" and make the requirement explicitly include both DL and UL TCI states as valid sources for timing reference:

- if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG.

- For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or UL TCI states having the same TAG as the uplink signal.

- Conclusion

In this contribution, we provide the following proposals：

Proposal 1: Update the specification to remove the bracket around "UL TCI state(s)" and make the requirement explicitly include both DL and UL TCI states as valid sources for timing reference:

- if “PRACH association indicator” in DCI format 1_0 is 1, the reference timing is the first detected path (in time) of one of the corresponding downlink reference signal(s) of DL or UL TCI state(s) of the reference cell associated with the second TAG.

- For inter-cell multi-TRP, for PUCCH/PUSCH/SRS: For each TAG, the reference point for PUCCH/PUSCH/SRS, is the first detected path (in time) of one of the corresponding downlink reference signal(s) of the reference cell associated with one of the DLorJointTCIState or UL TCI states having the same TAG as the uplink signal.

- Reference