3GPP TSG-RAN WG4 Meeting #116															R4-2509772

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:			7.6.3.2

Source: 	Xiaomi

Title: 						RF requirements for NR-NTN

Document for:	Discussion

- Introduction

Rel-19 NTN agreed to introduce HPUE for NR-NTN operation for uplink coverage improvement.

High power UE (HPUE) for NTN - Conduct the coexistence analysis based on TR 38.863 for the above example bands and corresponding power classes - Evaluate the impact on the uplink performance of TN (Terrestrial network) - Example bands include bands n256 and 256, n255 and 255 - PC2, PC1.5 (for NR-NTN only) and PC1 will be evaluated for ACLR and ACS - For NR-NTN and IoT-NTN: coexistence studies shall be carried out under the same conditions, including geographical separation between NTN UE and Terrestrial Networks (TN), as the co-existence studies in Rel-17 and Rel-18 - Specify high power UE (HPUE) for NR-NTN (Non-Terrestrial Networks) and IoT-NTN (NB-IoT and eMTC based NTN) in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - Support UE with power class 2 (PC2, 26 dBm) that applies to both handheld and non-handheld based on the co-existence study. - Support other power classes based on the co-existence and feasibility study - Non-hand held UE is prioritized - Specify RF requirements for all considered power classes for NR-NTN and IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands - Necessary Tx requirements, at least including - Investigate and if necessary, specify the solution to address potential SAR (Specific Absorption Rate restriction) requirements for handheld UE - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives

In this contribution, views on remaining open issues were provided.

- Discussion

Issue 1-1: Configured Tx power equation Agreement: - Agree to clarify and correct the configured transmitted power equation for NTN - [∆TC,c], ∆TIB,c , [∆TRxSRS] are not applicable to Rel-19 NR-NTN UE - FFS ΔPPowerClass - For ΔPPowerBoost, Rel-18 power boosting should be applicable to NR-NTN. - Regarding how to address these in CR drafting, following approach is agreed. - By referring to 38.101-1 and then exclude non-applicable parameters, e.g. ‘except for...’

∆TIB,c is the additional tolerance for serving cell c for CA, SUL EN-DC etc. which not applicated for NR-NTN UE. Similarly, ∆TRxSRS requirement from the SRS transmission occasions with usage in SRS-ResourceSet for NR-NTN UE not applicated for NR-NTN UE yet. Therefore, the values which reflect the effect from specific features can be removed from the Configured transmitted power equation in 38.101-5 for NR-NTN UE. These can be added in the future release when RAN4 discuss the features for NR-NTN UE.

Proposal 1:  Remove the square bracket of ∆TC,c and ∆TRxSRS, and not applicable to Rel-19 NR-NTN UE.

Issue 1-10: PC 1.5 UE architecture assumption Agreement: - PC1.5 for non-handheld NR-NTN UE is primarily considered with 2Tx in Rel-19 - PC1.5 with 1Tx is not precluded but with low priority. It can be treated once PC1.5 with 2Tx work is done.

NTN PC1.5 with 2Tx is the high priority and it should be finished on time in this release. Meanwhile, PC1.5 with 1Tx means restricted design requirement for the devices. More discussions are needed for the PC1.5 with 1Tx UE.  Therefore, to consider the time issue of Rel-19, we can discuss PC1.5 with 1Tx in Rel-20.

Proposal 2: Consider and study PC1.5 with 1Tx in Rel-20.

Issue 1-12: Power boosting for PC2 Way forward: - Further discuss evaluate the PC2 NTN power boosting option with low priority if time allows, and not target to define in Rel-19. - Further discuss following assumptions: - PC2 1Tx PA using existing TN impairment and targeting inner third allocations. - Applicable to both TN and NTN. - FFS applicability of such boosting to multi-Tx based on PC2 PAs

During the release 19 HPUE NTN discussion in RAN4, power boosting issue was not fully discussed and studied by the meeting. RAN4 also considered it as the low priority. Therefore, considering the time issue of this WI, and the difficulty of implementing. These assumptions can be discussed in the Rel-20 NTN topic.

Proposal 3: Do not discuss the power boosting for PC2 during Rel-19 and move the necessary topics to Rel-20.

- Conclusion

Proposal 1:  Remove the square bracket of ∆TC,c and ∆TRxSRS, and not applicable to Rel-19 NR-NTN UE.

Proposal 2: Consider and study PC1.5 with 1Tx in Rel-20.

Proposal 3: Do not discuss the power boosting for PC2 during Rel-19 and move the necessary topics to Rel-20.

Reference

[1] R4-2508008, WF on [115][128] NR_IoT_NTN_HPUE_part1, Samsung

[2] R4-2507567, Topic summary for [115][128] NR_IoT_NTN_HPUE_part1, Samsung