3GPP TSG-RAN WG4 Meeting #116				                                           R4-2509795

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.3.1

Title: 	Discussion on OD-SSB requirements

Source: 	Xiaomi

Document for:		Discussion

- Introduction

In RANP #103 meeting, the WID [1] for R19 NES was agreed and the objectives for OD-SSB are self-contained as below.

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

And in the last RAN4 meeting, the agreements for NES RRM were achieved in [4]. Therefore, in this discussion, we provided more analysis on the open issues of Rel19 NES [2][4].

- Discussion

  - General On-demand SSB(OD-SSB) requirements

In the previous RAN4 meetings, whether the OD-SSB based cell identification before deactivated SCell measurement can be skipped were discussed but without any agreements.

R4#114bis discussion Moderator suggests the group to check the following options(except the Case 1 Option 2). - The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios: - Case 1: No always-on SSB on the cell - Option 1: OD-SSB Scell being measured is contiguous to an active serving cell in the same band - Option 2-1: UE already finished the OD-SSB measurement before, and the OD-SSB deactivated period is within [Xs] and then the OD-SSB is transmitted again with the same spatial reception parameter. - Other detail condition is FFS. (Note: this option has already covered in issue 1-2-3) - Option 2-2: UE has sent a valid measurement report based on OD-SSB for the SCell being activated [5]s before, and then the OD-SSB is transmitted again with the same spatial reception parameter same transmission power, and on the same frequency carrier. - Other detail condition is FFS. - Option 3: Extend referenceCell to OD-SSB SCell - Other option is not precluded - Case 2: Always-on SSB on the cell - Option 1: Cell identification can be always skipped - Option 1a: Cell identification can be always skipped in scenario 2 - Option 2: follow SCell activation known condition - Other option is not precluded

Firstly, for case1, given RAN1 agreements below, we can conclude that：

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

R1#118 discussion Agreement: For a cell supporting on-demand SSB SCell operation, at least the following is supported - On-demand SSB on the cell is not located on synchronization raster. - On-demand SSB on the cell is non-cell-defining SSB FFS: Additional support of OD-SSB for CD-SSB located on sync-raster

On the other hand, OD-SSB based measurements for the deactivated SCell can be feasible with the valid timing and cell information. That is the pre-required information can be obtained from the serving cells associated with the SCells with OD-SSB if they can meet the specific conditions (e.g. adjacent together in both frequency domain and spatial domain).

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Also the specific known condition for the cell identification can be feasible. But if such known condition can’t be satisfied, how UE can obtain the necessary timing information shall be considered as well. In our view, in case of unknown condition, UE can also rely on the timing of associated serving cell.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Therefore, in order to simplify RAN4 specification, we prefer to define such condition for case 1 by Option 1 directly.

Secondly, for case2, since there accompanied always-on SSB is available, it is quite simple for UE implementation to detect/identify SCell to be measured with the always-on SSB indeed.

Thus, in short summary we can propose that:

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - OD-SSB Scell being measured is contiguous to an active serving cell in the same band with the same known conditions as the legacy NR cell identification

- Case 2: Always-on SSB on the cell

  - Case 1- OD-SSB based deactivated SCell L3 measurement

When the OD-SSB was triggered for SCell activation, the periodicity of OD-SSB is expected to be short as possible to enable UE can complete the measurement quickly which can save the network power. That is the exact meaning of “fast L3 measurement” in RAN4 agreements [4]. However, for the deactivated SCell, the periodicity of OD-SSB can be much sparser than these for the SCell activated or to be activated. As a result, UE need not to measure the OD-SSB so frequently to minimize UE’s power consumption. This is what “slow measurement” mean in comparison with the former one.

Thus, in the previous RAN4 meeting, the harmonized solution to balance the power consumption and the reliable measurement results via OD-SSB occasions is to restrict the periodicity of measurement based OD-SSB in the fast mode window (“Tfast”) and after that (e.g.“Tslow”) UE can relax the measurement with “measCycleSCell” or stop the measurement as agreements below.

Figure 1. OD-SSB measurement stages [4]

And in the last RAN4 meeting, the agreements on how to define the time window (Tfast) and corresponding UE measurement behavior was achieved in [4].

- issue 2: OD-SSB based deactivated SCell measurement – Procedure

For the OD-SSB based deactivated SCell measurement RAN4 has reached some agreement and open issues below.

Issue 1-1-2: OD-SSB based deactivated SCell measurement – Procedure Sub-issue 1: The start point of T2 Agreement: - The time point of OD-SSB deactivation. Sub-issue 2: The minimum interval(T2) between two FMWs Agreement: - For FR1, 5*max (measCycleSCell, DRX cycles) - For FR2, 4s for power class 1/5 and 3s for power class 2/3/4 Sub-issue 3 –UE’s behaviour when OD-SSB further activation/reconfiguration within T2 Agreement: - The UE is not required to perform “fast measurement” but follows legacy deactivated SCell measurement requirement within T2. Sub-issue 4 – UE’s behaviour after T2 when OD-SSB further activation/reconfiguration occurs within T2 between two FMWs The issue can be further discussed based on contribution driven, and the discussion will not impact the WI completion. - Option 1: When OD-SSB further activation/reconfiguration occurs within T2, the UE should perform ‘fast measurement’ immediately after T2. - Option 2: when OD-SSB further activation/reconfiguration occurs within T2, UE should follow legacy deactivated SCell measurement after T2. - Other option is not precluded.

Figure 2. UE behavior after T2

In principle, the UE behavior over all time window shall be considered jointly. Since as illustrated in Figure 2 after T2, when the OD-SSB was re-triggered, in order to further reduce UE power consumption, UE can follow the sparser measurements in comparison with the measurement in Tfast if the previous measurement result is still valid. This is similar with the known condition for the SCell activation in TS38.133 [3]. Otherwise, UE can follow the same UE behavior as that within Tfast.

Proposal 2: When the OD-SSB was re-activated during the last [5] seconds after deactivation, UE can follow the legacy deactivated SCell measurement cycle after T2.

- Sub-issue 4: OD-SSB in FMW collided with MG

In the last RAN4 meeting the issue for OD-SSB colliding with MG was discussed. And the following WF was achieved:

Issue 1-1-1: OD-SSB based deactivated SCell measurement – requirement in FMW Sub-issue 2: OD-SSB in FMW collided with MG - Option 1: UE is allowed to skip the OD-SSB measurement if FMW collides with MG occasion. - Option 2: UE to prioritize the OD-SSB carrier/layer within FWM

Since the handling on the MG colliding with other signal is extremely complicated issue before, we also prefer to reuse the legacy rules. For an instance, in the legacy measurement requirements, we supposed that the intra-frequency measurement will be dropped. Thus, if we following the same principle the measurements on the deactivated SCell based on OD-SSB shall be discarded.

Figure 3. Fast measurement window colliding with MG

Proposal 3: If OD-SSB within the FMW overlapped with the measurement gap occasion in time domain, UE can deprioritize OD-SSB measurement.

But on the other hand, in the previous RAN4 discussion on the length of FMW, such impacts due to the collision with MG was missed.

Agreement: - The time window is defined with Tuncertain plus actual measurement duration. - Tuncertain equals Tmin(RAN1) plus additional processing/preparation time, if it allows. - The actual measurement duration equals min(Treport , Tidentify) from the end of uncertain time. - Treport is the first OD-SSB based L3 measurement reporting - Tidentify equals the minimum RAN4 OD-SSB meas. period requirement as follow. - A detected cell: OD-SSB based measurement period - A newly detectable cell: OD-SSB based cell identification period No OD-SSB measurement requirement applies in fast window if NW deactivates the OD-SSB earlier than the end of min(Treport, Tidentify).

Observation 4: There is impacts on the effective measurement window length of FMW because of the collision with the MG.

Therefore, we suggest that:

Proposal 4: RAN4 can clarify the potential impacts on the length of FMW agreed in R4#114 due to OD-SSB collision with MG.

- Sub-issue 4: OD-SSB measurement in FMW(Case 1-2): Multiple active OD-SSB carriers

Issue 1-4-1: OD-SSB based multiple SCells activation R4#115 Agreement - RAN4 to define the requirements in the case of multiple OD-SSB SCells. - It doesn’t impact the WI’s completion. - The scope can be limited as follows: - In deactivated SCell stage, the time periods from OD-SSB activation to deactivation among multiple OD-SSBs’ frequency layers are non-overlapping. - [For multiple Scells activation, multiple Scells are purely OD-SSB’s Scells. - OB-SSB is assumed to be activated on all the multiple Scells to be activated before Scell activation command.]

In the previous meeting, whether the requirements for the multiple active OD-SSB carriers scenarios shall be defined was discussed. In our views, if the scenario is limited to multiple OD-SSBs activation duration are non-overlapping, there is no any different requirements for the measurement delay needed to be defined.

Observation 5: For the requirements in the case of multiple OD-SSB SCells, ideally the requirements for single OD-SSB can be reused if there are not any overlapping in time domain.

But the more practical condition to define the overlapping condition shall be considered. That is RAN4 still need to discuss some side conditions to apply these requirements.  For an example, how to justify the overlapping among these OD-SSBs in the multiple carriers shall be addressed in RAN4.

Generally, if the extra processing time for OD-SSB activation/deactivation needed, the minimum proximity time between the two adjacent OD-SSBs in the different carriers shall be specified at least.

Figure 4. Multiple activated OD-SSB carriers

Proposal 5: The condition on the overlapping among the multiple active OD-SSB carriers shall be defined in RAN4.

  - Case 2- OD-SSB based deactivated SCell L3 measurement

In the last RAN4 meeting, on how to define the case2 OD-SSB measurement requirements was discussed and following open issues are left.

Issue 1-2-2: Requirement for RAN1 Alt Time-C2 Agreement - Option 1: RAN4 to define the delay requirement only based on the OD-SSB periodicity. - Option 2: RAN4 to define the delay requirement only based on the OD-SSB periodicity only if - The periodicity of AO-SSB and OD-SSB are same - It doesn’t impact the WI’s completion. Issue 1-2-3: Requirement for scenario 3 (OD-SSB and AO-SSB are within different frequencies) Sub issue 2: Requirement for scenario 3 Agreement: RAN4 will further discuss whether and how to define the requirements for scenario 3, and meanwhile the discussion will not impact the WI completion.

In RAN1 #120 meeting, RAN1 agreed to introduce the scenario 3(OD-SSB and AO-SSB are within different frequencies).

RAN1 #120 Agreement Regarding the relation in terms of frequency location (i.e., center frequency) between the always-on SSB and on-demand SSB, - Alt 1: If always-on SSB is CD-SSB on a synchronization raster, the frequency location of on-demand SSB is different from the frequency location of always-on SSB. - On-demand SSB is not on sync raster - AO-SSB and OD-SSB are located in the same BWP - FFS: Additional conditions - Subject to separate UE capability - Note: UE is not required to measure both AO-SSB and OD-SSB

In RAN2 #129b meeting, an LS related to scenario 3 was sent to RAN4 as follow.

In RAN2#129b, RAN2 discussed how to configure L3 measurement when always-on SSB (A-SSB) and on-demand SSB (OD-SSB) have different center frequency, and made the following Working Assumption: Working assumption: - When A-SSB and OD-SSB have different center frequency, introduce a new servingCellMO in ServingCellConfig to indicate MO of OD-SSB. Note: The WA was made from RRC signaling perspective. RAN2 relies on RAN4/1 to decide whether to support the scenario when A-SSB and OD-SSB have different center frequencies or not.

In our views, the requirements when UE can be configured with activated OD-SSB and AO-SSB in the different carriers can be similar as the intra-frequency measurement because MOs associated with OD-SSB and AO-SSB are within serving cell. From RAN4 perspective, there are still several issues to be clarified.

Observation 6: if UE support the scenario 3, UE needs to support the CA combination of the carrier f1 and f0 at least.

Figure 5. OD-SSB and AO-SSB transmission in scenario 3

Proposal 6: UE measurement on the both MO shall be based on the gap-less measurement

Proposal 7: Beside the RAN2 supporting scenario 3, the CA combination capability shall be defined in RAN4.

  - OD-SSB based activation

In the last RAN4 meeting, on how to define the necessary requirement on SCell (de)activation with OD-SSB was NOT discussed. And the open issue left for this in R4#113 wad captured below.

<R4#115> Issue 1-3-1: The known cell condition in Case 2 Agreement - The current known SCell activation delay methodology can be reused in Case 2 at least for Alt Time-C1 with the following combination. - When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB. - FFS whether apply to Alt Time-C2, if the requirement is defined. - FFS whether apply to scenario 3, if the requirement is defined. - FFS whether additional AGC time should be included in the known SCell activation delay requirement for scenario 3, if defined.

For the detailed RRM requirements updates, in our view it needs more discussion under the different conditions and scenarios. For an example, as illustrated in Figure 6 below, for the case of “Scenario #2 and Case #1“ and known Scell activation case, if the first SSB transmitted after OD SSB trigger command can located before the time “n + THARQ+5ms+TfirstSSB +margin”, UE can follow the current behavior for Scell activation and corresponding requirements defined in TS38.133 for known cases can be reused. Otherwise, the requirements in current spec needs to be updated. However, for the unknown case since the more than one SSB needed for the SCell timing and frequency synchronization, the current requirements could not be reused with the condition above for known cases indeed. Especially for Alt Time-C2, since the OD-SSB and AO-SSB can be aperiodic UE’s synchronization before receiving the SCell activation command can be rely on either AO-SSB or OD-SSB. In some cases, it is possible that only one type of SSB available. Thus, the condition defined for Alt Time-C1 below seems be feasible for Alt-C2 as well

- When the SSB measured for known cell condition is AO-SSB or OD-SSB or the combine of AO-SSB and OD-SSB, and the SSB used in SCell activation procedure is AO-SSB, OD-SSB or the combine of AO-SSB and OD-SSB.

Figure 6. SCell activation based on AO-SSB and OD-SSB

And as the known condition for Alt Time-C1 was agreed in the last meeting, in our view, the similar condition can be reused for Alt Time-C2.

Proposal 8: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions in TS38.133 [3] can be reused for Case 2 Alt Time-C2.

  - Interruption

In the last meeting the following CR split works related to interruption requirements in TS38.133 was agreed.

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |

Additionally, for the interruption during measurement on deactivated SCC, when the deactivated SCC measurement based on OD-SSB, the potential impacts on current requirement in TS38.133 shall be investigated as mentioned in [5][6]. For an example, the “measCycleSCell” parameter used to define the legacy deactivated SCell measurement [3, TS38.133] could not available/used when SCell is activated after the fast mode window.

However, according TS38.331, measCycleSCell can be used for the SCell in the deactivated state only.

Therefore, we suggested that:

Proposal 9: The change on the clause 8.2.2.2.3 of TS 38.133 is needed. The CRs works for interruption requirements in TS38.133 can be updated as:

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |
| 8.2.2.2.3 | Interruptions during measurement on deactivated SCC |

Accordingly, we can also directly discuss the draftCR provide in [7].

- Conclusion

In this contribution, serval issues related to RRM in Rel19 NES are discussed. The observations and proposals can be summarized as:

General On-demand SSB(OD-SSB) requirements

Observation 1: Since OD-SSB are not located at synchronization raster, UE can’t perform the cell identification with OD-SSB for the initial access.

Observation 2: For measurements on the deactivated SCell with OD-SSB UE can know the correct timing and other SSB index information from the other serving cell (e.g. PCell) if their timing and spatial domain information are associated together.

Observation 3: UE can measure the OD-SSB without the cell identification by:

- The known conditions whether the valid synchronization timing can be obtained based on the previous measurements results or

- other serving cell relationship

Proposal 1: The OD-SSB based cell identification before deactivated SCell measurement can be skipped in the following scenarios:

- Case 1: No always-on SSB on the cell

  - OD-SSB Scell being measured is contiguous to an active serving cell in the same band with the same known conditions as the legacy NR cell identification

- Case 2: Always-on SSB on the cell

Case 1- OD-SSB based deactivated SCell L3 measurement

Proposal 2: When the OD-SSB was re-activated during the last [5] seconds after deactivation, UE can follow the legacy deactivated SCell measurement cycle after T2.

Proposal 3: If OD-SSB within the FMW overlapped with the measurement gap occasion in time domain, UE can deprioritize OD-SSB measurement.

Observation 4: There is impacts on the effective measurement window length of FMW because of the collision with the MG.

Proposal 4: RAN4 can clarify the potential impacts on the length of FMW agreed in R4#114 due to OD-SSB collision with MG.

Observation 5: For the requirements in the case of multiple OD-SSB SCells, ideally the requirements for single OD-SSB can be reused if there are not any overlapping in time domain.

Proposal 5: The condition on the overlapping among the multiple active OD-SSB carriers shall be defined in RAN4.

Case 2- OD-SSB based deactivated SCell L3 measurement

Observation 6: if UE support the scenario 3, UE needs to support the CA combination of the carrier f1 and f0 at least.

Proposal 6: UE measurement on the both MO shall be based on the gap-less measurement

Proposal 7: Beside the RAN2 supporting scenario 3, the CA combination capability shall be defined in RAN4.

OD-SSB based activation

Proposal 8: For the SCell activation/deactivation requirements with OD SSB, the UE known or unknow conditions in TS38.133 [3] can be reused for Case 2 Alt Time-C2.

Interruption

Proposal 9: The change on the clause 8.2.2.2.3 of TS 38.133 is needed. The CRs works for interruption requirements in TS38.133 can be updated as:

| 8.2.2.2.1 | Interruptions at SCell addition/release |
| --- | --- |
| 8.2.2.2.2 | Interruptions at SCell activation/deactivation |
| 8.2.2.2.3 | Interruptions during measurement on deactivated SCC |

- References

- RP-240830

- R4-2508277 WF on RRM requirements for Netw_Energy_NR_enh_Part1

- TS38.133 v19.1.0

- R4-2420098	WF on RRM requirements for Netw_Energy_NR_enh_Part1

- R4-2418282	Topic summary for [113][224] Netw_Energy_NR_enh_Part1

- R4-2503124

- R4-2509799