3GPP TSG-RAN WG4 Meeting #116	R4-2509807

Bengaluru, India, August 25th – 29th, 2025

Source: 	Xiaomi

Title: 	Discussion on AIoT device 1 RF requirements

Agenda item:	7.22.3.2

Document for:	Discussion

- Introduction

After much online and offline discussion, there’re still some remaining issues for the device RF requirements. This contribution provides our views on the open issues.

- Discussion

Generally, there’re no real product inputs from our company for the analysis for the AIoT device 1 RF requirements. So our analysis are based on our knowledge and some references. The general principles for our views are based on our understanding that the device 1 is low cost and low power device, so it’s not good to define too many and too complicated requirements.

2.1 Transmit output power

For transmit output power, the open issues are listed as followings,

  - FFS on how to determine device’s peak antenna gain direction

  - FFS on whether to measure multiple directions

  - FFS for BPSK

For the directions, it’s already agreed that the device’s peak antenna gain direction is based on declaration i.e. same peak gain direction for backscattering loss testing for SEM and emission requirement. So there’s no need to discuss the peak antenna gain direction issue. For the multiple directions issue, we have the following proposal based on the principles to control the cost for device 1.

Proposal 1: Only peak direction requirement for transmit output power is defined.

For the output power for BPSK modulation, in theory, the output power is 3 dB greater than QPSK modulation. We’re not sure if some implementation margin is needed but it is not very difficult to decide the value if the power for OOK modulation is agreed.

2.2 SFO requirement

We have the following proposal,

Proposal 2: No explicit SFO requirement is needed.

2.3 SEM and spurious emission requirement

In theory, for OOK and BPSK signal, the power ratio between the main lobe and the n-th side lobe is 20log10(n). It applies to different R number. The following two figures show the spectrum for R=1 and R=4.

Figure 1: Spectrum for R=1

Figure 2: Spectrum for R=4

Figure 3: Spectrum for R=8

The value of the SEM or spurious requirements is to set the emission limit for the transmitter then limit the interference to other system or other devices/readers. We proposed to test the case without SFS because we think there’s no extra RF efforts to be done for SFS case. However, it can be seen from Figure 1, Figure 2 and Figure 3 that the spectrum without SFS is very different with the spectrum with SFS. The sidelobe attenuations for different repetitions are similar, but the sidelobe’s positions are different for different repetitions. So if the emission boundary is set relative to R=1 case, it may not apply to other R numbers.

Observation 1: If the emission boundary is set relative to the device for R=1 case, it may not apply to other R numbers.

So it may be good to check the regulation. The following requirements are copied from ETSI EN 302 208.

The channel plan for lower band and upper band are copied in the following figures.

For the lower band, the 865 MHz to 868 MHz, four channels are specified. The bandwidth of each high power channel shall be 200 kHz. The four channels shall be spaced at equal intervals of 600 kHz. For the upper band, 915 MHz to 921 MHz, four channels are specified. The bandwidth of each high power channel shall be 400 kHz. The four channels shall be spaced at equal intervals of 1.2 MHz. After reviewing the channel arrangement, the SEM/emission are defined according to the channel spacing, i.e. the emission level should be in a limited level on other channels. For AIoT, due to the SFS mechanism, it’s difficult to follow the logic of ETSI EN 302 208.

Figure 4 is a candidate for the SEM and emission requirement according to the simulation. The requirement is a relative requirement according to the Tx power. Usually, the emission requirement is an absolute requirement. If absolute requirement is needed, the requirement can be translated according to maximum output power.

Figure 4：SEM candidate for 7.5kHz Tb D2R signal without SFS

Proposal 3: SEM requirement candidate for no SFS case is proposed in Figure 4.

If the requirement of absolute value is needed, NR UE general spurious emissions limits can be reused from Fc+24/Tb.

Proposal 4: NR UE general spurious emissions limits can be reused from Fc+24/Tb for no SFS case if absolute requirement needs to be defined.

For the boundary, if it can be defined as CBW related absolute value. Figure 1 to Figure 4 are the simulations for the data with bitrate = 1/7.5kHz, i.e. chiprate is 1/15kHz, the emission from 1RB boundary can be – 30dBc lower than the main lobe. But if the bitrate is increased, the emission at 1RB boundary increases because the sidelobe attenuation is relative to 1/Tb, not the absolute value. Taking bitrate = 1/30kHz, then the harmonic at 180kHz is only ~-20 dBc compared with the main lobe.

Figure 5: Spectrum for D2R signal with bitrate=1/30kHz

Observation 2: It’s difficult to have absolute boundary for emission requirements for different bitrate.

2.4 Reference sensitivity

For reference sensitivity, we have the following analysis and proposals for the open issues.

For the EIS direction, considering the low cost, we support only peaks direction is tested which aligns the transmit power methodology. However, we understand the concern of spherical coverage, so if the test cost is not increased that much, several directions can be measured. Similar with NR BS EIS test, several

Proposal 5: The following two options can be considered for the REFSNES requirement.

- EIS in the peak direction.

- Several directions based on the declaration if the extra test burden and cost are endurable.

For the testing metric, the last meeting’s status is as following,

- 10% miss detection rate, FFS on whether to consider false alarm

We checked RAN1 discussion, MDR and FAR are used to evaluate the design of R2D SIP (start indicator part) signal as following which is copied from TR 38.769.

In our understanding, MDR (missed detection rate) or FAR (false alarm rate) is used to characterize the performance of detection a specific signal or event. For AIoT, PRDCH is an important physical channel and the SNR threshold is higher than SIP. So the detection performance for SIP is not a proper criteria to decide device REFSENS. 50% success rate of identifying the tag is used in ETSI EN 302 208 as the test metric for integrator’s Rx requirements. We think success rate is more proper than MDR or FAR for device REFSENS performance. If the MDR definition is MDR = 1 – success rate, it may be ok. But we still think there’s possibility for some misleading. We prefer to use success rate 90% as the criteria. For the FAR, we think it’s not very essential and there’re not so many FAR evaluations for the whole inventory. In RAN1 evaluation for the SIP signal, some companies report FAR of ~0% and MDR of less than 10%. It is difficult to decide to FAR value without more study or simulation.

Based on all of the above consideration, we have the following proposal,

Proposal 6: 90% success rate of identifying the tag is used to test tag REFSENS for PRDCH channel. No FAR is needed.

- Conclusion

This contribution provides our analysis for the device 1 RF requirements. We have the following proposals and observations.

For transmit output power,

Proposal 1: Only peak direction requirement for transmit output power is defined.

For SFO,

Proposal 2: No explicit SFO requirement is needed.

For SEM and emission,

Observation 1: If the emission boundary is set relative to the device for R=1 case, it may not apply to other R numbers.

Observation 2: It’s difficult to have absolute boundary for emission requirements for different bitrate.

Proposal 3: SEM requirement candidate for no SFS case is proposed in Figure 4.

Proposal 4: NR UE general spurious emissions limits can be reused from Fc+24/Tb for no SFS case if absolute requirement needs to be defined.

For REFSENS,

Proposal 5: The following two options can be considered for the REFSNES requirement.

- EIS in the peak direction.

- Several directions based on the declaration if the extra test burden and cost are endurable.

Proposal 6: 90% success rate of identifying the tag is used to test tag REFSENS for PRDCH channel. No FAR is needed.

- References

[1] R4-2508116, “WF on A-IoT_device”, CMCC, RAN4#115

[2] ETSI EN 302 208