3GPP TSG-RAN4 Meeting #106	R4-2509818

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	7.7.5

Source: 	Xiaomi

Title: 						View on supporting new VSAT type for Mobile VSAT communicating with LEO only

Document for:	Discussion

- Introduction

In Rel-19 Ku band WI, one of objective is to support the scenario Mobile VSAT communicating with LEO by extending existing VSAT type 4 and Type 5 requirements whenever applicable.

The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 (The US is in scope for the WI using the current allocation of the Ku band. The band introduction in specification is pending FCC decision): - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate Tx/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for coexistence scenario, frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services. - Specify RF requirements for mobile VSAT served by NGSO and GSO based on existing mobile VSAT type 5 RF requirements - Investigate and if possible specify common FR1 and FR2 radiated RF requirements for Satellite Access Node (SAN) and NTN VSAT types (excluding types 1 to 3) in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B. Define two sets of bands, one for FR1 numerology and the other for FR2 numerology, including the support of the following: - Ku band #1a and #1b as FR1-NTN supporting 10 MHz, 20 MHz, 25 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing, and 10MHz, 15MHz, 25MHz, 35MHz and 50MHz channel bandwidths with 15 kHz subcarrier spacing. - Note: Same subcarrier spacing is assumed for SSB and data channels - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing. - Note: Simultaneous operation of FR1-NTN and FR2-NTN numerology is not supported in the UE - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A. - Specify RRM requirements including timing requirements for mobile VSAT served by NGSO - NOTE: RRM requirements are defined for VSAT supporting FR1-NTN operation only, or for FR2-NTN operation only.

During previous RAN4 meetings, satellite industry clarified the target sceanrio is Mobile VSAT supporting with LEO only. Regarding hwo to treat this scenario,  RAN4 still didn’t get  formal decision.

- Issue 1-2-1: Clarification on the minimum peak EIRP requirements Agreement: - Currently the VSAT type 4 & type 5 covers both GSO and LEO, but companies show strong need on allowing VSAT support LEO only in Rel.19. - Consider a new type for LEO only based on the same VSAT antenna size assumption as type 4 & 5 as a working assumption - Issue 1-2-3: Minimum peak EIRP requirements for LEO Agreement: - Confirm minimum peak EIRP at 60 dBm as min peak EIRP for both electronically steering VSAT and mechanical steering VSAT connecting to LEO.

In this contribuion, views are provided for how to treat different VSAT types in this release and long term vision for the future.

- Discussion

Capability signalling perspective:

In existing RAN4 specifications, different VSAT types were introduced covering following scenarios:

Table 9.2.1.0-1: The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 52 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| NOTE 1: The NTN VSAT types are assuming NTN VSAT has only one antenna beam towards one satellite at a given time in this release. NOTE 2: NTN VSAT may need power reduction to comply with OFF-axis EIRP requirement defined in clause 9.2.2. There is no requirement for the potential power reduction. |  |  |

From capability perspective, 3 existing capability IEs can cover different VSAT types in terms of VSAT mobility type, VSAT antenna type and NTN supporting scenarios (“ntn-VSAT-MobilityType-r18”, “ntn-VSAT-AntennaType-r18” and “ntn-ScenarioSupport-r17”).

By combinations of above 3 capabilities, we can cover different target scenarios/VSAT types as summarized in below table:

| NTN VSAT Mobility | NTN VSAT Antenna Type (Beam steering) | NTN scenario | NTN VSAT Type |
| --- | --- | --- | --- |
| Fixed | Mechanical | GSO only | NA |
| Fixed | Mechanical | NGSO only | NA |
| Fixed | Mechanical | GSO and NGSO | 1 |
| Fixed | Electronic | GSO only | NA |
| Fixed | Electronic | NGSO only | 3 |
| Fixed | Electronic | GSO and NGSO | 2 |
| Mobile | Mechanical | GSO only | 4 |
| Mobile | Mechanical | NGSO only | NA |
| Mobile | Mechanical | GSO and NGSO | NA |
| Mobile | Electronic | GSO only | 5 |
| Mobile | Electronic | NGSO only | Target new scenario for Ku band |
| Mobile | Electronic | GSO and NGSO | NA |

Observation 1: From UE capability perspective, existing capability IEs (“ntn-VSAT-MobilityType-r18”, “ntn-VSAT-AntennaType-r18” and “ntn-ScenarioSupport-r17”) already cover different VSAT types. There is no additional capability signalling required to support Mobile VSAT with electronic antenna with NGSO/LEO only.

Requirements impact aspect:

Another key issue what’s the difference from RAN4 requirements perspective (RF/RRM) for supporting different scenarios as mentioned above.

- Compared to VSAT type 5 (Mobile VSAT communicating with GSO with electronic steering antenna), the difference for RRM is uplink timing requirement (which already resolved in RRM session).

- Compared to VSAT type 5 (Mobile VSAT communicating with GSO with electronic steering antenna), the difference for RF is Minimum peak EIRP requirements and EIS requirements.  (The expected work is minor)

Observation 2: From RAN4 requirements perspective, only EIRP and EIS requirements have difference compared to VSAT type 5 (Mobile VSAT communicating with GSO with electronic steering antenna) to support Mobile VSAT with LEO only with electronic steering antenna.

We also summarize, the applicable requirements for different device types in case more device types introduced in the future.

| NTN VSAT Type | TS 38.101-5 RF requirements |
| --- | --- |
| New-a Fixed VSAT communicating with GSO only with mechanical steering antenna | Reusing VSAT Type 1 requirements |
| New-b Fixed VSAT communicating with LEO only with mechanical steering antenna | Reusing VSAT Type 3 requirements |
| New-C Fixed VSAT communicating with GSO only with electronic steering antenna | Reusing VSAT Type 1 Requirements |
| New-d Mobile VSAT communicating with GSO and LEO with mechanical steering antenna. | Reusing VSAT Type 4 Requirements |
| New-e Mobile VSAT communicating with LEO only with mechanical steering antenna. | Reusing VSAT Type 4 Requirements except Minimum peak EIRP and EIS |
| New-f Mobile VSAT communicating with LEO only with electronic steering antenna. | Reusing VSAT Type 4 Requirements except Minimum peak EIRP and EIS |
| New-g Mobile VSAT communicating with GSO and LEO with electronic steering antenna. | Reusing VSAT Type 5 Requirements |

Specification drafting

The open question is how to support this demand in RAN4 specifications, two alternative ways

- Alternative 1: introduce new dedicated VSAT type e.g. VSAT 6

The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
|  | 6 | Mobile VSAT communicating with LEO only with electronic steering antenna. |
|  | 7 | Mobile VSAT communicating with GSO and LEO with electronic steering antenna. |

Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n509, n508 | 4 | 70 |
|  | 5, 7 | 70 |
| n509, n508 | 6 | 60 |
| NOTE: Minimum peak EIRP is defined as the lower limit without tolerance. |  |  |

Table 9.2.1.2-2: Maximum output power limits for Mobile VSAT

| Operating band | UE Type | TRPMAX (dBm) | EIRPmax (dBm) |
| --- | --- | --- | --- |
| n512, n511, n510, n509, n508 | 4 | 35 | 76.2 |
|  | 5, 6, 7 | 43 | 76.2 |
| NOTE: Maximum EIRP is defined using 13RBs allocation with 120kHz SCS. |  |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510, n508, n508 | Mobile VSAT | 4, 5, 7 | ≤ -122 |
|  | Mobile VSAT | 6 | ≤ -115.6 |

- Alternative 2: Update existing VSAT type to including more scenarios with requirements applicable rule clarification

The definitions of NTN VSAT Types

| NTN VSAT class | NTN VSAT type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 22 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT with mechanical steering antenna. |
|  | 5 | Mobile VSAT with electronic steering antenna. |

Table 9.2.1.2-1: Minimum peak EIRP for Mobile VSAT

| Operating band | UE Type | Min peak EIRP (dBm) |
| --- | --- | --- |
| n512, n511, n509, n508 | 4 | 70NOTE2 |
|  | 5 | 70NOTE2 |
| n509, n508 | 4 | 60NOTE3 |
| NOTE1: Minimum peak EIRP is defined as the lower limit without tolerance. NOTE2: Applicable for VSAT which support communication with GSO NOTE3: Applicable for VSAT which support communication with LEO only |  |  |

Table 10.3.2-2: The range of EISREFSENS_50MHz declared by vendor per NTN VSAT

| Operating band | NTN VSAT class | NTN VSAT type | The range of EISREFSENS_50MHz (dBm) |
| --- | --- | --- | --- |
| n512, n511 | Fixed VSAT | 1, 2 | ≤ -122 |
|  |  | 3 | ≤ -115.6 |
| n512, n511, n510, n509, n508 | Mobile VSAT | 4, 5 | ≤ -122NOTE1 |
|  |  | 4, 5 | ≤ -115.6NOTE2 |
| NOTE1: Applicable for VSAT which support communication with GSO NOTE2: Applicable for VSAT which support communication with LEO only |  |  |  |

Observation 3: Two alternatives can be considered for supporting mobile VSAT with LEO only with electronic steering antenna

- Alternative 1: introduce new dedicated VSAT type e.g. VSAT type 6

- Alternative 2: Update existing VSAT type to including more scenarios with requirements applicable rule clarification e.g., remove limitation on NTN scenarios in VSAT type definition with requirements applicable rules (EIRP and EIS)

Proposal 1: Supporting the scenario Mobile VSAT with LEO only with electronic steering antenna in Rel-19 Ku bands.

Proposal 2: Introduce VSAT type 6 which target for mobile VSAT with electronical steering antenna supporting LEO only in Rel-19 for Ku bands.

Proposal 3: Further discuss how to reorganize VSAT type in RAN4 NTN UE RF specifications to be compatible with different scenarios in future proof manner.

- Conclusion

In this contribuion, views are provided for how to treat different VSAT types in this release and long term vision for the future.

Observation 1: From UE capability perspective, existing capability IEs (“ntn-VSAT-MobilityType-r18”, “ntn-VSAT-AntennaType-r18” and “ntn-ScenarioSupport-r17”) already cover different VSAT types. There is no additional capability signalling required to support Mobile VSAT with electronic antenna with NGSO/LEO only.

Observation 2: From RAN4 requirements perspective, only EIRP and EIS requirements have difference compared to VSAT type 5 (Mobile VSAT communicating with GSO with electronic steering antenna) to support Mobile VSAT with LEO only with electronic steering antenna.

Observation 3: Two alternatives can be considered for supporting mobile VSAT with LEO only with electronic steering antenna

- Alternative 1: introduce new dedicated VSAT type e.g. VSAT type 6

- Alternative 2: Update existing VSAT type to including more scenarios with requirements applicable rule clarification e.g., remove limitation on NTN scenarios in VSAT type definition with requirements applicable rules (EIRP and EIS)

Proposal 1: Supporting the scenario Mobile VSAT with LEO only with electronic steering antenna in Rel-19 Ku bands.

Proposal 2: Introduce VSAT type 6 which target for mobile VSAT with electronical steering antenna supporting LEO only in Rel-19 for Ku bands.

Proposal 3: Further discuss how to reorganize VSAT type in RAN4 NTN UE RF specifications to be compatible with different scenarios in future proof manner.

Reference

[1] R4-2508778	Way Forward for [115][314] NR_NTN_Ku_Band_UE_SAN_RF, Moderator (CHTTL)

[2] 3GPP TS 38.101-5 v19.1.1