3GPP TSG-RAN4 Meeting #106	R4-2509820

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	7.1.2.3.2

Source: 	Xiaomi

Title: 						UE feature for 6Rx UE

Document for:	Discussion

- Introduction

In Rel-19 UE RF enahcnement WI, one of objective is to introduce 6Rx UE with uper to 6 layer device type and non-handheld device type.

According to previous RAN4 agreements, RAN4 categorize device type capability based on the maximum number supported layers for 6Rx UE:

R4-2503014: Issue 2-1-1: Way forward for 6-layers support for handheld UE and FWA devices Way Forward: - RAN4 Chair to take note of the following in the meeting report: - “RAN4 unable to conclude on a common set of simulation assumptions to determine feasibility of 6 layer for handheld UE In Rel-19” - 6-layer performance requirements will be only defined for FWA. - 4-layer performance requirements will be defined and will apply to FWA and handheld UE devices. - The fact that 6-layer is only supported for FWA shall be explicitly captured at least in the RAN4 specifications.

In the UE feature list, RAN4 also agreed to introduce UE feature with optional capability for 6Rx UE with 6-layer capability:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 46-6 | Support maximum 6 MIMO layers for DL reception | Indicates whether the UE supports maximum 6 spatial multiplexing layers for DL reception for FWA. |  | Yes | N/A | FWA does not support maximum 6 MIMO layers for DL reception | Per FSPC | N/A | Yes (only FR1) | N/A |  | Optional with capability signalling |

The remaining issue is how to treat 6Rx UE with maximum 4-layer capability. In this discussion, we discuss UE feature for 6Rx UE.

- Discussion

In previous release, we didn’t discriminate number of maximum DL layers and maximum number of Rx as it’s equivalent by assuming UE will report maximum layer capability (maxNumberMIMO-LayersPDSCH) same as number of Rx.  For example, for 4Rx mandatory bands, UE mandate to report maximum number of DL layers as 4. And for 8Rx UE, UE needs to report supporting maximum DL layers as 8.  And from NW scheduling perspective, it makes more sense UE reporting maximum number of layers instead of number of Rx as NW can schedule different CWs and layers based on UE capability on maximum number of layers.

From RAN4 demodulation requirements perspective, different requirements and test cases were specified based on number of Rx i.e., 1Rx, 2Rx, 4Rx and 8Rx. And then test appliable rule was introduced based on UE capability on number of Rx/layers (maxNumberMIMO-LayersPDSCH) as descripted in TS 38.101-4 section 5.1.

As mentioned above, this is first release we have introduced separate UE capacities which maximum number of layers is not equivalent to number of Rx. And separate requirements /test cases are introduced for these two kinds of UE capabilities.

Observation 1: From RAN4 performance requirements perspective, separate requirements and test cases will be introduced for 6Rx UE supporting up to 4 layer and 6Rx UE supporting up to 6 layers.  And requirements applicable rule needs to be introduced to differentiate different UE capability (6Rx UE with maximum 4-layer capability, 6Rx UE with maximum 6-layer capability)

R4-2508727 Issue 1-1: Test applicability rule for 6Rx tests - Proposals - Option 1: Introduce UE declaration and design proper test applicability rule in the specification as per the objective of perf part in RP-250780 if no explicit UE capability signalling to differentiate UE type introduced by RAN2. - Option 2: Introduce test applicability rule only for 6 layers cases - Option 3: Postpone the discussion after RAN2 finishes the UE capability signalling design

Similar as previous discussion in RAN4, some UE capabilities are required to differentiate UE with separate performance requirements/test cases, on the other side no capability signalling is required as there is no need for network aware of such capabilities.

There are several prerequisite examples existing in the specification e.g. receiver type MMSE-IRC receiver and R-ML receiver; 8Rx receiver (simplified 8Rx SU-MIMO receiver and normal 8Rx SU-MIMO receiver).

Proposal 1: Introduce new UE feature for 6Rx UE with maximum 4 layer

- This feature is optional without UE capability signalling

# R4-2508086 # <Topic #2> MIMO layer evaluation for 6Rx UE ## <Sub-topic 2-1> Release independency for 6Rx UE Agreement: - Option 1: - Support 6Rx with up to 4 MIMO layers for both handheld UE and FWA from Rel-18 - Support 6Rx with up to 6 MIMO layers for FWA from Rel-19 - Option 2: - Support 6Rx from Rel-19 - Option 3: - Support 6Rx from Rel-18

We support option 1 to take 6Rx with up to 4 MIMO layers from Rel-18 for release independent.

- Conclusion

Observation 1: From RAN4 demodulation/CSI requirements perspective, separate requirements and test cases will be introduced for 6Rx UE supporting up to 4 layer and 6Rx UE supporting 6 layers.  And requirements applicable rule needs to be introduced to differentiate different UE capability (6Rx UE with maximum 4 layer capability, 6Rx UE with maximum 6 layer capability)

Proposal 1: Introduce new UE feature for 6Rx UE with maximum 4 layer

- This feature is optional without UE capability signalling

| 46-7 | Support maximum 4 layers for DL reception with 6Rx | Indicates whether the UE supports maximum 4 spatial multiplexing layers for DL reception with 6Rx |  | n/a | n/a | UE does not support maximum 4 MIMO layers for DL reception with 6Rx | N/A | no | FR1 only | N/A |  | Optional without UE capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 2: Support option 1 to take 6Rx with up to 4 MIMO layers from Rel-18 for release independent.

Reference

[1] R4-2503014, “WF on requirements for 6Rx,”

[2] R4-2508727, Way Forward for Topic for [115][326] NR_ENDC_RF_Ph4_Demod_6Rx, Huawei

[3] R4-2508086	WF on 6Rx and SRS IL, Huawei