3GPP TSG-RAN4 Meeting #106	R4-2509821

# Bengaluru, India, 25th August 2025 – 29th August 2025

Agenda Item:	7.3.3

Source: 	Xiaomi

Title: 						View on LB CA with switching

Document for:	Discussion

- Introduction

In previous RAN-P, a new WI was agreed to enable LB-LB CA with swicthing in [1]. The motivication of this WI was to efficiently use the SDL spectrum together with low FDD band via CA by swicthing to eliminate UE implemenation channllenge  of supporting co-current transmission/reception between these two carriers due to antenna performance with wide frequnecy separation (fractional bandwith), and or duplexer design w/o sufficient frequency gap between DL and UL.

Detailed WID is reproduced here for reference:

Introduce physical layer procedures and requirements to enable low band carrier aggregation via switching according to the following objectives: - Specify UE requirements, including at least switching gap (if needed), and corresponding physical layer procedures to allow switching between {case 1, case 2} [RAN4, RAN1] - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - RAN1 to specify only a semi-static switching pattern based on RRC configuration, liaising with RAN2 and RAN4 as necessary - Specify the switching delay and time mask for carrier switching [RAN4] - Specify necessary RRM requirements [RAN4] - Define the corresponding UE capabilities [RAN4, RAN2, RAN1] - Consider the following deployment constraints: - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers Note 1: Specify requirements for the feature with the following example band combination: CA_n5A-n29A in this WI, with additional band combinations to be handled via the basket work item approach Note 2: Strive to minimize the RAN1 impact

In this contribution, we analyze the remaining open issues to UE RF requirements for LB-LB CA via switching.

- Discussion

Capability signalling perspective:

Issue 2: Agreements related to UE capabilities - A new feature group is included in the Rel-19 feature list for NR_LBCA_Sw - Related to concurrent Rx operation, the following aspect is included in the feature group: - [The con-current reception on FDD band and SDL band is not supported. The con-current operation between FDD UL and SDL is not supported.] - Only switchedDL option can be considered for low-low CA downlink carrier switching in this WID. - Support 70us as a candidate value of switching gap for 15kHz SCS in addition to 35us and 140us

Regarding concurrent Rx operation, we suggest to follow the description in the WID, carrier switching between following 2 cases:

-	Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2

-	Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1

The feature set can be updated as following:

Proposal 1: For co-current operation description in UE feature set, propose to follow the WID description:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 55. NR_LBCA_SW | 55-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between an FDD band (case1) and SDL band (case2): - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 switchingPeriodForFDD-SDL indicates the length of the switching time between{case 1, case 2}: 35us represents 35 us of switching time, 70us represents 70us of switching time,140us represents 140us of switching time, as specified in TS 38.101-1. |

Uplink Tx on/off time mask

Issue 1: Agreements related to the time mask requirement - A text proposal to TR38.768 in R4-2507932 captures the relevant agreements - How to specify the DL time mask: - The UE DL behaviour following the switch operation can be discussed in the RRM session - Considering that only one RAN4 meeting is left before the conclusion of the work item, companies are encouraged to exchange views via the RAN4 email reflector in advance of RAN4 #116 to converge on the technical content of the CR to 38.101-1 - RAN4 to further discuss whether the time mask should show the switching period/gap location and whether the diagram similar to Figure 5.2.2-1 and 5.2.3-1 in TR38.768 should be considered.

For carrier switching between {case 1, case 2} as mentioned above, both DL and UL will be involved during the switching gap as specified by RAN1 procedures. The whole procedure was descripted in TP to TR (RP-R4-2507932):

| Figure 5.2.2-1: FDD to SDL switch with the switching gap on the switched-from carrier. | Figure 5.2.3-1: SDL to FDD switch with the switch occurring before the start of the UL slot. |
| --- | --- |

RAN4 agreed to decouple DL part and UL part for FDD-SDL CA via switching. From RAN4 UE RF requirements perspective, RAN4 will specify uplink Tx on/off mask requirements on FDD carrier within the switching gap configured by network. The UE DL behaviour following the switch operation can be discussed in the RRM session.

Regarding UL time mask requirements, following latest agreements from RAN1 that switching gap always located switching from carrier which means:

Observation 1: On switching period/switching gap location following agreements reached by RAN1

- For SDL carrier switching to FDD carrier, the switching period/switching gap located out of FDD carrier slot.

  - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

- For FDD carrier switching to SDL carrier, the switching period/switching gap located within FDD carrier slot

With above RAN1 agreements, we suggest to introduce uplink Tx on/off time mask as following:

Proposal 2: Introduce Uplink Tx on/off time mask on FDD carrier for LB CA switching as following:

ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL PCell/carrier 1

RMC issue:

Regarding RMC for uplink Tx switching requirements, and for DL requirements on LB sw band combinations, new RMC is needed to be aligned with RAN1 design on switching gap and switching pattern.

Observation 2: New RMC are required for LB switching.

Following RAN1 agreements, 40ms periodicity with bitmap can be configured as per UE based RRC configuration.  Restriction of maximum X switch(es) within Y slot(s), and FFS: detailed value and relation of X and Y.

Observation 3: Detailed switching pattern still subject to RAN1 agreements and operators’ input.

We proposed a tentative switching pattern for discussion as following:

Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

- 0 as FDD carrier, 1 as SDL carrier

- 1 switch within 10 slots

Above design can help to simplify the RMC with per sub-frame meanwhile allow SSB transmission per 20ms.

- Conclusion

Proposal 1: For co-current operation description in UE feature set, propose to follow the WID description:

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 55. NR_LBCA_SW | 55-1 | Switching period for switching between a LB FDD band and an SDL band | UE to indicate support of LB-LB carrier aggregation via switching between an FDD band (case1) and SDL band (case2): - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 switchingPeriodForFDD-SDL indicates the length of the switching time between{case 1, case 2}: 35us represents 35 us of switching time, 70us represents 70us of switching time, 140us represents 140us of switching time, as specified in TS 38.101-1. |

Observation 1: On switching period/switching gap location following agreements reached by RAN1

- For SDL carrier switching to FDD carrier, the switching period/switching gap located out of FDD carrier slot.

  - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA

- For FDD carrier switching to SDL carrier, the switching period/switching gap located within FDD carrier slot

Proposal 2: Introduce Uplink Tx on/off time mask on FDD carrier for LB CA switching as following:

ON/OFF time mask for NR UL transmission for DL CA via switching with non-CA in the UL

Observation 2: New RMC are required for LB switching.

Observation 3: Detailed switching pattern still subject to RAN1 agreements and operators’ input.

Proposal 3: A tentative switching pattern for RRM as proposed

Bitmap (0000000000, 1111111111, 0000000000, 1111111111)

- 0 as FDD carrier, 1 as SDL carrier

- 1 switch within 10 slots

- UL TA assume as 0 us for RMC definition and test

Reference

[1] RP-243317, New WID on low band carrier aggregation via switching, AT&T TELUS, Bell Mobility, BT plc, Skyworks Solutions Inc., Anterix, Southern Linc, AT&T, Boost Mobile Network, Murata, Ericsson, Samsung, Qorvo, Nokia, Huawei, HiSilicon, vivo, OPPO, Qualcomm, Google, Sony, ZTE, Intel Corporation, MediaTek, LGE

[2] R4-2503016 WF o low band switching, Apple

[3] R4-2505223 WF on UE RF requirements for LB CA via switching, Apple

[4] R4-2504895, WF on RRM requirements for NR_LBCA_Sw, Apple

[5] R4-2508118,WF on NR_LBCA_Sw, Apple

Annex:

Agreements made in RAN1#120bis

Agreement - For Rel-19 low NR band carrier aggregation via switching, support only semi-static configuration of switching pattern to a UE based on UE-specific RRC configuration between the following two cases - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2 - Case 2: Rx on SDL carrier 2 and no Tx/Rx on FDD carrier 1 - FDD carrier 1 is PCell and SDL carrier 2 is SCell - SCS 15KHz on both carriers Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, it is not expected that the same slot (1ms) contains both symbol(s) configured as FDD carrier and symbol(s) configured as SDL carrier. Agreement For Rel-19 low NR band carrier aggregation via switching, the semi-static switching pattern and corresponding switching gap are based on the downlink timing (DL reception time at UE side) of Pcell. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the time unit (resolution) of the semi-static switching pattern configuration is slot (1ms for 15kHz SCS). Agreement - For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, support bitmap design - NW configures a periodicity, denoted as P slots, and a bitmap of P bits. Each bit in the bitmap indicates which carrier is configured for the corresponding slot, ‘0’ indicates FDD carrier 1, and ‘1’ indicates SDL carrier 2 - The pattern starts from the beginning of SFN 0 of PCell - Restriction of maximum X switch(es) within Y slot(s), - FFS: detailed value and relation of X and Y. - FFS: other restriction Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap, - For switching from SDL carrier to FDD carrier, the switching gap is always assumed at the “switch from” carrier, i.e., SDL carrier. - For switching from FDD carrier to SDL carrier, downselect one of the following solutions - Alt 1: Switching gap is always assumed at the “switch from” carrier, i.e., FDD carrier - Alt 2: NW configures whether the switching gap is at the FDD carrier or SDL carrier - Alt 3: Switching gap is always assumed at the SDL carrier Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, semi-static switching pattern is periodic with periodicity of P slots (ms), support P= 40 ms Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, the duration of the switching gap is RRC configured by the NW, - Switching gap duration can be different for FDD to SDL switch, and SDL to FDD switch. - Note: NW ensures that the switching gap is enough to cover at least the switching period as in RAN4 LS (R1-2501702) and UL TA (if needed) - For the UL TA, - Switching gap for SDL to FDD switch ends at the end of slot on SDL - Note: RAN1 assumes that switching gap for SDL to FDD switch includes UL TA - FFS: whether it has specification impact

Agreements made in RAN1#121

Agreement For Rel-19 low NR band carrier aggregation via switching, support configuration of only one switching pattern to a UE based on UE-specific RRC configuration. Agreement For RRC configuration of semi-static switching pattern in Rel-19 low NR band carrier aggregation via switching, regarding the location of the switching gap for switching from FDD carrier to SDL carrier, only support the switching gap is located at the “switch from” carrier, i.e., FDD carrier. Agreement - For Rel-19 low NR band carrier aggregation via switching, in terms of the collision handling - The following cases for collision handling are specified where UL TA is assumed to be 0 for the determination of overlapping. The UL TA =0 assumption only applies to the UL transmission configured by higher layer - Collision 1: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 2: If a UE is configured by higher layers to transmit a PUSCH, PUCCH, PRACH, or SRS in a set of symbols of an UL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not transmit the PUSCH, PUCCH, PRACH, or SRS - Collision 3: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 4: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the SDL carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as FDD carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - Collision 5: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the switching gap, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS. - Collision 6: If a UE is configured by higher layers to receive a PDCCH, PDSCH, PRS or CSI-RS in a set of symbols of a DL slot on the FDD carrier that overlaps, even partially, with the set of symbols of the DL slot indicated as SDL carrier, the UE does not receive the PDCCH, PDSCH, PRS or CSI-RS - For the other cases, i.e., DL and UL signals/channels scheduled/triggered by DCI - A UE is not expected any scheduling/triggering from network that will lead to above collisions - Except that the PUSCH, PUCCH, PRACH, or SRS transmission that overlapped with the switching gap on SDL carrier when the SDL carrier switches to FDD carrier is not considered as collision - UE expects that the switching gap configured by the NW for SDL carrier to FDD carrier switching covers at least the switching period as in RAN4 LS (R1-2501702) and UL TA