3GPP TSG-RAN WG4 Meeting #116		R4-2509832

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.6.5.2

Source:		Samsung

Title:	Discussion on RRM impacts by using NGSO in the test

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#106 meeting. The objectives of performance part of NTN testing for NGSO are described in [1] as:

The objectives of performance part for enhanced requirements and test methodology for NR and IoT NTN include: NTN testing for NGSO - Specify the TE-emulated channel model with varying Doppler and delay shifts for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands - Match the satellite motion trajectory based on the ephemeris for NGSO (Non-Geostationary Orbit) scenarios - Inform RAN5 to assist specifying RF frequency error tests, if needed - Checking if the existing frequency error requirement, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement - Specify UE demodulation performance requirement(s) of PDSCH for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Minimize the number of demodulation performance requirements - Specify the applicability of the tests under the TE-emulated channel model

In last RAN4#115 meeting, it has already been agreed in [1] there are no core impacts by NTN testing for NGSO. The only remained open issue is how to apply NTN testing for NGSO in performance part. In this contribution, we provide our view of the RRM performance part of NTN testing for NGSO.

- Discussion

In the existing test cases, for NR in 38.133 Annex, there are two series of TCs for NR-NTN:

#### A.14.3.1.1 NR UE Transmit Timing Test for FR1 ### A.14.3.2 Timing advance for satellite access #### A.14.3.2.1 SA FR1 timing advance adjustment accuracy

For the first test case, it is to test the UE transmit timing.

For the second test case, it is to test the timing advance adjustment.

In both tests, AWGN channel was used in the tests. For the first step, the NGSO channel model should be replaced from Rel-19.

| Propagation condition |  | 1,2 | AWGN |
| --- | --- | --- | --- |

| Propagation condition | - | AWGN |
| --- | --- | --- |

Proposal 1: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

For the Transmit timing test, the test equipment verified the timing based on UE estimated value.

Proposal 2: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements. The NTA,adjUE in the test requirements is calculated by UE itself.

- For the timing advance test, FFS on whether addition margin should be added in the test requirements.

For Iot-NTN tests in 36.133, five TCs are specified:

## A.13.4 Timing and signalling characteristics for satellite access ### A.13.4.1 UE transmit timing for satellite access A.13.4.1.1 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under normal coverage for Satellite Access #### A.13.4.1.2 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage for Satellite Access #### A.13.4.1.3 E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE Standalone mode under enhanced coverage with segment transmission in NGSO for Satellite Access ### A.13.4.2 UE timing advance for satellite access A.13.4.2.1 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Normal Coverage for Satellite Access A.13.4.2.2 HD-FDD UE Timing Advance Adjustment Accuracy Test for UE Category NB1 in Standalone Mode under Enhance Coverage for Satellite Access

Proposal 3: For five TCs for IoT-NTN, use the same principle as NR-NTN.

In last meeting, some companies proposed to add NGSO channel in all Test cases in NTN testing.

However, in the WID, it explicitly specified the scope is for uplink timing RRM performance tests.

- Specify UE RRM performance test of uplink timing for NGSO for NR-NTN and IoT-NTN in the FR1-NTN bands and the corresponding LTE bands based on the above channel model - Checking if the existing RRM uplink timing requirements, can be met under the TE-emulated channel model without intention of modifying the existing core requirement

Proposal 4: Do not extend NGSO channel for all NTN test cases to align with scope in WID.

- Conclusion

In this contribution, we analyze the potential impact on RRM performance requirements for NTN testing for NGSO and provide our proposals:

Proposal 1: First, NGSO channel model should be added in these TCs in the table. The corresponding configuration of NGSO channel should be added.

- During the tests, UE position is needed to be added and set by AT command at the beginning of T1. In successive time periods of different T, UE position should keep unchanged. The TE adds the NGSO channel model by itself.

Proposal 2: For NR-NTN

- For transmit timing test, no impacts for the test procedure and test requirements. The NTA,adjUE in the test requirements is calculated by UE itself.

- For the timing advance test, FFS on whether addition margin should be added in the test requirements.

Proposal 3: For five TCs for IoT-NTN, use the same principle as NR-NTN.

Proposal 4: Do not extend NGSO channel for all NTN test cases to align with scope in WID.

- Reference

[1] RP-242901, Revised WID on Enhanced requirements and conductive test methodology for NR NTN and IoT NTN

[2] R4-2508261, WF on RRM requirements for NR_IoT_NTN_req_test_enh, Xiaomi