3GPP TSG-RAN WG4 Meeting #116		R4-2509838

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.21.5

Source:		Samsung

Title:	Discussion on RRM performance requirements of Rel-19 SBFD

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#108 meeting. The objectives of performance part of this WI are described in [1] as:

The objectives are as follows [RAN4]: - Specify BS RF conformance requirements to support SBFD operation at gNB - Specify necessary RRM performance requirements to support SBFD operation - Specify necessary BS and/or UE demodulation performance requirements to support SBFD operation

Until last RAN4#115 meeting, RAN4 discuss the RRM core requirements and achieved lots of agreements, the endorsed draft Big CR was in [2].

In this contribution, we provide our view of the RRM performance part of this WI.

- Discussion

In previous meetings, RAN4 discuss the RRM core requirements and achieved lots of agreements of Rel-19 duplex.

In last RAN4#115 meeting, the open issues of RRM performance part are captured in [3]:

### Sub-topic 3-2: Accuracy requirements #### Issue 3-2-1: L1-SRS-RSRP – side condition on time offset #### Issue 3-2-2: L1-SRS-RSRP – side condition on SRS BW #### Issue 3-2-3: L1-SRS-RSRP – side condition on SRS Es/Iot #### Issue 3-2-4: L1-SRS-RSRP – accuracy requirements #### Issue 3-2-5: L1-CLI-RSSI – accuracy requirements #### Issue 3-2-6: L1-RSRP and L1-SINR – accuracy requirements ### Sub-topic 3-3: Report mapping #### Issue 3-3-1: Absolute report mapping #### Issue 3-3-2: Differential report mapping ### Sub-topic 3-4: Test cases #### Issue 3-4-1: General test setup #### Issue 3-4-2: Test case list #### Issue 3-4-3: Test case details and test purpose #### Issue 3-4-4: Test case scenarios

The performance requirements can be divided into two parts: Measurement accuracy and Test cases:

Measurement accuracy

In previous meetings, the measurement accuracy of different measurements has been discussed already.

L1-SRS-RSRP measurement accuracy

In RAN4#114 meeting, the side condition and accuracy requirements are agreed:

#### Issue 1-2-1: Side conditions - Agreements (from online session) - For SRS configuration, reuse R16 assumptions on SRS configuration for L1 SRS-RSRP measurements - Agreements - Further discuss timing offset side condition for accuracy requirements in the performance part of the WI, including whether to consider intra-cell scenario only or both intra- and inter-cell scenarios for defining the side condition. - For SRS Es/Iot, further discuss the following options - Option 1: 1dB - Option 2: > 1dB - For SRS BW, further discuss the following options - Option 1: 24RB - Option 2: 48RB

In previous meeting, we provided the analysis of the L1-SRS-RSRP are sensitive to the residual timing error especially for inter-cell scenario. The number of samples will be related to the different cases.

Case 1: 4.67us for FR1, 3.67us for FR2 (inter-cell case) Case 2: 1.67us for FR1, 0.67us for FR2 (intra-cell case)

An LS was sent to RAN1 to check the scope of L1 CLI measurement.

RAN1 sent a reply LS for the L1-CLI measurement:

When discussing L1 UE-to-UE CLI measurement reporting, RAN1 has not made a distinction between the two application scenarios mentioned in the RAN4 LS (intra-cell vs. inter-cell UE-to-UE L1 CLI measurement). L1 UE-to-UE CLI measurement and reporting specified by RAN1 can be applied for both application scenarios. For the case of intra-cell L1 SRS-RSRP measurement and reporting, a victim UE may be configured with a set of SRS-RSRP measurement resources that overlap with the resources used by the aggressor UE(s) to transmit SRS since the gNB has knowledge the aggressor UE’s SRS configurations. However, for inter-cell L1 SRS-RSRP measurement, SRS resource set configurations may need to be exchanged among gNBs. But specification impact of this in Xn/F1 has been excluded according the following note from the WID - The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs From RAN1 perspective, intra-cell UE-to-UE L1 CLI measurement scenario can be prioritized for the requirements of L1 SRS-RSRP measurement.

From RAN1 perspective, there is no distinction between the intra-cell UE-to-UE L1 CLI measurement and inter-cell UE-to-UE L1-CLI measurement, but intra-cell scenario can be prioritized.

We also update our simulation results in previous meetings and observed the simulation results from other companies.

The observations from all simulations results:

Observation 1: For case 2, intra-cell case, with the number of samples for measurement increased, the improvement is not so larger, it is less than 0.5dB by extended to 3 samples to 1 sample.

However, for inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

Observation 2: For inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

By our preference, prioritized intra-cell scenario for single set of requirements with one shot measurement is enough.

Proposal 1: For L1-SRS-RSRP, use intra-cell scenario for single set of requirements with one Side condition and accuracy.

Consider the usage of SBFD, the existing 48PRBs are a little limited for NW implementation for this feature. We support to change the SRS bandwidth to >24PRBs.

Proposal 2: For L1-SRS-RSRP, change the SRS bandwidth to >24PRBs for the accuracy requirements.

L1-CLI-RSSI measurement accuracy

In existing RAN4 requirements, L3 CLI-RSSI accuracy is defined in 38.133 10.1.22.2.

In RAN1 agreement of CLI-RSSI,

Method#1: UE measures RSSI within DL subband

Method #3: UE measures RSSI within UL subband

Measurement resource type#1: One CLI-RSSI measurement resource is configured within a DL subband

Measurement resource type#2: One CLI-RSSI measurement resource is configured across two DL subbands

It is agreed in RAN1 the RSSI report should be a wideband reporting. The measurement for Method#1 and Method#3 all reflects to the received power on the measurement resources for REs. From the link level simulation, there is no difference for the power sum up for all the REs of the measurement resources. Therefore, we think the R16 accuracy CLI-RSSI requirements can be reused in Rel-19 L1-CLI-RSSI measurement.

Proposal 3: For L1-CLI-RSSI, reuse R16 accuracy requirements for CLI-RSSI for both Measurement resource type#1 and Measurement resource type#2. The R16 accuracy requirements can be reused for Method#3.

Test cases

In core requirements for Rel-19 SBFD, RAN4 will finalized the new core requirements for:

- L1-SRS-RSRP

- L1-CLI-RSSI

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

In general, RAN4 should add new test cases to verify the new core requirements above.

For SSB based measurement, although there are no new core requirements. RAN4 to reuse the existing requirements. However, SSB measurements are quite important. We need to add test cases for SBFD-aware UE to verify the SSB measurement as well.

Agreement in RAN4#112 Issue 2-3-2: Requirements for SSB based measurement Agreement: - For SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. Further discuss for SSB based neighbour cell measurement in RAN4. - Note: The serving cell is SBFD cell, and the neighbour cell is SBFD or non-SBFD cell. Agreement in RAN4#113 #### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - For SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

Proposal 4: RAN4 to add new test cases for SBFD-aware UE for:

- L1-SRS-RSRP

- L1-CLI-RSSI (cover Method#1 and Method#3)

- SSB based measurement for L1 and L3 measurements.

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

For L1 measurement, RAN4 to add the TCs to verify the measurement resource is collision with UL transmission.

For the general test configuration like DU and/or DUD case, RAN4 to decide for each one when core requirements are finalized.

- Conclusion

In this contribution, we analyze the potential impact on RRM performance part of Rel-19 SBFD and provide our proposals:

Observation 1: For case 2, intra-cell case, with the number of samples for measurement increased, the improvement is not so larger, it is less than 0.5dB by extended to 3 samples to 1 sample.

Observation 2: For inter-cell case, one sample measurement degraded a lot especially in FR2 deployments.

Proposal 1: For L1-SRS-RSRP, use intra-cell scenario for single set of requirements with one Side condition and accuracy.

Proposal 2: For L1-SRS-RSRP, change the SRS bandwidth to >24PRBs for the accuracy requirements.

Proposal 3: For L1-CLI-RSSI, reuse R16 accuracy requirements for CLI-RSSI for both Measurement resource type#1 and Measurement resource type#2. The R16 accuracy requirements can be reused for Method#3.

Proposal 4: RAN4 to add new test cases for SBFD-aware UE for:

- L1-SRS-RSRP

- L1-CLI-RSSI (cover Method#1 and Method#3)

- SSB based measurement for L1 and L3 measurements.

- CSI-RS based RLM/BFD/CBD evaluation

- CSI-RS based L1-RSRP evaluation

- CSI-RS based L1-SINR evaluation

- CSI-RS L3 measurement

For L1 measurement, RAN4 to add the TCs to verify the measurement resource is collision with UL transmission.

For the general test configuration like DU and/or DUD case, RAN4 to decide for each one when core requirements are finalized.

- Reference

[1] RP-251874, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei

[2] R4-2508459, Draft big CR for RRM requirements of Sub-band full duplex, Huawei, HiSilicon, Samsung

[3] R4-2508274, WF on RRM requirements for NR_duplex_evo, Huawei