3GPP TSG-RAN WG4 Meeting #116		R4-2509840

Bengaluru, India, August 25th – 29th, 2025

Agenda item:		7.27.5.2

Source:		Samsung

Title:	Discussion on RRM requirements of other aspects in Rel-19 NTN phase 3

Document for:		Discussion

- Introduction

In last RAN4#115 meeting, RAN4 achieved agreement in high level for SMTC extension in [1]. In this contribution, we discuss the potential impacts in RAN4 RRM for SMTC.

- Discussion

In the LS from RAN2, RAN2 assume to add further enhancement of SMTC in Rel-19 NTN Phase 3. In last RAN4#115 meeting, RAN4 achieved the consensus of feasibility and reply to RAN2. The details of exact RRM requirements change are still open as below:

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities Agreement: (Ad-hoc) From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs. - In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference: - FFS on the exact RRM requirement change. - FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment Agreement: online From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements. - RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer. Agreement: online The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI. ## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) No discussion in this meeting: - Due to the potential enhancement of the SMTC configuration in Issue #0, companies' views on this matter appeared to be diverse.

In existing RAN4 requirements for both RRC idle mode and RRC connected mode, the assumption is the periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

Observation 1: Periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

Observation 2: SMTC are widely used in so many RRM requirements including:

- Cell reselection in idle mode,

- Handover series,

- RRC re-establishment,

- RRC connection release with redirection

- RLM

- BFD/CBD

- L1 measurement

- L3 measurement

- Etc

If RAN2 enhance the SMTC with different periodicity, first issue for RRM impact is which SMTC should be used? Second issue is to be discussed it what is the relationship between the SSB periodicity and different SMTC sets and condition should be specified for all RRM requirements?

Proposal 1: RAN4 to discuss the impacts by SMTC extension for above table.

In previous RAN2#129bis meeting, the agreements are shown as:

Agreements;

1.	From SSB extension point of view, RAN2 assumes there is no need to introduce new barring bits

2.	We wait for further progress in RAN1 on link level enhancements before further discussing the possible impacts on access barring

3.	RAN2 considers to support configuring two different SMTC periodicities (with different offsets) for SMTCs in one frequency layer for idle, inactive and connected mode. We ask RAN4 whether it is feasible to support this in Rel-19 timeframe (also include previous agreement that at any time the UE will not use more SMTCs in parallel than in previous releases).

4.	We support configuring more than 4 SMTCs per frequency (e.g. 6) for idle/inactive UEs. It will be up to UE implementation to select which of the SMTCs to consider (send this RAN2 decision to RAN4 for checking)

5.	Network can provide assistance information (for Rel-19 UEs, not necessarily supporting DL CE) on the association between SMTC and location to help UE to perform SMTC selection for idle/inactive mode. FFS on the details of location information, e.g. serving cell SSB index, reference location, etc. In any case it is up to UE implementation on how to utilize the assistance information for SMTC selection in idle/inactive mode.

In last RAN2#131 meeting, the agreements and WA are shown as:

Agreements:

1.	the maximum configured SMTCs per frequency for idle/inactive UEs is 6 (can come back if we find and issue)

2.	To support SMTC enhancements for DL CE UE, introduce a new SMTC list instead of extending legacy smtc4list (can consider signalling optimizations for the new list to refer to the content of smtc4list to avoid signalling duplications)

3.	We introduce a location-based SMTC selection procedure where each SMTC can be associated with a reference location of the intended neighbor cells that need to be measured by the UE. FFS if also an SSB-index based SMTC selection is supported

4.	No enhancement for cell selection/reselection for DL CE is pursued (can re-discuss this if we will finally introduce a new barring scheme)

Working Assumption:

1.	We introduce a mechanism to assist the NW to configure the SMTCs in connected mode, according to one of the following 2 options:

Alt 1: UE provide the closest N reference locations/neighbor cells to network.

Alt 2: UE reports the selected SMTCs from configured SMTC set to network.

We continue in the next meeting on whether to confirm the WA (and go for either Alt1 or Alt2) or whether we don’t introduce any enhancement (i.e. not confirm the WA)

Observation 3: RAN2 only introduce the location-based SMTC selection in RRC_Connected mode.

In existing requirements in RRC idle mode like cell reselection, the SMTC & DRX are used to define such requirements.

Table 4.2C.2.3-1: Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra DRX cycle length [s]; Scaling Factor (N1); Tdetect,NR_Intra [s] (number of DRX cycles); Tmeasure,NR_Intra [s] (number of DRX cycles); Tevaluate,NR_Intra [s] (number of DRX cycles). FR1. 0.32; 1; 11.52 x N1 x M2 (36 x N1 x M2); 1.28 x N1 x M2 (4 x N1 x M2); 5.12 x N1 x M2 (16 x N1 x M2). 0.64; 17.92 x N1 (28 x N1); 1.28 x N1 (2 x N1); 5.12 x N1 (8 x N1). 1.28; 32 x N1 (25 x N1); 1.28 x N1 (1 x N1); 6.4 x N1 (5 x N1). 2.56; 58.88 x N1 (23 x N1); 2.56 x N1 (1 x N1); 7.68 x N1 (3 x N1). NOTE 1: M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC 4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected. NOTE 2: The UE is not required to meet the requirements for 2.56 s DRX cycle length for earth-moving LEO deployment..

In the yellow part of existing requirements, it was introduced by smtc2 in Rel-16 enhancement.

In latest RAN2 discussion, the motivation is still the same. From the neighbor cells in the same satellite or different satellite, the SSB periodicity may different. In addition, for different satellites, the propagation delays are also different. By using different SMTCs with different SMTC periodicities is the approach which RAN2 would like to use in Rel-19.

If two different SMTC periodicities are supported, RAN4 existing requirements can be reused if the requirements applied by using the shorter SMTC.

Proposal 2: For idle mode, considering cell reselection requirements, if two different SMTC periodicities are supported and no other enhancement, RAN4 existing requirements can be reused. No requirements if the SMTC is smaller than the actual SSB periodicity among the cells on the same frequency layer.

However, in existing requirements in RRC connected modes, for the same frequency, RAN4 have so many requirements, and some requirements depends on SMTC like the measurement in observation #2. RAN4 can use the NW location-based information to update each requirement. Further information is needed.

Some requirements are considering the collision of SMTCs like serving cell L1-RSRP measurements. Some requirements are assuming the SMTC periodicity from serving cell for the same carrier. The rules and updates for each requirement may be different. RAN4 need to check them case by case.

Proposal 3: For connected mode, if configuring two different SMTC periodicities (with different offsets) in one frequency layer, we observe RAN4 have RRM impacts in different requirements. The support and modification for each requirement are different. RAN4 need to check and discuss them case by case.

Take an example, if the different SMTC periodicities are configured by neighbor cells or different satellites. RAN4 can use the selected SMTC for requirements.

Proposal 4: For the requirements for measurement, RAN4 can use the selected SMTC for requirements. However, how to config by NW, it is not clear in RAN2 now.

For the requirements with default SMTC like RRC Re-establishment:

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i) No discussion in this meeting: Due to the potential enhancement of the SMTC configuration in Issue #0, companies' views on this matter appeared to be diverse.

The default SMTC is the periodicity if the same frequency layer is not configured, the UE assume the SSB periodicity as the 160ms which is same as what we agreed for inter-frequency carrier is not provided.

Proposal 5: For the requirements with default SMTC in Issue 1-2-A, RAN4 assume the value of TSMTC as the 160ms.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for DL coverage enhancement:

Observation 1: Periodicity of multiple SMTCs for the same ssb frequency are the same although multiple SMTC can be supported in Rel-17 NTN.

Observation 2: SMTC are widely used in so many RRM requirements including:

- Cell reselection in idle mode,

- Handover series,

- RRC re-establishment,

- RRC connection release with redirection

- RLM

- BFD/CBD

- L1 measurement

- L3 measurement

- Etc

Proposal 1: RAN4 to discuss the impacts by SMTC extension for above table.

Observation 3: RAN2 only introduce the location-based SMTC selection in RRC_Connected mode.

Proposal 2: For idle mode, considering cell reselection requirements, if two different SMTC periodicities are supported and no other enhancement, RAN4 existing requirements can be reused. No requirements if the SMTC is smaller than the actual SSB periodicity among the cells on the same frequency layer.

Proposal 3: For connected mode, if configuring two different SMTC periodicities (with different offsets) in one frequency layer, we observe RAN4 have RRM impacts in different requirements. The support and modification for each requirement are different. RAN4 need to check and discuss them case by case.

Proposal 4: For the requirements for measurement, RAN4 can use the selected SMTC for requirements. However, how to config by NW, it is not clear in RAN2 now.

Proposal 5: For the requirements with default SMTC in Issue 1-2-A, RAN4 assume the value of TSMTC as the 160ms.

- Reference

[1] R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Qualcomm