3GPP TSG-RAN WG4 Meeting #116	R4-2509856

Bengaluru, India, 25-29 August, 2025

Source: 	Qualcomm Incorporated

Title: 	Views on SCM for UE Demodulation

Agenda item:	 7.12.3

Document for:	Agreement

- Introduction

In the previous RAN4 Meeting #115, we contributed to the SCM Study Item sharing considerations on the Study Item on the use spatial channel model for demodulation performance requirements.

According to the outcome of the discussion during the meeting and based on the approved Way Forward [1], we provide in this contribution further observations as part of the ongoing study.

# Views on CDL modeling

## Angular Scaling approach

In previous RAN4 meetings, including the last RAN4 meeting #115, the interpretation of the angular scaling procedure has been subject of multiple discussions, during and in between meetings.

The discussions regarded for example whether to implement the scaling per-ray or per cluster, how to apply wrapping, and how to interprete the details of the formulation currently included in 3GPP TR 38.901, which can be open to different interpretations.

In the last plenary meeting, RAN has approved the new version of TR38.901, version 19.0.0, and it is our view that section 7.7.5.1 now clearly specifies the procedures to apply the angular scaling and translation, and we have iincluded a reference to that in the new pCR for 38.753.

Proposal 1: RAN4 to use the updated angular scaling procedure in TR38.901, version 19.0.0, section 7.7.5.1.

However, it should be noted that the procedure does not yet provide a dedicated expression to compute a scaled and translated cluster angle, but only a ray angle. Given that the RAN4 procedure detailed in 38.753 references cluster angles (after the scaling procedure) in order to apply the cluster reduction algorithm for both spatial filtering and for creating the final cluster profile table, we have proposed a companion draftCR for endorsement in R4-2509115 to introduce Equation 7.7-5d, which includes a variable for the output cluster angle.

Proposal 2: RAN4 to endorse the draftCR R4-2509115 that proposes to introduce a variable to capture the scaled and translated cluster angle. A mirror CR is submitted to RAN1 for approval.

## Changes to the Cluster Reduction procedure

In the companion pCR to this contribution (R4-2509195) we have updated the cluster reduction procedure to better align with the new angular scaling procedure defined in the latest 38.901 v19.0.0 TR.

Certain changes, in particular the detail of the spatial filtering being done using the cluster angle and ray angle might have minor impact on the outuput CDL tables, which might differ from the one used to generate results so far. However, we do not believe that this has an impact on the validity of the alignment and conclusions reached on this SI.

Proposal 3: RAN4 should capture in TR38753 that the latest version of the CDL cluster reduction procedure as submitted to RAN4#116 could result in a reduced CDL table with minor deviations with respect to the one used to generate simulation results, however no impact is expected to the conclusions.

# Notes on the time-domain behaviour of the TR 38.753 CDL channel

In the WF from the last meeting one of the SCM channel properties captured for comparison was the “Time Coherence as channel correlation between symbols”. We would like to provide the qualitative observations below related to this metric in this section

We have investigated the time evolution of CDL channel coefficients, which results to be deterministic, given the complex gain per each ray at time t=0. That is because, according to the agreements so far, the radiation pattern gains, analog beamforming gains, polarization leakage, initial phases, etc, can all be combined into ray gains that depend on the realization (initial seed) but are static for every run. Each cluster is then composed by a set of 20 rays, which sum up to the cluster average power according to the CDL profile table.

Our study of the autocorrelation of the cluster (e.g., the dominant clusters) shows a periodic nature due to the inherent periodicity of the cluster’s ray components, with high level of possibly unwanted correlation persisting for a very long duration. Additionally, correlation is pronounced between the in-phase and quadrature coefficient components. It is also worth noting that the autocorrelation does not depend on the initial seed, as the time evolution of the cluster rays is deterministic.

Proposal 4: RAN4 to capture the qualitative observations included in this sections related to the autocorrelation function of the CDL clusters. In detail, we have observed its long duration and its high value between I/Q components.

- Conclusions

Proposal 1: RAN4 to use the updated angular scaling procedure in TR38.901, version 19.0.0, section 7.7.5.1.

Proposal 2: RAN4 to endorse the draftCR R4-2509115 that proposes to introduce a variable to capture the scaled and translated cluster angle. A mirror CR is submitted to RAN1 for approval.

Proposal 3: RAN4 should capture in TR38753 that the latest version of the CDL cluster reduction procedure as submitted to RAN4#116 could result in a reduced CDL table with minor deviations with respect to the one used to generate simulation results, however no impact is expected to the conclusions.

Proposal 4: RAN4 to capture the qualitative observations included in this sections related to the autocorrelation function of the CDL clusters. In detail, we have observed its long duration and its high value between I/Q components.

- References

[2] “R4-2508776 Way Forward for [115][321] NR_SCM”, 3GPP TSG-RAN WG4 Meeting #115