3GPP TSG-RAN WG4 Meeting #116															R4-2509902

Bangalore, India, 25th – 29th Aug 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | 2933 | rev | - | Current version: | 16.23.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_newRAT-Core) R16 correction on modifiedMPR-Behaviour (refer to latest spec version) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | OPPO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2025-08-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - In some of the bands, the meaning of “if the bit is not set” is confusing, it could be not set to 1 or not set to 0 but the meaning is opposite. It should be “if the bit is not set to 1”. - Regarding the spec version referred in the table, issues exist: - Sometimes it refers to an old version specification whose requirements has been changed/corrected in later releases. If the old version specification is used then the corrections maded in later versions will be bypassed. - For example, n41 bit 0 and 1 refers to clause 6.2B.2.1 of 38.101-3 v15.5.0, but the requirements in this version is still in “[ ]”, and requirements are corrected in later versions. - Another issue is about n41 where the bit 0/1 is controdicting with bit 2 in the application of MPR. Currently it says bit 0/1 shall set to 1, and bit 2 may be set to 1. These bits refer to different specification versions but actually they are similar except one typo crrection of table number. Therefore, to solve this controdiction, the MPR in bit 2 could be removed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Change from “if the bit is not set” to “if the bit is not set to 1”. - Replace the old version of specification with the latest version. - Remove MPR from bit 2 of n41. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will still unclear or even incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | L.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

************************<Start of change> ********************************************

L.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPR-Behavior indicated per supported NR band in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a given version of this specification. A modified MPR or A-MPR behaviour can apply to a supported NR band in stand-alone operation (including CA and NN-DC operation) or in non-standalone operation with the said NR band as part of an EN-DC or NE-DC band combination.

NOTE 1:	In the present release, the modifiedMPR-Behavior is indicated [7] by an 8-bit bitmap per supported NR band.

Table L.1-1: Definitions of the bits in the field modifiedMPR-Behavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 with latest Rel-15 specification | - This bit shall be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 with latest Rel-15 specification | - This bit shall be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 2 | - EN-DC contiguous and non-contiguous intraband A-MPR as defined in 38.101-3 with latest Rel-16 specification. If this bit is not set to 1 the UE uses Rel-15 A-MPR for EN-DC contiguous and non-contiguous intraband A-MPR | -This bit may be set to 1 by a UE supporting DC_(n)41AA or DC_41A_n41A EN-DC |
| n71 | 0 (leftmost bit) | - EN-DC contiguous intraband MPR as defined in clause 6.2B.2.1 of 38.101-3 with latest Rel-15 specification | - This bit shall be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |

************************End of change ***************************************************