3GPP TSG-RAN4 Meeting #116	R4-2509919

# Bengaluru, India, 25th - 29th August, 2025

Agenda item:		7.10.3.1

Source:				Keysight Technologies UK Ltd

Title:					On XR OTA Testing with Partial Radiated Surfaces

Document for:		Approval

# Introduction

This contribution is a continuation of [1] regarding partial radiated surfaces applied to XR OTA testing while taking agreements from the last meeting [2] into account.

# Discussion

In RAN4#115, a number of operators suggested new measurement approaches/techniques [3]. These and others, discussed during the last meeting, e.g., near horizon metrics, [2]

Issue 1-2-3: New performance metric for XR devices Agreements: RAN4 further study new metric and corresponding test procedure/data processing for XR devices, e.g.,. ⦁ Weighted TRP & TRS ⦁ Near horizontal part radiated power (NHPRP) and Near horizontal part Isotropic Sensitivity (NHPIS) ⦁ Others If no consensus reached, the WI can also be closed with agreed TRP/TRS metric for XR devices.

are further reviewed here.

The operators suggested a novel approach to define metrics/requirements by defining “high-priority directions” and “low-priority directions” for XR OTA testing [3]

General XR TRP/TRS test procedure The current TRP/TRS minimum requirement approach has been sufficient for many devices, but it presents a significant shortcoming for XR headworn devices, which are worn with a consistent orientation against the user's body. In such scenarios, even if the overall TRP/TRS meets the required standard, a few low-power measurements (deep nulls) in certain directions may lead to inconsistent coverage. For XR headworn devices, where performance uniformity is crucial for maintaining seamless connectivity in an immersive experience, such nulls can lead to user dissatisfaction and degraded performance and coverage. These issues underscore the need for a more granular analysis approach that does not allow the averaging process to obscure significant performance deficiencies. The fixed orientation of XR headworn makes them particularly vulnerable to the effects of deep nulls, and with the reduced RX antennas, it is imperative that evaluation and minimum requirement methods evolve accordingly. At the same time, the fixed orientation can be exploited to optimize antenna designs to prioritize certain directions and deprioritize others. Proposal 1: Introduce high priority and low priority directions for XR headworn TRP/TRS minimum requirements, each with its own minimum TRP/TRS values. Relaxed minimum requirements are expected for low priority directions, while higher minimum requirements are expected for high priority directions. The determination of the range for both directions can be discussed. Proposal 2: Introduce a new metric for XR headworn devices as an additional minimum requirement for high priority directions. This metric could be implemented without altering the current measurement procedure or requiring additional equipment, ensuring seamless integration into existing testing frameworks. Proposal 3: The following are the key elements for introducing a new metric for XR headworn: - Measurement Procedures: The standard method for TRP measurement could remain unchanged. Effective Isotropic Radiated Power (EIRP) and Effective Isotropic Sensitivity (EIS), are measured and saved for each angle/position. - Introducing a Post-Measurement Threshold Analysis: Once the data is collected, each individual measurement point is compared against threshold values of EIRP and EIS. Determining the exact threshold values will be FFS. Every measurement point must meet or exceed the agreed-upon threshold (above min EIRP and EIS threshold values). Alternative test solutions One of the challenges for XR headworn devices during OTA testing is their lower battery life compared to handheld devices. Increasing the grid size for TRP and TRS testing can help reduce the measurement time. Proposal 4: Increase the measurement grid size for XR low priority directions.

Examples of high-priority directions are shown in Figure 1 for two prominent VR devices in the market. The most logical approach to define high-priority directions is to consider conical regions originating from potential regions where antennas could be integrated in an XR device which are most likely the front as well as the sides of the DUT. As such, the high-priority conical directions are shown with the yellow cones towards the sides (with ±30° angular width) and towards the front (with ±60° angular width); the low-prio directions are assumed to be outside those cones.

Figure 1: Visualization of high-prio directions for two prominent VR devices.

Regarding Proposal 1 from [3], it does not really seem appropriate to define “relaxed minimum requirements” for the low-priority directions as the radiation in those directions could/should be minimized to optimize the coverage in the high-priority directions and the requirements should be applicable to the directions that matter only.

Proposal 1: If low and high-priority regions are defined, define requirements for the high-priority directions only, i.e., not for the low-priority directions

Figure 2 shows the same VR devices with the classical measurement grid of Dq=Df=15°. The grid points inside the high-priority directions (enclosed by the yellow circles/cones) are marked in green while the grid points outside the high-priority directions are marked in red.

Figure 2: Visualization of grid points inside and outside the high-prio directions for two prominent VR devices.

Alternate high-priority directions/cone angles are shown in Figure 3 for one XR device only.

Figure 3: Visualization of alternate high-priority areas; left: front&side cone angles: ±45°; centre: front cone angle: ±60°, side cone angle: ±45°, right: front&side cone angles: ±60°

Conical partial surfaces are considered for NTN OTA testing; however, concerns were raised about the integration over arbitrary directions and/or directions other than the top&bottom poles for complexity reasons [4]. It was subsequently agreed to consider partial spheres around the top pole only [2]

Issue 2-1-4: Whether the partial sphere is using the =0° (top pole) as a center Agreements: the partial sphere metric is using the  =0° (top pole) as a center

It is therefore proposed not to consider multiple partial conical surfaces as high-priority directions for the same complexity concerns as voiced in [4]

Proposal 2: Do not consider multiple partial conical surfaces as high-priority directions to avoid complexity for system vendors

Instead, it is proposed to consider near-horizon partial surfaces that seem more suitable for XR devices given the typical use case with consistent orientation of the head and the primary direction of base stations in the hear horizon range as well as the lack of importance to prioritize radiation towards the top (+z) and bottom (-z) of the DUTs. A near-horizon partial surface approach is visualized in Figure 4. Here, the angular range measured from the horizon is ±30° with a full azimuthal coverage; the high-priority directions are within the yellow latitude lines and the grid points within the high-priority directions are marked in green while the grid points outside those the high-priority directions are marked in red.

Figure 4: Visualization of near-horizon partial surface with ±30° angular width.

Alternate near-horizon partial surfaces are shown Figure 5 including a novel near-horizon approach that limits the azimuthal range and not considers radiation towards the back of the XR devices.

It is proposed to consider the near-horizon partial integrated metrics, NHPRP (Near Horizon Partial Radiated Power) for TX and NHPRS (Near Horizon Partial Radiated Sensitivity) for RX, for XR devices rather than full TRP/TRS metrics as this will assess performance in the directions that matter most for these types of devices and it supports a significant reduction in test time which could be problematic for some XR device types, e.g., glasses with limited battery capacity. Feedback from operators and OEMs is requested on the range of near horizon coverage and whether full (360°) or rather partial azimuthal coverage (270°) is more desirable.

Proposal 3: If high-priority regions are considered for XR devices, select the near-horizon as high-priority direction and define near-horizon partial integrated metrics, NHPRP for TX and NHPRS for RX, for XR devices rather than full TRP/TRS metrics

Proposal 4: Feedback from operators and OEMs is requested on the range of near horizon coverage, e.g., ±30° or ±45°, and whether full (360°) or rather partial azimuthal coverage (270°) is more desirable

Figure 5: Visualization of alternate near-horizon partial surfaces, top: ±45° angular width with full azimuthal coverage, centre: ±45° angular width with 270° azimuthal coverage, bottom: ±30° angular width with 270° azimuthal coverage.

# Conclusion

The following observations and conclusions were made in this contribution.

Proposal 1: If low and high-priority regions are defined, define requirements for the high-priority directions only, i.e., not for the low-priority directions

Proposal 2: Do not consider multiple partial conical surfaces as high-priority directions to avoid complexity for system vendors

Proposal 3: If high-priority regions are considered for XR devices, select the near-horizon as high-priority direction and define near-horizon partial integrated metrics, NHPRP for TX and NHPRS for RX, for XR devices rather than full TRP/TRS metrics

Proposal 4: Feedback from operators and OEMs is requested on the range of near horizon coverage, e.g., ±30° or ±45°, and whether full (360°) or rather partial azimuthal coverage (270°) is more desirable

# References

- R4-2505849, On Partial Radiated Surfaces, Keysight Technologies, 3GPP TSG-RAN4 Meeting #115, May 2025

- R4-2508782, Way Forward for [115][337] TRP_TRS_MIMO_OTA_Ph3, vivo, 3GPP TSG-RAN WG4 Meeting #115, May 2025

- R4- 2507005, Discussion on OTA requirements for XR Headworn Devices, Vodafone, Telecom Italia S.p.A., Deutsche Telekom, 3GPP TSG RAN WG4 Meeting #115, May 2025

- R4-2505849, On Partial Radiated Surfaces, Keysight Technologies, 3GPP TSG-RAN4 Meeting #115, May 2025