3GPP TSG-RAN WG4 Meeting #116	R4-2509920

Bangalore, India, August 25th – August 29th, 2025

Source:	Nokia

Title:	On General Aspects of AI/ML for NR Air Interface Phase 1

Agenda item:	7.17.2

Document for:	Discussion

Introduction

In RAN4#115 meeting, RAN4 continued the discussion on the general aspects of AI/ML based on the objectives agreed in RAN4#107, WI ‎‎[1].

In this paper, we share our further views on the following topics, mainly for UE sided model cases:

- Post deployment testing

- Life-Cycle Management related aspects

- On Generalization, number of tests and non-stationary conditions

Discussion

Post deployment testing

On the topic of post-deployment testing, following agreement was reached in RAN4#115:

Agreements: Option 2 (Post-deployment post-activation functionality testing based on performance monitoring) is prioritized. - it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases

The proposed definition of test-active configuration (for post-deployment testing Option2) provided in [Nokia R4-2507429] was:  the “functionality/ configuration which is available in (supported by the) UE, it is RRC configured by the gNB/NW for post-deployment testing and, has been RRC/MAC activated by the gNB/NW for post-deployment testing”. Furthermore, it was indicated that in post-deployment Option2, the UE is configured and provides monitoring reports; the NW takes ML management (“LCM”) actions (RRC re-configurations) based on monitoring reports and the in/applicability indications received from the UE.

Based on the current RAN4 agreements for rel. 19 on performance monitoring, RAN4 to consider the test-active functionality/ configuration for post-deployment validation.

We recognize, this definition of test-active configuration for post-deployment testing Option2, requires further clarifications.

From our perspective, a key difference between an activated configuration (RAN2) and test-active configuration is that for test-active configuration, the UE only needs to report the performance monitoring metrics and not necessarily the inference outputs (measurement reports, etc) to the NW.

The performance monitoring metrics to be reported for a test-active configuration may also be configured differently if necessary and depending on the use case, based on the configuration capabilities/options defined in RAN1. For example, the performance monitoring configuration for test-active could contain the ground-truth, inference output, and inputs in some use cases.

An example for possible test-active configuration of the performance monitoring in the beam management use case BM-Case1, is given below based on the current RAN1 agreements [RP-251185]. The related RAN1 #120bis and #121 working assumption and agreements are (monitoring Type 1 Option 2 corresponds to UE-assisted performance monitoring):

Working assumption For BM-Case 1, the beam prediction accuracy is calculated based on N latest transmission occasion(s) of monitoring resources with linked inference report no later than CSI reference resource corresponding to the CSI report for monitoring ​ - wherein N (N>=1) is configured in CSI-ReportConfig​ - FFS on additional rule for counting N linked pair​ - For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring Agreement For UE-sided model monitoring Type 1 option 2, regarding the type of resource for the set for monitoring, support at least periodic CSI-RS, semi-persistent CSI-RS and SSB Agreement For UE-sided model monitoring Type 1 option 2, support the following combination for inference report type and monitoring report type: Monitoring report type Inference report type; P report; SP report; AP report. AP report; Not support; Not support; Support. SP report; Not support; Support; Support. P report; Support; Support; Support. Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1) Agreement For BM-Case 1, one resource set for monitoring is configured in one CSI-ReportConfig for monitoring.

Based on these design agreements in RAN1​, different parameter values can be provided in CSI-ReportConfig to the UE for monitoring and for test-active monitoring configurations, e.g. for the parameters N (used to derive the RS-PAI report quantity) and for the monitoring type (AP, SP or P):

- Performance monitoring Config, with N = 1 and type SP,

- Test-active performance monitoring Config with N = 7 and type P

Furthermore, the activated configuration (RAN2) and a test-active configuration may or may not need to be used by the UE simultaneously, depending on the UE capabilities and the configuration parameters provided. There could be UEs which have processing capacity, and indicate availability of, to execute both in parallel, i.e. provide to the NW the usual reports based on the activated configuration #1, while only sending monitoring reports for test-active configuration #2. For other less capable UEs, the reporting for the two configurations can be executed in time multiplexed manner.

Another case is when the test-active configuration is known to use AI/ML algorithms (based on UE capability) while the normal active configuration is for a non-AI/ML (version of) the same feature. In this case the outcome of the performance monitoring configuration for test-active can be compared to the non-AI/ML measurement reports used as ground-truth information.

The test-active configuration and corresponding performance monitoring results collected by the NW are UE specific, just like any other RRC configuration. Hence, the test-active configuration is valid for UEs and for the target feature (use case) and not for any specific ML models (nor their providers) used in the UE.

The NW can trigger the test-active configuration to be used by the UE (via RRC/MAC) for example, when the UE indicates the applicability of a functionality/configurations which was previously indicated as inapplicable to the NW. In principle, a configuration being inapplicable could be for a variety of reasons, like low power in the UE, low AI/ML processing resources in the UE, the lack of model availability based on the UE’s velocity, etc. None of these mean that the UE cannot perform well with the same configuration later, hence reported as not applicable.

A test-active configuration can be provided/signalled and triggered by the Network (gNB/LMF) to the UE using RRC/MAC signalling for transitions from inapplicability to applicability or first applicability report of a non-conformance tested configuration (if detectable).

For practical reasons, the NW may keep using the currently active applicable configuration, either AI or non-AI, for this UE (including performance monitoring for it in case of AI configuration) and switch to another configuration (declared by the UE as applicable) only when unacceptable performance degradation is detected for the currently active configuration or there is need for re-pairing of UE and NW side configurations (two sided models). Naturally, the NW is not expected to have to trigger the test-active configuration for each transition from inapplicable to applicable reported by the UE. The actual trigger decision criteria are left for the NW implementation.

The newly tested AI/ML configuration (which was in test-active mode) may not necessarily be activated immediately after the testing. The actual activation decision criteria is left for the NW implementation.

Lastly, the performance monitoring testing requirements designed in RAN4 for each use case, will be applicable also to the testing of the test-active configuration. This includes the testing of different performance metrics reporting. An additional RAN4 test requirement could be to have both an activated configuration and a test-active configuration provided to the UE, and executed depending on the UE capabilities (as explained above).

The introduction of the test-active configuration can lead to the need to adjust the corresponding RAN2 procedures e.g., re-configuration messages of the performance monitoring to enable monitoring of the test-active functionality/configuration, in RAN2

RAN4 agreed to define performance requirement for conformance testing for AI/ML use cases. Furthermore, RAN4 is currently discussing if any new requirements for performance monitoring metrics need to be considered. Since, post-deployment option 2 is based on performance monitoring framework, no additional requirement than requirements for performance monitoring metrics and reporting should be needed for post-deployment validation.

Based on the current RAN1 and RAN2 agreements for Release 19 on performance monitoring, RAN4 to not consider new requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration.

For the UEs deployed in the field, the AI/ML model/functionality updates can happen either in parallel or in batches. Therefore, it might happen that multiple UEs with the updated AI/ML functionality reports the functionality/configuration applicability update to the gNB/Network simultaneously or within a very short interval of time. In such cases, one possibility is that the gNB/Network will perform each of the required post-deployment tests for each selected UE where the AI/ML functionality applicability update happened. However, this approach is not optimal in the sense that: (1) It requires significant time-frequency resources to perform the required post-deployment tests in each selected UE, (2) Performing each post-deployment test run in each UE will increase the overall testing-time and drains more UE battery. Furthermore, there is a high probability that UEs reporting AI/ML functionality applicability update in a short period of time belong to the same UE vendor with similar or same AI/ML functionality. Therefore, running individual tests in each of such UEs will not consider the above fact to optimize the in-field testing.

Hence, there is a need to design an “optimal” in-field testing framework to enable the post-deployment testing of multiple UEs deployed in the field which results in optimal usage of the network resources and enhances the reliability of the in-field testing/monitoring.

RAN4 to investigate the potential solutions which can enable improved in-field post-deployment testing/monitoring of multiple UEs resulting in efficient usage of the network and UE resources.

Life-Cycle Management related aspects

Requirements for performance monitoring reporting

Following agreement was reached in RAN4#114bis meeting on the delay requirements for reporting/monitoring:

Agreement: - Delay requirements for performance monitoring reporting from the UE (e.g.., Type 1 Option2, UE-assisted monitoring for BM) in case-by-case manner - FFS on which use cases the requirement will be defined for - FFS on the details for each use case in WID

Agreement from RAN4#115:

Agreement: Monitoring reporting delay and accuracy: Discuss how to define delay and accuracy requirements for UE sided monitoring where the UE reports the metric: - BM-Case1 and possibly BM-Case 2 o Performance monitoring type 1 option 2 - If it proves to be not possible to define, then reverse the agreement - Note: For BM-Case2, it may not be completed in Rel-19. If it is not covered in Rel-19, there would not be a monitoring delay requirement. If it would be in Rel-19, a monitoring delay requirement is needed. CSI prediction: - CSI prediction, the details are subject to RAN1 decision - RAN4 will further study the feasibility to specify the corresponding delay and/or accuracy requirements for UE sided monitoring

RAN1 agreement about performance monitoring metric:

RAN1#121 Agreement For beam prediction accuracy report for monitoring, the report quantity RS-PAI is Np (0 ≤ Np≤ N) - Where Np is the total count of accurate reference signal prediction instance(s) that meets the condition, among N latest transmission occasion(s) of monitoring resources that no later than CSI reference resource corresponding to the CSI report for monitoring - condition: - for the transmission occasion of monitoring resources, it has a linked inference report - at least one of the nrofBestBeamforMonitoring-r19 identified CSI-RS resources, or SS/PBCH Block resources mapped to one of the nrofreportedpredictedrs-r19 reported P-CRI(s) or P-SSBRI(s), of the linked report of the CSI Reporting Setting for inference - if this condition is met, the transmission occasion is counted as an accurate reference signal prediction instance; otherwise, it is not counted as an accurate reference signal prediction instance. - Where N = 1, 3, 7, 15 is configured in CSI-ReportConfig - the size of CSI field associated with the RS-PAI is log2(N+1)

From the RAN1 definition of the report quantity of the beam prediction accuracy report for monitoring, we can see that this performance monitoring metric is different from the beam prediction accuracy metrics being discussed in RAN4 as part of the performance requirements. Hence, RAN4 can define accuracy requirements for this metric, which can be tested at least in static conditions.

RAN4 to define accuracy requirements for UE-assisted monitoring where the UE reports the monitoring metric (i.e., monitoring Type 1 Option 2) in BM-Case1 and BM-Case2. The accuracy of the performance monitoring metric should be tested at least in static radio conditions.

Similar to accuracy requirements, RAN4 can also define delay requirements for RS-PAI reporting in BM-Case1 and BM-Case2, where the starting point of RS-PAI reporting delay is when UE receives the monitoring RS resources, while the ending point of this delay can be when UE reports the performance monitoring metric, RS-PAI.

RAN4 to define delay requirements for UE-assisted monitoring where the UE reports the performance monitoring metric, RS-PAI, in BM-Case1 and BM-Case2. The starting point of this delay can be when UE sends RRCReconfigurationComplete message in response to the configuration of monitoring RS resources via RRCReconfiguration, and the ending point can be when UE reports the first performance monitoring metric, RS-PAI.

To ensure reliable performance monitoring of AI/ML-based CSI prediction, it is important to define accuracy requirements for the similarity metrics reported by the UE, such as SGCS1. SGCS1 quantifies the similarity between predicted and ground-truth CSI, and under controlled static conditions, a well-functioning prediction model should yield values close to 1. By establishing threshold-based criteria (e.g., SGCS1 ≥ 0.8), RAN4 can enable meaningful evaluation of prediction accuracy using test equipment, even without visibility into the internal AI/ML model. This supports consistent and trustworthy performance monitoring across implementations.

Threshold-based criteria with respect to the reported SGCS under static channel conditions can ensure that the reported SGCS metrics reliably reflect the prediction quality of the AI/ML model, supporting consistent and interpretable performance monitoring.

RAN4 to define accuracy requirements for CSI prediction performance monitoring, specifically for UE-reported SGCS1 values.

In the context of AI/ML-based CSI prediction, the reporting delay is defined as the time interval from the RRC (re)configuration, where the network activates the AIML functionality and configures the monitoring resources, to the transmission of the first performance monitoring report by the UE. This definition aligns with the framework used in beam management use cases and ensures consistency across different monitoring scenarios. The delay requirement is particularly critical for the first monitoring occasion, as it reflects the time needed for the UE to process the configuration, activate the prediction model, and generate the initial SGCS-based report. Establishing a clear delay requirement helps validate the responsiveness and readiness of the prediction functionality under the Type 3 monitoring framework agreed in RAN1.

RAN4 to define the delay requirement for the first performance monitoring report in CSI prediction, based on the time from RRC (re)configuration Complete to the transmission of the initial SGCS-based report, in alignment with the beam management framework.

Delay requirements for activation

The following agreement was reached in RAN4#115 meeting:

Activation delay: - Activation delay requirements created for the periodic, semi-persistent and aperiodic CSI reporting for beam prediction - It is FFS for CSI prediction - For Option A, RAN4 will send LS to RAN2 to clarify the related mechanism. - For semi-persistent CSI reporting - Activation delay starts at the reception of the MAC-CE/DCI - For aperiodic CSI reporting - Activation delay starts at the reception of the DCI

In RAN4#115 meeting, RAN4 sent an LS to RAN2 to clarify if the applicable functionality is already activated before reporting applicable functionalities via RRCReconfigurationComplete in step 4 or UE starts to activate the applicable functionality upon reporting applicable functionalities via RRCReconfigurationComplete in step 4?

As per our understanding, some processing delay may be needed to activate the applicable functionality upon reporting applicable functionalities via RRCReconfigurationComplete in step 4.

RAN4 to consider RRCReconfigurationComplete message containing applicable functionality report as the starting point to define activation delay requirement for periodic CSI reporting for AI/ML BM use case.

RAN4 to consider first inference report as the end point to define activation delay requirements for periodic/aperiodic/semi-persistent CSI reporting for AI/ML BM use cases.

Delay requirements for deactivation

WF from RAN4#115 meeting:

WF: Deactivation delay for CSI prediction: - The following questions were noted for further discussion - Can NW differentiate between the predicted CSI reporting and the measured CSI reporting? - In case of switch between AI, is there existing reporting to differentiate the reports before and after the deactivation.

Since for the network sided operation, network will rely on the reports of the activated AI/ML configurations and since multiple AI/ML or non-AI/ML configurations can be configured on the UE simultaneously, Network sided operations may not be impacted much by the untimely deactivation of an AI/ML configuration.

RAN4 to not define any deactivation delay requirements for AI/ML configurations.

On Generalization, number of tests and non-stationary conditions.

The following Agreements were reached in RAN4#115:

Agreements: The high level and general guidelines for the generalization test are provided as follow. The exact decisions are to be made case by case. - Study and , if feasible, define requirements for each AI/ML functionality - Define one test per UE capability as a minimum - FFS >1 test per UE capability - Define a minimum set of test configurations (including NW sided conditions if any) as mandatory for testing of AI/ML-enabled Feature Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

The current UE AI/ML feature list support discussed in RAN1 is captured in R1-2504673 for the Beam Management, CSI Prediction and Positioning AI/ML use cases. Any ‘AI/ML functionality’ discussed in RAN4 (as in agreement above) will be eventually defined by the associated Feature Group (FG) e.g., FG# 58-1-1 “Increased number of reported RSs for beam management” [R1-2504673] and the corresponding UE feature list parameters as captured in RAN2 (in future TS38.822 versions).

We note the following:

For Release 19, all the NR_AI/ML_air feature groups (58), as currently agreed in R1-2504673, are all marked as ‘Optional with capability signalling’.

For Release 19, all the NR_AI/ML_air feature groups for beam management and CSI prediction use cases (58-0, 58-1 and 58-3) as currently agreed in R1-2504673, are all marked as ‘Need for the gNB to know if the feature is supported’

Therefore, the RAN4 requirements must be studied for the agreed NR_AI/ML_air FGs, potentially including several of their components, determined to be critical for feature performance reasons. The UE (AI/ML) capabilities definitions (in future TS38.306 versions) are using the feature groups and UE feature list parameters.

RAN4 requirements for generalization must be studied at the granularity of the defined NR_AI/ML_air Feature Groups and, including one or more of their defined components and parameters.

Furthermore, on the issue of static and non-static scenarios for testing, following agreement was reached in RAN4#114bis meeting:

Agreement: - Static scenarios are assumed by default. Non-static scenarios are introduced on a use-case specific basis if needed for testing. - FFS whether a mixed datset can be created for testing generalization, and whether such a mixed dataset would be a static or non-static scenario

From our perspective, it won’t be meaningful to agree on a mixed dataset to test generalization aspects, since it will be like testing AI/ML model and not the telecom aspects of the AI/ML functionality. Therefore, a more suitable way to test generalization aspects is by testing it in different scenarios as discussed above.

RAN4 should not create a mixed dataset for generalization testing since the objective is not to test the AI/ML model.

Conclusion

In this contribution, we addressed the following topics mainly for UE sided model cases:

- Post-deployment testing

- Life-Cycle Management related aspects

- Generalization, number of tests and non-stationary conditions

Following Observations and Proposals were made in this paper:

Proposal 1: Based on the current RAN4 agreements for rel. 19 on performance monitoring, RAN4 to consider the test-active functionality/ configuration for post-deployment validation.

Observation 1: A test-active configuration can be provided/signalled and triggered by the Network (gNB/LMF) to the UE using RRC/MAC signalling for transitions from inapplicability to applicability or first applicability report of a non-conformance tested configuration (if detectable).

Observation 2: The newly tested AI/ML configuration (which was in test-active mode) may not necessarily be activated immediately after the testing. The actual activation decision criteria is left for the NW implementation.

Proposal 2: Based on the current RAN1 and RAN2 agreements for Release 19 on performance monitoring, RAN4 to not consider new requirements and tests to ensure the required functioning of performance monitoring procedures for the test-active functionality/ configuration.

Proposal 3: RAN4 to investigate the potential solutions which can enable improved in-field post-deployment testing/monitoring of multiple UEs resulting in efficient usage of the network and UE resources.

Proposal 4: RAN4 to define accuracy requirements for UE-assisted monitoring where the UE reports the monitoring metric (i.e., monitoring Type 1 Option 2) in BM-Case1 and BM-Case2. The accuracy of the performance monitoring metric should be tested at least in static radio conditions.

Proposal 5: RAN4 to define delay requirements for UE-assisted monitoring where the UE reports the performance monitoring metric, RS-PAI, in BM-Case1 and BM-Case2. The starting point of this delay can be when UE sends RRCReconfigurationComplete message in response to the configuration of monitoring RS resources via RRCReconfiguration, and the ending point can be when UE reports the first performance monitoring metric, RS-PAI.

Observation 3: Threshold-based criteria with respect to the reported SGCS under static channel conditions can ensure that the reported SGCS metrics reliably reflect the prediction quality of the AI/ML model, supporting consistent and interpretable performance monitoring.

Proposal 6: RAN4 to define accuracy requirements for CSI prediction performance monitoring, specifically for UE-reported SGCS1 values.

Proposal 7: RAN4 to define the delay requirement for the first performance monitoring report in CSI prediction, based on the time from RRC (re)configuration Complete to the transmission of the initial SGCS-based report, in alignment with the beam management framework.

Proposal 8: RAN4 to consider RRCReconfigurationComplete message containing applicable functionality report as the starting point to define activation delay requirement for periodic CSI reporting for AI/ML BM use case.

Proposal 9: RAN4 to consider first inference report as the end point to define activation delay requirements for periodic/aperiodic/semi-persistent CSI reporting for AI/ML BM use cases.

Proposal 10: RAN4 to not define any deactivation delay requirements for AI/ML configurations

Observation 4: For Release 19, all the NR_AI/ML_air feature groups (58), as currently agreed in R1-2504673, are all marked as ‘Optional with capability signalling’.

Observation 5: For Release 19, all the NR_AI/ML_air feature groups for beam management and CSI prediction use cases (58-0, 58-1 and 58-3) as currently agreed in R1-2504673, are all marked as ‘Need for the gNB to know if the feature is supported’

Proposal 11: RAN4 requirements for generalization must be studied at the granularity of the defined NR_AI/ML_air Feature Groups and, including one or more of their defined components and parameters.

Proposal 12: RAN4 should not create a mixed dataset for generalization testing since the objective is not to test the AI/ML model.

- References

- RP-250792, Revised WID: Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.