3GPP TSG-RAN WG4 Meeting #116	R4-2509930

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on General Aspects of AI/ML for NR Air Interface Phase 2

Agenda item:	7.18.2

Document for:	Discussion

Introduction

In RAN#107, the revised SID of the Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2 was agreed [1]. In this revised SID, the following scope was agreed for RAN4:

- Testability and interoperability [RAN4]: - Further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift - RAN5 aspects related to testability and interoperability to be addressed on a request basis

In this contribution, we share our views on the following topics for two-sided models:

- LCM-related aspects, including relation to LCM of UE-sided models and performance monitoring

- Generalization/scalability aspects

- Post-deployment testing

Discussion

LCM-related aspects for two-sided models

Relation to LCM of UE-sided models

The LCM-related aspects for UE-sided use cases are being addressed in RAN4 under both the General Aspects and the use case-specific discussions. For instance, delay requirements for AI/ML functionality/configuration activation and deactivation are discussed as part of the LCM requirements. These discussions rely heavily on the LCM definitions and procedures agreed by RAN1/RAN2. While waiting for more input from RAN1/RAN2 regarding the LCM framework for two-sided models in the Rel-20 WI, RAN4 can initiate the two-sided model LCM discussions by answering the questions of whether and how to reuse the UE-sided model LCM requirements for the two-sided use cases (mainly CSI compression in the Rel-20 WI).

RAN4 to consider leveraging the LCM requirements being defined for the UE-sided use cases and using the Rel-19 NR-AI/ML-Air LCM conclusions as a baseline in the LCM discussions for the 2-sided models.

Performance monitoring in the CSI compression use case

We quote below the agreements related to performance monitoring in CSI compression from several RAN1 meetings.

RAN1#117 Agreement Further study following monitoring options in Rel-19, including the necessity and feasibility, - NW-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the target CSI reported by the UE via legacy eT2 codebook or eT2-like high-resolution codebook (Case 1) - SRS-based monitoring - UE-side monitoring, considering overhead, latency, complexity, monitoring accuracy, UE capability - Based on the output of the CSI reconstruction model at the UE (Case 2-1) - Note: CSI reconstruction model at the UE-side can be the same as the actual CSI reconstruction model used at the NW-side, a reference model provided by NW, or a proxy model developed by the UE side. - Via direct estimation of intermediate KPI (e.g., SGCS) without reconstructing a target CSI (Case 2-2) - Via estimation of monitoring output other than intermediate KPI without reconstructing a target CSI - Based on precoded RS (e.g., CSI-RS, DMRS) transmitted from NW based on the output of the CSI reconstruction model - Based on the output of the CSI reconstruction model indicated by the NW via legacy eT2 codebook or eT2-like high-resolution codebook Regarding monitoring metrics: - Monitoring accuracy also includes generalization considerations, if applicable. - Complexity also includes LCM complexity, if applicable. - Monitoring overhead, latency, complexity, and accuracy analysis may have to consider using at N>1 CSI feedback occasions. - Testability of UE reported metrics Discussion may include the following aspects: - Consideration of Options 1-5 and their sub-options for alleviating / resolving the issues related to inter-vendor training collaboration - Temporal domain aspects of CSI compression - How the above monitoring approaches or combination of them may help identifying the cause (e.g., NW side, UE side, data drift) of the performance degradation Note: for UE-side monitoring, the final reported monitoring output, if specified, may be different, e.g., be further derived based on the output of the above approaches. Note: implementation-based monitoring solutions can be considered in assessing the necessity of the above monitoring approaches. RAN1#120bis Conclusion For NW-side monitoring with target CSI reporting - Target CSI reporting via legacy CSI codebooks can be used for NW-side monitoring - Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side monitoring with additional cost of complexity and overhead at UE side. RAN1#120bis Conclusion UE-side monitoring is feasible. RAN1#120bis Observation - Some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports. RAN1#121 Agreement For NW-side performance monitoring, RAN1 concludes that target CSI reporting via legacy codebooks can be used. Target CSI reporting with CSI codebook enhancement via higher-resolution parameter combination may be beneficial for improving NW-side performance monitoring with additional cost of complexity and overhead at UE side. For UE-side side monitoring, RAN1 concludes that it is feasible, but some companies think that, at least in some UE-side monitoring options, NW-side monitoring with target CSI reporting is needed to check the reliability of UE-side monitoring reports.

- According to the above agreements, for NW-side performance monitoring with target CSI reporting, UE can report target CSI via legacy CSI codebooks. This means that UE is supposed to report the codebook-based CSI report as in the legacy scenario. Hence, the requirements on such reports should not differ from legacy requirements.

In case of NW-side performance monitoring with target CSI reporting in the two-sided CSI compression use case, no new requirements are needed for performance monitoring reporting.

In case of UE-side performance monitoring, UE is expected to report the performance monitoring metric(s) (e.g., a KPI computed based on the output of the CSI reconstruction model, an intermediate KPI computed without reconstructing a target CSI). Such reports are different from legacy PMI reports, hence, RAN4 should consider defining accuracy and delay requirements for performance monitoring reporting from the UE in this case using input from RAN1 (i.e., exact definition of the performance monitoring metric(s)).

In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 to use input from RAN1 regarding the performance monitoring metric(s) to be reported by the UE to decide whether to define accuracy and delay requirements for performance monitoring reporting.

Generalization/scalability aspects for two-sided models

The following agreement was made in RAN4#115 meeting regarding generalization testing for UE-sided use cases.

RAN4#115 Agreement Potential areas to consider for generalization testing: - gNB array parameters - port layouts, array size, antenna virtualization - propagation conditions - Deployment scenarios - Carrier frequencies - Speeds - Indoor/outdoor - Bandwidths - SNR

The Rel-18 generalization evaluations for CSI compression considered generalization over deployment scenarios (e.g., UMi, UMa), indoor/outdoor UE distributions (e.g., 80%/20%, 0%/100%), carrier frequencies (e.g., 3.5/4GHz, 2GHz), and TxRU mappings (e.g., [2,8,2], [4,4,2]). For the generalization testing in the two-sided CSI compression use case, these parameters can be considered, at least at the first stage of the discussion.

RAN4 to consider the following parameters for the generalization testing in the two-sided CSI compression use case:

- Deployment scenarios

- Indoor/outdoor UE distributions

- Carrier frequencies

- TxRU mappings

According to the RAN1 agreement stated below, the Rel-19 scalability evaluations for CSI compression consider scalability over CSI payload sizes, bandwidths and subband sizes, and Tx port numbers.

RAN1#120 Agreement For model structure scalability study for temporal domain Case 0, for the choice of {number of Tx ports, CSI feedback payload size, bandwidth}, take the following as baseline values. - Number of Tx ports - [N1, N2, P] = [2, 8, 2] (32 CSI-RS ports) - [N1, N2, P] = [4,4,2] (32 CSI-RS ports) - [N1, N2, P] = [2, 4, 2] (16 CSI-RS ports) - CSI feedback payload size - Multiple values, encouraged to pick at least one value from each of small, medium, and large payload regions X, Y, and Z. - Bandwidth & subband size - All subbands - A subset of all the subbands.

As the scalability concept has not been discussed yet in the context of RAN4 testing of AI/ML use cases, we propose to bring this aspect into consideration for the two-sided use cases.

RAN4 to discuss the need for scalability testing in the two-sided CSI compression use case (e.g., in terms of CSI payload sizes, bandwidths and subband sizes, and Tx port numbers).

Post-deployment testing for two-sided models

Two options for post-deployment testing were discussed as part of the Rel-19 NR_AIML_Air general aspects:

- Option 1: Post-deployment pre-activation functionality update testing

- Option 2: Post-deployment post-activation functionality testing based on performance monitoring

Option 1 ensures that the AI/ML functionality/configuration can be tested before it is activated by the NW for inference and reporting. On the other hand, Option 2 utilizes the performance monitoring at the NW-side to detect performance changes of the already activated AI/ML functionality/configuration.

In the RAN4#115 meeting, there was an agreement to prioritize Option 2 for the post-deployment testing in UE-sided use cases.

RAN4#115 Agreement Option 2 (post-deployment post-activation functionality testing based on performance monitoring) is prioritized. - it should be further investigated/established that reliable and testable monitoring procedures can be established for all use cases

Although the sub-bullet point in the above agreement used the term “for all use cases”, it is unclear whether Option 2 can be also prioritized for the two-sided use cases, at least based on this agreement. Hence, we propose to clarify this, for example by clearly stating that Option 2 is also prioritized for post-deployment testing of two-sided models.

RAN4 to clarify whether the agreement about post-deployment testing Option 2 prioritization (post-deployment post-activation functionality testing based on performance monitoring) is valid for the two-sided use cases also.

Conclusion

The following proposals were made in this contribution:

Proposal 1: RAN4 to consider leveraging the LCM requirements being defined for the UE-sided use cases and using the Rel-19 NR-AI/ML-Air LCM conclusions as a baseline in the LCM discussions for the 2-sided models.

Proposal 2: In case of NW-side performance monitoring with target CSI reporting in the two-sided CSI compression use case, no new requirements are needed for performance monitoring reporting.

Proposal 3: In case of UE-side performance monitoring in the two-sided CSI compression use case, RAN4 to use input from RAN1 regarding the performance monitoring metric(s) to be reported by the UE to decide whether to define accuracy and delay requirements for performance monitoring reporting.

Proposal 4: RAN4 to consider the following parameters for the generalization testing in the two-sided CSI compression use case:

	Deployment scenarios

	Indoor/outdoor UE distributions

	Carrier frequencies

	TxRU mappings

Proposal 5: RAN4 to discuss the need for scalability testing in the two-sided CSI compression use case (e.g., in terms of CSI payload sizes, bandwidths and subband sizes, and Tx port numbers).

Proposal 6: RAN4 to clarify whether the agreement about post-deployment testing Option 2 prioritization (post-deployment post-activation functionality testing based on performance monitoring) is valid for the two-sided use cases also.

- References

- RP-250308, “Revised SID: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface Phase 2”, Qualcomm, RAN#107, March 12-14, 2025, Incheon, South Korea.