3GPP TSG-RAN WG4 Meeting #116		               			        R4-2509937

Bengaluru, India, August 25th – 29th, 2025

Source:	Sony

Title:	RF requirements of ambient IoT device 1

Agenda item:	7.24.3

Document for:	Approval

- Background

The RF requirements of low-power wake-up signal (LP-WUS) and low-power wake-up receiver (LP-WUR) for NR were further discussed in RAN4#114bis [1], and a WF was agreed [2]. In this contribution, we further discuss the REFSENS and other RF requirements of LP-WUR based on our LLS results.

- Applicability of RF requirement

RAN4 firstly agreed that defining two types of RF requirements for LP-WUR with the NF+ IM as below would be defined in RAN4#114:

- Set 1: LP-WUR IM+NF = 18.0 dB

- Set 2: LP-WUR IM+NF = 13.5 dB

Moreover, it has been further agreed in RAN4#115 that SNR=-4.5dB for envelope detection and OFDM-based receiver.

Therefore, based on those two agreements, the REFSENS requirement for the two types of LP-WUR can be calculated, where the corresponding REFSENS value would be:

- Set 1: RFSENS = -94.5 dBm

- Set 2: RFSENS = -99 dBm

The key remaining issue is the applicability of the two sets of requirements. Set 1 and set 2 NFs are based on the assumption of OOK receiver and OFDM receiver, respectively. An OOK receiver can only decode OOK waveform, while an OFDM receiver can decode both OFDM and OOK waveforms, and therefore, it is reasonable that the LP-WUR types are not mapped with the waveform in a 1-1 manner. However, to clarify the applicability, one approach from the RAN4 perspective could be based on the capability of decoding different waveforms or RMC, e.g., define LP-WUR type 1 if it only supports an OOK-based waveform with more relaxed RF requirements, while LP-WUR type 2 supports both OOK and OFDM waveforms with more stringent RF requirements.

Proposal 1: RAN4 agrees with the following principle regarding requirement applicability: LP-WUR type 1 only supports an OOK-based waveform with more relaxed RF requirements, and LP-WUR type 2 supports both OOK and OFDM waveforms with more stringent RF requirements.

|  | OOK waveform | OFDM waveform |
| --- | --- | --- |
| LP-WUR Type 1 (relaxed requirements) |  |  |
| LP-WUR Type 2 (stringent requirements) |  |  |

Based on the principle mentioned above and to avoid further fragmentation, it would be beneficial if RAN4 could associate the LP-WUR type with RAN1 UE feature groups (FG) in RAN4 specifications. Although the UE FG discussion is still ongoing, based on some discussion so far, RAN4 can consider associating the RAN4-defined LP-WUR type with RAN1 UE FG.

Proposal 2: The RAN4-defined LP-WUR type shall be associated with the RAN1-defined UE feature groups to avoid further fragmentation of the LP-WUR implementation.

| RAN4 LP-WUR type | RAN1 UE FG for LP-WUS |
| --- | --- |
| Type 1 | 62-1 LP-WUS operation in IDLE/INACTIVE mode based on OOK signal |
| Type 2 | 62-1a LP-WUS operation in IDLE/INACTIVE mode based on OFDM overlaid sequence |
| Type 1 | 62-2 LP-WUS operation in CONNECTED mode based on OOK signal |
| Type 2 | 62-2a LP-WUS operation in CONNECTED mode based on OFDM overlaid sequence |

- ACS requirement consideration

ACS requirement is another issue with significant diversity of views between companies at this stage. In general, RAN4 has agreed that the blocking signal level for LR should be the same as MR. However, the wanted signal level for LP_WUS and whether a guard band should be assigned are still pending discussion.

In our view, from the test perspective, as RAN4 has agreed that Verify LP-WUR based on 1% MDR of LP-WUS which can be tested based on UE’s response to the NW/TE upon successfully detecting the LP-WUS (e.g. ACK/NACK in CONNECTED state or MSG1/3 in IDLE state or other methods) in RAN#115. This means that as long as the UE can response to the network correctly, it does not matter which hardware UE would use in the test. Therefore, in our view, there is no risk for UE to fail the ACS test as long as the wanted signal level is not lower than MR, since the UE can fallback to the MR if the LR failed. This is also the correct UE behavior in the field, as per our understanding, where the UE will fallback to the MR when its LP-WUR fails to detect LP-WUS.

Observation 1: UE is allowed to fallback to MR when it fails to detect LP-WUS, therefore, there is no risk for UE to fail the ACS test as long as the wanted signal level is no lower MR.

Although a low-complexity LP_WUR with reduced ACS performance may provide an instant power saving gain compared to a more advanced LP-WUR design. However, if it frequently suffers from the jammer in the field, the average power consumption may be diluted and could be even worse since it will fall back to the MR all the time.

Observation 2: A low-performance LP-WUR may provide better instant power saving gain, but if it needs to fall back to the MR more often, the average power consumption may increase.

Therefore, with the considerations above, we suggest RAN4 define the ACS wanted signal level as LP-WUS REFSENS+ 14 dB (same delta as MR) as a balance design. For LP-WUR with even lower performance, it can fallback to MR in the test to meet the requirement.

Proposal 3: RAN4 defines the ACS wanted signal level as LP-WUS+ 14 dB (same delta as MR). For LP-WUR with even lower ACS performance, it can fallback to MR in the test to meet the requirement.

In addition, from the network perspective, it is also meaningful to understand whether a sufficient guard band allows the UE to consistently detect LP-WUS under the same interference and signal conditions as MR. This would apparently guide network deployment based on the frequency and location of WUS, provided a wider spectrum is available from the operator. Therefore, it is proposed that RAN4 define a test case for ACS, where the interference and wanted signal levels are the same as MR, with a sufficiently large guard band placed between the interference signal and the wanted signal.

Proposal 4: RAN4 defines a separate test case for ACS, where the interference and wanted signal levels are the same as MR, with a sufficiently large guard band placed between the interference signal and the wanted signal.

- Bandwidth applicability

LP-WUR could be particularly useful for IoT type applications, which goes to deep sleep mode more often and get the device to be woke up quicker if the device is in an eDRX cycle. For NR device, such type of applications could be supported by (e)Redcap devices with small CBW, e.g., 3MHz in the coming release. Therefore, it is proposed that LP-WUS must be supported also in 3MHz CBW with applicable numerology combinations.

Proposal 5: LP-WUS must also be supported in 3MHz CBW with applicable numerology combinations.

- Conclusion

In this contribution, we have discussed LP-WUS and LP-WUR for NR. The following observations and proposals have been made:

Observation 1: UE is allowed to fallback to MR when it fails to detect LP-WUS, therefore, there is no risk for UE to fail the ACS test as long as the wanted signal level is no lower MR.

Observation 2: A low-performance LP-WUR may provide better instant power saving gain, but if it needs to fall back to the MR more often, the average power consumption may increase.

Proposal 1: RAN4 agrees with the following principle regarding requirement applicability: LP-WUR type 1 only supports an OOK-based waveform with more relaxed RF requirements, and LP-WUR type 2 supports both OOK and OFDM waveforms with more stringent RF requirements.

Proposal 2: The RAN4-defined LP-WUR type shall be associated with the RAN1-defined UE feature groups to avoid further fragmentation of the LP-WUR implementation.

Proposal 3: RAN4 defines the ACS wanted signal level as LP-WUS+ 14 dB (same delta as MR). For LP-WUR with even lower ACS performance, it can fallback to MR in the test to meet the requirement.

Proposal 4: RAN4 defines a separate test case for ACS, where the interference and wanted signal levels are the same as MR, with a sufficiently large guard band placed between the interference signal and the wanted signal.

Proposal 5: LP-WUS must also be supported in 3MHz CBW with applicable numerology combinations.

- References

R4-2504689		Topic summary for [114bis][134] NR_LPWUS_UERF	Moderator(Vivo)

R4-2505234		WF on UE RF requirements for LP-WUS	Vivo