3GPP TSG-RAN WG4 Meeting #116		               R4-2509938

Bengaluru, India, August 25th – 29th, 2025

Source:	Sony

Title:	Phase continuity for OCC and GNSS IDC issue

Agenda item:	7.27.3.2

Document for:	Approval

# Background

The phase continuity requirement for OCC grouping was discussed in RAN#115, where the following agreement has been made:

- As a starting point, reuse DMRS-bundling requirement values and side conditions

  - Add “No UE specific and common TA updates” to side conditions

  - FFS for further updates based on inputs in next meeting

- UE needs to be able to indicate OCC2/4 feature and/or OCC duration support for GSO deployment scenario only, for NGSO deployment scenario only, or for both GSO and NGSO deployment scenario

  - Further check and optimize alignment with RAN1 capabilities

# Phase continuity for OCC grouping

In the coming LS from RAN1, it states:

RAN1 assumes that the UE is required to maintain phase continuity and power consistency for the duration of one OCC group with PUSCH.

•	FFS: under which conditions the above applies, e.g. under the same conditions that phase continuity applies for DMRS bundling.

action

RAN1 respectfully ask RAN4 whether the same phase continuity requirements as for DMRS bundling can be applied for OCC length 2 and/or OCC length 4 under the same conditions as for DMRS bundling, or if new requirements are needed.

RAN4 requirements on DMRS bundling for slot < 8 is defined as the phase difference between slot p-1 and slot p is below 25 degrees, as shown below:

Therefore, RAN4 requirements can guarantee the phase variation of each UE individually within a certain range. However, with OCC grouping, to ensure the paired UE can actually maintain orthogonality with each other, we understand that the phase difference between the paired UE is a more critical issue. Taking OCC-4 as an example, presumably the network could group with 2 UEs with the same initial phase, e.g., phase difference is 0 degrees on the first slot. In the worst case, the phase difference on the last slot can be up to 150 degrees, and whether such a phase difference is acceptable requires further study.

Observation 1: Unlike DMRS bundling, the phase difference between paired UE is more critical for OCC grouping than the phase variation of each UE. The phase difference between the grouped UE can be up to 150 degrees, which is the worst-case scenario under current DMRS bundling requirements.

A preliminary simulation result with a simplified assumption is presented in Fig. 1 to verify if the existing DMRS bundling requirement would be sufficient for OCC-4. In this simulation, the phase variation between slot p-1 and slot p is assumed to be uniformly distributed from -φ to – φ, but there is no phase variation within each slot. It is also assumed that the initial phase of both UEs is 0, and the receiver knows the channel.

Figure 1 . The performance with respect to different phase variations between slot p and slot p+1 with OCC 4.

This simulation result shows that the SNR degradation for BER = 1% is below 1 dB for phase variation below 40°. From this result, the existing DMRS bundling requirement should be sufficient. Therefore, RAN4 can take existing phase continuity requirements as a starting point and define the requirement for UE support OCC. Meanwhile, the same applicability of phase continuity requirements as for DMRS bundling shall still be applied.

Observation 2: The existing DMRS bundling requirement on the phase difference between slot p-1 and slot p with 25 degrees is sufficient to ensure the OCC performance with grouped UE.

Proposal 1: RAN4 takes the existing phase continuity requirement to define the phase continuity requirement for UE support of OCC.

On the side condition, RAN4 has defined the following list for NR NTN DMRS bundling as in 38.101-5:

-RB allocation in terms of length and frequency position does not change, and intra-slot and inter-slot frequency hopping is not activated.

-Modulation order does not change.

-No network commanded TA takes effect.

-The TPMI precoder does not change.

-There is no change in UE transmission power level, and no change in the level of P-MPR applied by the UE.

-UE is not scheduled with uplink transmission of other physical channel/signal in-between the PUSCH or PUCCH transmissions.

-Doppler conditions are set to zero and delay conditions are set to constant.

It is necessary to reuse those conditions for the DMRS phase continuity requirement for OCC grouping. However, it is unclear if any new side condition, “No UE-specific and common TA updates,” should be added to the list. To our understanding, the network is not aware when and how the UE adjusts its UE-specific TA, which may lead to a situation where the network is not aware that the UE has lost phase continuity. On the other hand, it is understandable that the phase continuity may not be maintained under a TA change. Therefore, RAN4 may need to be careful if such a side condition should be added.

Observation 3: Although the TA adjustment will affect phase continuity, the loss of phase continuity over UE-specific TA adjustments may result in the network being unaware that the UE has lost its phase continuity.

Moreover, such a side condition did not exist in the earlier release for DMRS bundling, and thus, it is unclear if such a side condition would be necessary to be introduced for OCC operation.

Proposal 2: RAN4 needs to discuss further if “No UE specific and common TA updates” should be added as a side condition.

# GNSS IDC

In the LS replied from RAN2, it states [1]:

RAN2 respectfully asks RAN4 to take the aforementioned RAN2 decisions into account and provide the corresponding requirements, e.g., the periodicity and duration of IDC free time to help the UE read the GNSS.

Meanwhile, the current IDC-TDM solution is specified as below in 38.331:

Based on the information above, it is our understanding that RAN2 requires information on how often the GNSS measurement event would happen and how long the measurement gap would require.

GNSS validity duration was specified for IoT NTN since Rel-17, a UE autonomously reported value indicating the remaining GNSS validity duration in the UE.

In our view, the large range of validity duration specified for IoT NTN is also valid for NR NTN, as it can accommodate scenarios under different speeds, etc. Therefore, it is proposed that those numbers of NR NTN be reused.

Proposal 3: The GNSS validity duration values of IoT NTN in TS 36.331 can be reused for NR NTN.

Measurement gap on GNSS has been introduced for IoT NTN since Rel-17, which ranges up to 31s (see table below). Per our understanding, such a long measurement gap was defined to accommodate a cold start of the GNSS receiver with IoT NTN devices, as the GNSS receiver will be totally shut off during the NTN transmission for the sake of power consumption. However, it is unclear if such a limitation would also be valid for NR NTN. For a normal measurement gap without a cold start, a measurement gap of a few seconds would be sufficient.

Proposal 4: A measurement gap of a few seconds would be sufficient if the GNSS receiver is not cold-started.

# Conclusion

In this paper, we presented our view on the phase continuity for OCC grouping and GNSS IDC issue, where the following observations and proposals are made:

Observation 1: Unlike DMRS bundling, the phase difference between paired UE is more critical for OCC grouping than the phase variation of each UE. The phase difference between the grouped UE can be up to 150 degrees, which is the worst-case scenario under current DMRS bundling requirements.’

Observation 2: The existing DMRS bundling requirement on the phase difference between slot p-1 and slot p with 25 degrees is sufficient to ensure the OCC performance with grouped UE.

Observation 3: Although the TA adjustment will affect phase continuity, the loss of phase continuity over UE-specific TA adjustments may result in the network being unaware that the UE has lost its phase continuity.

Proposal 1: RAN4 takes the existing phase continuity requirement to define the phase continuity requirement for UE support of OCC.

Proposal 2: RAN4 needs to discuss further if “No UE specific and common TA updates” should be added as a side condition.

Proposal 3: The GNSS validity duration values of IoT NTN in TS 36.331 can be reused for NR NTN.

Proposal 4: A measurement gap of a few seconds would be sufficient if the GNSS receiver is not cold-started.

# References

- R2-2501568	Response LS on simultaneous operation between GNSS and NR NTN

- R4-2505305 	LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3