3GPP TSG-RAN WG4 Meeting #116						       R4-2509939

Bengaluru, India, August 25th – 29th, 2025

Source:	Sony

Title:	OCC scheme on the RF requirements for IoT NTN

Agenda item:	7.28.3

Document for:	Discussion

# Background

In RAN4#113, the impact of OCC scheme on IoT NTN RF requirements has been further discussed [1], and in particular, it has been agreed [2]:

#### Issue 2-1: Whether UE is required to maintain phase continuity and power consistency for the duration of OCC group.

- Agreement:

  - FFS on UE RF requirements and whether related UE capabilities would be needed.

In this contribution, we further discuss on the need of RF requirements for phase continuity for OCC group.

# UE RF impact to support CFO grouping

Regarding the CFO grouping, an LS on NR NTN OCC was sent from RAN1[3], which indicates that phase continuity would be required to support CFO grouping with OCC-2 and OCC-4. Although the LS was not intended for IoT NTN, at least for slot OCC2 with SCS = 15 kHz, we expect to see a similar impact on CFO grouping as for NR NTN.

Without phase continuity, ensuring the phase change between grouped UEs is within the tolerance range becomes challenging, particularly since the phase of the paired UE may drift towards opposite directions. Therefore, it is proposed that the phase continuity requirements for CFO grouping be defined. In our analysis for NR NTN, we showed that the current phase continuity requirement with 25 degrees between slot p-1 and slot p would be sufficient [4].

The DMRS bundling requirements defined for NR NTN can be taken as a starting point, where a 25-degree phase difference between slot p and slot p+1 can be used. Meanwhile, the event that may potentially break the phase continuity shall also be defined.

Proposal 1: Take the DMRS bundling requirement for NR NTN to define the phase continuity requirements for CFO grouping of IoT NTN with slot-level OCC.

# Conclusion

In this contribution, we make the following observations and conclusions:

Proposal 1: Take the DMRS bundling requirement for NR NTN to define the phase continuity requirements for CFO grouping of IoT NTN with slot-level OCC.

# References

- R4-2507847	Topic summary for [115][315] IoT_NTN_Ph3	Moderator (MediaTek)

- R4-2508767	Way Forward for [115][315] IoT_NTN_Ph3		MediaTek

- R4-2505305 	LS on requirements for the phase continuity and power consistency for OCC with PUSCH in NR NTN Ph3

- R4-2507131	Discussion on the phase continuity and GNSS issue for NR NTN