3GPP TSG-RAN WG4 Meeting #116							         	       	R4-2509943

# Bengaluru, India, 25th – 29th August, 2025

Title: 	Proposal on OOB range and RB extension principles for MPR reduction

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.2.2.1	Power domain enhancements for single carrier

Document for:	Approval

# 1	Background

In RAN#115, we provided in [1] our views on how to determine bandwidth (OOB region) and RB extensions for ratios of ½ and ¼. A way forward [2] captured agreements on the topic in [2]. In this contribution, we provide measured verification of our bandwidth and RB extension principles and its impacts on affected Tx requirements for all the applicable allocations.

# Discussion

## 2.1 Way forward input

The way forward [2] lists the agreed options and open items for MPR reduction with bandwidth and RB extension. The related part is copied below in italics.

Issue 1-1-1: Extension values/ratio: Agreement in main session:

- At least ½ and ¼ are introduced as the extension ratio to apply to both single-sided and dual-sided cases

- It is FFS on the details of symmetric and asymmetric cases for dual-sided case where non-zero extension is applied for both sides.

- It is FFS on details of the single side case.

Issue 1-1-2: Guard band for extended CBW: WF

- GB for extended CBW is a virtual concept to consider the boundary applying the shifted OOBE, which is not reflected in the specification and it can be calculated by extended CBW, UE CBW, NRB and extended NRB

- Extended CBW is not limited to the existing UE CBW

- RAN4 studies whether the extended BW emissions can be guaranteed by only measuring the existing CBW supported in the band (close to the extended BW) given the relaxed extended GB and preliminary measurements performed suggests

Issue 1-1-3: Derivation of new inner RB region in extended CBW: WF

The following approach is considered as starting point to be further discussed in next meeting, notation should be further considered as well

- For NRB based RB_start coordinate system origin, the formula of new inner region can be changed as below:

  - RBStart,Low_new = max(0, RBStart,Low – RBshift_low);

  - RBStart,High_new = min(NRB – LCRB, NRB + RBshift_high – RBStart,Low_new – LCRB);

  - RBStart,Low_new  ≤  RBStart  ≤  RBStart,High_new, and LCRB  ≤  ceil((NRB+ RBshift_low + RBshift_high)/2);

  - RBshift_low, RBshift_high ≤ NRB/2

- Other approach is not precluded

- The approach shall be generic to cover all possible extension cases, including symmetric, asymmetric and different extension ratios

Issue 1-2-1: Derivation of the extended IBE: Agreement in main session:

- Fixed IBE used for extended CBW at each side of UE CBW as the baseline, the value is derived from the outmost allocated RB of UE CBW (illustrated in a figure to be developed)

  - The details and the related test cases can be discussed and decided later.

  - If significant issue is identified, other options can be further considered.

Issue 1-2-1: Derivation of the extended IBE: WF

Further discuss whether to apply the extended IBE in the GB of extended CBW.

Issue 1-2-2: Relaxation of the extended IBE

- Options

  - Option 1: using BPSK EVM in the IBE formula for all extended RBs

  - Option 2: no relaxation is needed.

- WF

  - If there is no obvious system performance impact observed, option 1 is adopted

Issue 1-3-1: NW signaling: Agreement in main session:

- The power enhancement feature is enabled by the gNB RRC signalling according to UE capabilities,

  - The details of the signaling including the contents are FFS

Issue 1-3-1: NW signaling: WF

- It is up to the NW to judge the gap width on both sides of the UE

- NW sends info of extension ratio (at least 1/2) per side

Issue 1-3-2: UE capability: Agreement in main session:

- No separate UE capabilities for scenario 1 and scenario 2

- Capability 1: Indicates whether UE supports MPR reduction for single UL carrier with 1/2*UE CBW extension only

  - Both dual-sided and single sided cases are supported.

- Capability 2: Indicates whether UE supports MPR reduction for single UL carrier with both ½ and 1/4*UE CBW extension

  - Both dual-sided symmetric and single sided cases are supported.

  - For dual-sided case, it is FFS if asymmetric cases and a separated capability is introduced.

- Note: the other ratio number depends on discussion of issue 1-1-1

As it can be observed, although there was good progress on the out-of-band and in-band emissions and capability for extension ratios, there is still several details to be nailed down which we address in this contribution by clarifying the extension principles, their impact on requirements, and their verification via a set of measurements and the capability definitions.

## 2.2 Extension ratio principles and impact to requirements

Rather than discussing about bandwidth extension or RB extension, we prefer to design the reduced MPR capabilities on the concept of extension ratio and its consequence on:

- RB extension which drives the new IBE requirements and modify the inner/outer allocations to allow reduced MPR

- OOB emission offset which drive the new ACLR, SEM and spurious emissions requirements.

We have already agreed extension ratios ½ or ¼ on one side or each side, we thus have:

- A lower and upper extension ratios of ½ for capability 1

- Additional lower and upper extension ratios of ¼ for capability 2

- The UE supporting the MPR reduction capability will be essentially signalled the lower and upper extension ratio couplet by the BS which we denominate {ExtLow,ExtHigh} in accordance to its capability

- For UE not supporting MPR reduction or if BS does not signal a valid extension couplet, the default extension ratio {ExtLow,ExtHigh}couplet is {0,0}

- For UE capability 1 valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/2}, {1/2,0} and {1/2,1/2}

- For UE capability 2 additional valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/4}, {1/4,0}, {1/4,1/4}, [{1/4,1/2}, {1/2,1/4}]

- Capability 1 and 2 correspond to 8 valid {ExtLow,ExtHigh}couplets which can be signalled with 3 bits.

For UE supporting a extension ratio couplet {ExtLow,ExtHigh}:

- The following RB extensions apply:

- Lower RB extension is ceil(ExtLow*NRB)

- Upper RB extension is ceil(ExtHigh*NRB)

- With the NRB associate with the configured UE channel bandwidth and SCS in Table 5.3.2-1 of 38.101-1

- The following OOB emission limit offsets apply:

- Lower OOB emission frequency offset compared to the channel BW is - ExtLow* BWChannel

- Upper OOB emission frequency offset compared to the channel BW is ExtHigh* BWChannel

- Lower OOB emission frequency offset compared to the carrier centre is - BWChannel *(1/2+ExtLow)

- Upper OOB emission frequency offset compared to the carrier centre is BWChannel *(1/2+ExtHigh)

- With BWChannel corresponding to the configured UE channel bandwidth

- Using {0,0} as the default extension ratio couplet ensures that there is no RB extension or OOB emission frequency offset.

The RB extensions, OOB emission offsets for all the specified FR1 channel bandwidths are provided in Annex 1 in Chapter 5. It also covers a number of illustrations of the 20MHz CBW extension ratio cases.

Proposal on MPR reduction optional capabilities:

- For UE not supporting MPR reduction or if BS does not signal a valid extension couplet to a UE supporting MPR reduction capability, the default extension ratio {ExtLow,ExtHigh}couplet is {0,0}

- For UE capability 1 valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/2}, {1/2,0} and {1/2,1/2}

- For UE capability 2 additional valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}

- Capability 1 and 2 correspond to 8 valid {ExtLow,ExtHigh} couplets which can be signalled to the UE with 3 bits.

- The capability is applicable to single CC UL for all channel bandwidths except 3MHz and 7MHz.

- The capability is applicable to power class 3, 2 and 1.5 regardless of the number of Tx supported.

Proposal to Inner, Outer and Edge allocation definition in 38.101-1 section 6.2.2: the following changes are adopted:

- Note that the same Edge allocation text need to be added below Table 6.2D.2-5

************** Beginning of changes ***************

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + ceil(ExtLow * NRB )   ≤  RBStart,High + ceil(ExtLow * NRB ) + ceil(ExtHigh * NRB ), and

LCRB ≤ ceil(NRB/2) + ceil(ExtLow * NRB/2) + ceil(ExtHigh * NRB/2)

where ExtLow and ExtHigh are the lower and higher frequency extension ratios signalled to UEs supporting the MPR reduction capability for all channel bandwidths except 3 MHz and 7 MHz by the {ExtLow, ExtHigh} extension ratio couplet:

- For UEs not supporting the optional MPR reduction capability the default extension ratio couplet is {0,0}

- For UEs supporting the MPR reduction capability not receiving a valid extension ratio couplet, the extension ratio couplet is set to {0,0}

- For UE supporting “half extension” MPR reduction capability the valid {ExtLow,ExtHigh} extension ratio couplets are: {0,1/2}, {1/2,0}, {1/2,1/2}

- For UE additionally supporting “fourth extension” MPR reduction capability the valid additional {ExtLow,ExtHigh} extension ratio couplets are: {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel LCRB ≤ 2 RBs, except for PC1 UE supporting other bands than n14.

For UEs supporting the optional MPR reduction capability, Edge RB allocations are only defined for {ExtLow, ExtHigh} extension ratio couplet {0,1/2}, {1/2,0}, {0,1/4}, {1/4,0} at the lowermost or uppermost edge of the channel for ExtLow = 0 and ExtHigh = 0 respectively.

Any allocation that is not an Inner or Edge allocation is an Outer allocation.

************** End of changes ***************

Proposal to spectrum emission mask in 38.101-1 section 6.5.2.2 (no modification to additional emission requirements): the following changes are adopted:

************** Beginning of changes ***************

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) (NOTE 1) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |
| NOTE 1: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, ΔfOOB is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower and higher boundary respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |  |  |

************** End of changes ***************

Proposal to ACLR in 38-101-1 section 6.5.2.4.1 (no modification to UTRA ACLR): the following changes are adopted:

************** Beginning of changes ***************

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency with a spacing defined in Table 6.5.2.4.1-1.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

When the IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is set to 1 for a UE supporting the capability of powerBoosting-pi2BPSK-QPSK-r18 or capability of powerBoosting-pi2BPSK-QPSK-Modified-r18, for power class 2 UE, the ACLR requirement of PC2 applies. For power class 3 UE, the ACLR requirement of PC3 applies.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NR ACLR adjacent channel Spacing | (MHz) | Nominal channel spacing (Note 2) |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 2: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, the nominal channel spacing is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower and higher adjacent channel respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |

************** End of changes ***************

Proposal to Spurious emissions in 38-101-1 Table 6.5.3.1-1: the following changes are adopted:

************** Beginning of changes ***************

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 (NOTE 1) |
| NOTE 1: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, OOB boundary FOOB is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower frequency and higher range respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |

************** End of changes ***************

For IBE, it is agreed that the requirement is extended to the virtual and non-allocated extended RBs abutted on each side of the configured UE channel bandwidth RB allocation. For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}:

- The RB extension abutted to the lower edge of the configured UE channel bandwidth RB allocation is ExtLow*NRB

- The RB extension abutted to the higher edge of the configured UE channel bandwidth RB allocation is ExtHigh*NRB

It was also agreed that the IBE requirement will be a flat requirement based on the IBE required for the outermost RB allocation on each side. The only pending question is whether the formula is calculated assuming Pi/2-BPSK EVM.

Proposal to In-band emissions in 38-101-1 Table 6.4.2.3-1: the following changes are adopted: Extended RBs need to meet the IBE required for the outermost RB allocation on each side assuming Pi/2-BPSK EVM in the formula.

************** Beginning of changes ***************

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2, 12) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. NOTE 12: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, the In-band requirement is extended to an additional non-allocated RB range of ExtLow* NRB and Exthigh* NRB immediately adjacent to the lower and higher edge of the RB allocation range respectively. These additional lower and higher non-allocated RBs shall meet the general requirement of the lower and higher outmost UE valid RB allocation respectively, calculated with Pi/2-BPSK EVM. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |  |

************** End of changes ***************

## 2.2 Verification of extension ratio principles and requirements.

To verify the proposed extension ratio principles, we have measured a significant number of waveforms OOB emissions (ACLR/SEM/Spurious emissions for the worst-case proposed extensions ratios (asymmetric extension on the lower frequency side as this has the lowest guard band) for 5, 20, 50 and 100MHz channel bandwidths for PC3 and PC2.

The raw analysis for PC3 is presented in Annex section 6.1 and the analysis of the PC2 is presented in Annex section 6.2.

Observations:

- The results show that all the proposed extension ratios meet the expect emissions at the inner MPR value for all the related extended inner allocations with margin (denoted I with OK as pass/fail status).

- As expected, most of the redefined outer allocations (denoted O with KO as pass/fail status) fail emissions. However, a few of the redefined outer allocations (denoted O with OK+ as pass/fail status) pass emissions

- Except for IBE, which is not verified but is met with the inner allocation MPR, we can conclude that the proposed extension ratio principles can meet all the requirements with margin.

- Except for UEs that do not support all the mandatory CBW in a band it should be noted that the extended bandwidths for most the emission requirements is similar to an existing supported channel bandwidth which is already tested.

Conclusion: Since the performance is verified for PC3 and PC2 PAs and:

- PC1.5 2Tx use two PC2 PAs and its MPR accounts for reverse IMD

- PC2 2Tx PAs use two PC3 PAs and its MPR accounts for reverse IMD and 1dB extra ACLR requirement

- Similarly for 3Tx and 4Tx cases that combines PC3 and PC2

The proposed extension ratio principles are valid for PC3, PC2 and PC1.5 regardless of the number of Tx.

# Conclusion

In this contribution, we formalized the principles of extension ratios and their resulting impact to the affected Tx requirements and make the following proposals.

Proposal on MPR reduction optional capabilities:

- For UE not supporting MPR reduction or if BS does not signal a valid extension couplet to a UE supporting MPR reduction capability, the default extension ratio {ExtLow,ExtHigh}couplet is {0,0}

- For UE capability 1 valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/2}, {1/2,0} and {1/2,1/2}

- For UE capability 2 additional valid extension ratio {ExtLow,ExtHigh}couplets are: {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}

- Capability 1 and 2 correspond to 8 valid {ExtLow,ExtHigh} couplets which can be signalled to the UE with 3 bits.

- The capability is applicable to single CC UL for all channel bandwidths except 3MHz and 7MHz.

- The capability is applicable to power class 3, 2 and 1.5 regardless of the number of Tx supported.

Proposal to Inner, Outer and Edge allocation definition in 38.101-1 section 6.2.2: the following changes are adopted:

- Note that the same Edge allocation text need to be added below Table 6.2D.2-5

************** Beginning of changes ***************

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met

RBStart,Low  ≤  RBStart + ceil(ExtLow * NRB )   ≤  RBStart,High + ceil(ExtLow * NRB ) + ceil(ExtHigh * NRB ), and

LCRB ≤ ceil(NRB/2) + ceil(ExtLow * NRB/2) + ceil(ExtHigh * NRB/2)

where ExtLow and ExtHigh are the lower and higher frequency extension ratios signalled to UEs supporting the MPR reduction capability for all channel bandwidths except 3 MHz and 7 MHz by the {ExtLow, ExtHigh} extension ratio couplet:

- For UEs not supporting the optional MPR reduction capability the default extension ratio couplet is {0,0}

- For UEs supporting the MPR reduction capability not receiving a valid extension ratio couplet, the extension ratio couplet is set to {0,0}

- For UE supporting “half extension” MPR reduction capability the valid {ExtLow,ExtHigh} extension ratio couplets are: {0,1/2}, {1/2,0}, {1/2,1/2}

- For UE additionally supporting “fourth extension” MPR reduction capability the valid additional {ExtLow,ExtHigh} extension ratio couplets are: {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel LCRB ≤ 2 RBs, except for PC1 UE supporting other bands than n14.

For UEs supporting the optional MPR reduction capability, Edge RB allocations are only defined for {ExtLow, ExtHigh} extension ratio couplet {0,1/2}, {1/2,0}, {0,1/4}, {1/4,0} at the lowermost or uppermost edge of the channel for ExtLow = 0 and ExtHigh = 0 respectively.

Any allocation that is not an Inner or Edge allocation is an Outer allocation.

************** End of changes ***************

Proposal to spectrum emission mask in 38.101-1 section 6.5.2.2 (no modification to additional emission requirements): the following changes are adopted:

************** Beginning of changes ***************

The spectrum emission mask of the UE applies to frequencies (ΔfOOB) starting from the  edge of the assigned NR channel bandwidth. For frequencies offset greater than ΔfOOB, the spurious requirements in clause 6.5.3 are applicable.

NOTE:	For measurement conditions at the edge of each frequency range, the lowest frequency of the measurement position in each frequency range should be set at the lowest boundary of the frequency range plus MBW/2. The highest frequency of the measurement position in each frequency range should be set at the highest boundary of the frequency range minus MBW/2. MBW denotes the measurement bandwidth defined for the protected band.

The power of any UE emission shall not exceed the levels specified in Table 6.5.2.2-1 for the specified channel bandwidth.

Table 6.5.2.2-1: General NR spectrum emission mask

| ΔfOOB (MHz) (NOTE 1) | Channel bandwidth (MHz) / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth |
| --- | --- | --- | --- | --- | --- |
|  | 3 | 5 | 10, 15, 20, 25, 30, 35, 40, 45 | 50, 60, 70, 80, 90, 100 |  |
| ± 0-1 | -13 | -13 | -13 |  | 1 % of channel BW |
| ± 0-1 |  |  |  | -24 | 30 kHz |
| ± 1-5 | -10 | -10 | -10 |  | 1 MHz |
| ± 5-6 | -25 | -13 |  |  |  |
| ± 6-10 |  | -25 |  |  |  |
| ± 5-BWChannel |  |  | -13 |  |  |
| ± BWChannel-(BWChannel+5) |  |  | -25 |  |  |
| NOTE 1: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, ΔfOOB is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower and higher boundary respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |  |  |

************** End of changes ***************

Proposal to ACLR in 38-101-1 section 6.5.2.4.1 (no modification to UTRA ACLR): the following changes are adopted:

************** Beginning of changes ***************

NR Adjacent Channel Leakage power Ratio (NRACLR) is the ratio of the filtered mean power centred on the assigned NR channel frequency to the filtered mean power centred on an adjacent NR channel frequency with a spacing defined in Table 6.5.2.4.1-1.

The assigned NR channel power and adjacent NR channel power are measured with rectangular filters with measurement bandwidths specified in Table 6.5.2.4.1-1.

If the measured adjacent channel power is greater than –50 dBm then the NRACLR shall be higher than the value specified in Table 6.5.2.4.1-2.

When the IE powerBoostPi2BPSK-r18 or powerBoostQPSK-r18 is set to 1 for a UE supporting the capability of powerBoosting-pi2BPSK-QPSK-r18 or capability of powerBoosting-pi2BPSK-QPSK-Modified-r18, for power class 2 UE, the ACLR requirement of PC2 applies. For power class 3 UE, the ACLR requirement of PC3 applies.

Table 6.5.2.4.1-1: NR ACLR measurement bandwidth

| Channel bandwidth | (MHz) | 3,5,10,15,20,25,30,35,40,45,50 | 60,70,80,90,100 |
| --- | --- | --- | --- |
| REF_SCS | (kHz) | 15 | 30 |
| NR ACLR measurement bandwidth | (MHz) | MBW=REF_SCS*(12*NRB+1)/1000 |  |
| NR ACLR adjacent channel Spacing | (MHz) | Nominal channel spacing (Note 2) |  |
| NOTE : “NRB” in the formula is the maximum transmission bandwidth configuration as defined in Table 5.3.2-1. NOTE 2: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, the nominal channel spacing is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower and higher adjacent channel respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |

************** End of changes ***************

Proposal to Spurious emissions in 38-101-1 Table 6.5.3.1-1: the following changes are adopted:

************** Beginning of changes ***************

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30, 35, 40, 45, 50, 60, 70, 80, 90, 100 | BWChannel + 5 (NOTE 1) |
| NOTE 1: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, OOB boundary FOOB is modified by ExtLow* BWChannel and Exthigh* BWChannel for the lower frequency and higher range respectively. This capability is not supported for 3 MHz and 7 MHz channels. |  |

************** End of changes ***************

For IBE, it is agreed that the requirement is extended to the virtual and non-allocated extended RBs abutted on each side of the configured UE channel bandwidth RB allocation. For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}:

- The RB extension abutted to the lower edge of the configured UE channel bandwidth RB allocation is ExtLow*NRB

- The RB extension abutted to the higher edge of the configured UE channel bandwidth RB allocation is ExtHigh*NRB

It was also agreed that the IBE requirement will be a flat requirement based on the IBE required for the outermost RB allocation on each side. The only pending question is whether the formula is calculated assuming Pi/2-BPSK EVM.

Proposal to In-band emissions in 38-101-1 Table 6.4.2.3-1: the following changes are adopted: Extended RBs need to meet the IBE required for the outermost RB allocation on each side assuming Pi/2-BPSK EVM in the formula.

************** Beginning of changes ***************

Table 6.4.2.3-1: Requirements for in-band emissions

| Parameter description | Unit | Limit (NOTE 1) |  | Applicable Frequencies |
| --- | --- | --- | --- | --- |
| General | dB |  |  | Any non-allocated (NOTE 2, 12) |
| IQ Image | dB | -28 | Image frequencies when output power > 10 dBm | Image frequencies (NOTES 2, 3) |
|  |  | -25 | Image frequencies when output power ≤ 10 dBm |  |
| Carrier leakage | dBc | -28 | Output power > 10 dBm | Carrier leakage frequency (NOTES 4, 5) |
|  |  | -25 | 0 dBm ≤ Output power ≤ 10 dBm |  |
|  |  | -20 | -30 dBm ≤ Output power < 0 dBm |  |
|  |  | -10 | -40 dBm ≤ Output power < -30 dBm |  |
| NOTE 1: An in-band emissions combined limit is evaluated in each non-allocated RB. For each such RB, the minimum requirement is calculated as the higher of PRB - 30 dB and the power sum of all limit values (General, IQ Image or Carrier leakage) that apply. PRB is defined in NOTE 10. NOTE 2: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured average power per allocated RB, where the averaging is done across all allocated RBs. For pi/2 BPSK with Spectrum Shaping, the limit is expressed as a ratio of measured power in one non-allocated RB to the measured power in the allocated RB with highest PSD. NOTE 3: The applicable frequencies for this limit are those that are enclosed in the reflection of the allocated bandwidth, based on symmetry with respect to the carrier leakage frequency, but excluding any allocated RBs. NOTE 4: The measurement bandwidth is 1 RB and the limit is expressed as a ratio of measured power in one non-allocated RB to the measured total power in all allocated RBs. NOTE 5: The applicable frequencies for this limit depend on the parameter txDirectCurrentLocation in UplinkTxDirectCurrent IE, and are those that are enclosed either in the RB containing the carrier leakage frequency, or in the two RBs immediately adjacent to the carrier leakage frequency but excluding any allocated RB. NOTE 6: LCRB is the Transmission Bandwidth (see clause 5.3). NOTE 7: NRB is the Transmission Bandwidth Configuration (see clause 5.3). NOTE 8: EVM is the limit specified in Table 6.4.2.1-1 for the modulation format used in the allocated RBs. NOTE 9: is the starting frequency offset between the allocated RB and the measured non-allocated RB (e.g. ∆RB = 1 or ∆RB = -1 for the first adjacent RB outside of the allocated bandwidth. NOTE 10: is an average of the transmitted power over 10 sub-frames normalized by the number of allocated RBs, measured in dBm. NOTE 11: For almost contiguous allocations defined in clause 6.2.2, LCRB = NRB_alloc + NRB_gap with no in-gap emission requirement. NOTE 12: For UEs supporting the optional MPR reduction capability receiving a valid {ExtLow,ExtHigh} extension ratio couplet out of {0,1/2}, {1/2,0}, {1/2,1/2}, {0,1/4}, {1/4,0}, {1/4,1/4}, {1/4,1/2}, {1/2,1/4}, the In-band requirement is extended to an additional non-allocated RB range of ExtLow* NRB and Exthigh* NRB immediately adjacent to the lower and higher edge of the RB allocation range respectively. These additional lower and higher non-allocated RBs shall meet the general requirement of the lower and higher outmost UE valid RB allocation respectively, calculated with Pi/2-BPSK EVM. This capability is not supported for 3 MHz and 7 MHz channels. |  |  |  |  |

************** End of changes ***************

Conclusion: Since the performance is verified for PC3 and PC2 PAs and:

- PC1.5 2Tx use two PC2 PAs and its MPR accounts for reverse IMD

- PC2 2Tx PAs use two PC3 PAs and its MPR accounts for reverse IMD and 1dB extra ACLR requirement

- Similarly for 3Tx and 4Tx cases that combines PC3 and PC2

The proposed extension ratio principles are valid for PC3, PC2 and PC1.5 regardless of the number of Tx.

# References

[1] R4-2507742 RB and BW for 2x and 1.5x extension and optional smaller granularity extension, Skyworks Solutions Inc., RAN4#115

[2] R4-2507930 WF on NR_ENDC_RF_Ph4_part1,	Huawei, RAN4#115

[3] R4-2511577 Draft CR to R19 38.101-1 to introduce MPR reduction from bandwidth and RB extension ratios, Skyworks Solutions Inc., RAN4#115

# Annex 1: Illustration and parameters for all extension ratios.

Table 1: Extended OOB range offset, extended RBs and resulting guard-bands for all specified FR1 CBW

Figure 1: Illustration of the different extended RBs and OOB ranges for all the extension cases of 20MHz and comparison with existing CBW

Figure 2: 20MHz inner (green)/outer (orange)/edge (red) allocations regions for (left to right): initial CBW, {1/2,1/4} and {1/4,1/2} extensions

Figure 3: 20MHz inner (green)/outer (orange)/edge (red) allocations regions for (left to right): {0,1/2}, {1/2,0} and {1/2,1/2} extensions

Figure 4: 20MHz inner (green)/outer (orange)/edge (red) allocations regions for (left to right): {0,1/4}, {1/4,0} and {1/4,1/4} extensions

# Annex 2: Raw measurement data

The Tables Ax provides the RAW measured verification data per the Table A0 template for ACLR and SEM+Spurious emissions margins for:

- PC3 in chapter 6.1 and PC2 in chapter 6.2

- Channel bandwidth / SCS of: 5MHz/15kHz, 20MHz/15kHz, 50MHz/15kHz and 100MHz/30kHz

- CP-OFDM and DFT-s-OFDM for various allocations with their Inner/Outer definition as per the extension ratios. Allocations on the right side have been used most often as they have the lowest distance to where the emissions apply.

- Pass/fail for inner/outer

- Baseline extension ratio {0,0} applying MPR according to inner, outer definition

- Optional extension ratio {X,Y} with X and Y = 0, ½, ¼ applying MPR according to inner only

Table A 0: template for tables in Annex

| CBW/SCS |  |  | Channel Bandwidth and SCS |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Waveform | CP/DF |  | Waveform type (QPSK CP-OFDM/DFT-s-OFDM) |  |  |  |  |  |  |
|  | Alloc |  | Allocation LCRB/RB/RBstart |  |  |  |  |  |  |
| Pout w MPR |  |  | Measured output power (with MPR for {0,0} with inner MPR for others) |  |  |  |  |  |  |
| Extension {0,0} | I/O |  | Inner (I) or outer (O) tag as extension ratio |  |  |  |  |  |  |
|  | OK/KO |  | OK | inner pass = expected | OK+ | Outer pass = better | KO | outer fail= expected |  |
|  | ACLR | low | Lower ACLR margin |  |  |  |  |  | fail |
|  |  | high | Higher ACLR margin |  |  |  |  |  | marginal |
|  | SEM+ SPUR | low | Lower SEM+Spurious emissons margin |  |  |  |  |  | Small margin |
|  |  | high | Higher SEM+Spurious emissons margin |  |  |  |  |  | Large margin |

## 6.1 PC3 measurements for 5, 20, 50 and 100MHz CBW

Table A 1: 5MHz CBW with 15kHz SCS PC3 verification versus extension ratios

Table A 2: 20MHz CBW with 15kHz SCS PC3 verification versus extension ratios

Table A 3: 50MHz CBW with 15kHz SCS PC3 verification versus extension ratios

Table A 4: 100MHz CBW with 30kHz SCS PC3 verification versus extension ratios

## 6.2 PC2 measurements for 5, 20, 50 and 100MHz CBW

Table A 5: 5MHz CBW with 15kHz SCS PC2 verification versus extension ratios

Table A 6: 20MHz CBW with 15kHz SCS PC2 verification versus extension ratios

Table A 7: 50MHz CBW with 15kHz SCS PC2 verification versus extension ratios

Table A 8: 100MHz CBW with 30kHz SCS PC2 verification versus extension ratios