3GPP TSG-RAN WG4 Meeting #116	R4-2509951

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM core requirements for SSB adaptation

Agenda item:	7.23.3.2

Document for:	Discussion

Introduction

The discussion on SSB adaptation as part of the third objective “adaptation of common signal/channel transmissions” of the Rel-19 WID on Enhancements of network energy savings for NR [1], was continued at RAN4#115 meeting [2] and the agreements were captured in the WF [3]. In this contribution, we continue the discussion on remaining open issues related to RRM core requirements for SSB adaptation.

Discussion

  - Progress status in RAN4 and RAN1

At RAN4#115 meeting, the following has been agreed on RRM requirements for SSB adaptation [3].

- Sub-topic 1-1 SSB adaptation – General Issue 1-1-1: Processing time for SSB adaption Agreement RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. - FR1: 2ms FR2: 3ms - FFS L3 transition period measurement - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-2: L1 requirements after transition - DRX Agreement: - For L1 requirements impacts due to SSB adaptation, when DRX is configured, follow legacy principle to define requirements, which means the SSB adaption only impacts TSSB in the measurement period Issue 1-2-3: L1 requirements - whether to release the measurement results from resources before transition Agreement: - No spec. impact is identified for releasing the measurement results. - It’s up to UE implementation.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – transition for serving cell Agreement RAN4 to define L3 requirements at transition for SSB adaptation same as legacy L3 transition period requirement. - The cell identification and measurement period requirements apply based on the longer period before or after the transition. Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell Agreement - Subsequent to transition, serving cell identification and measurement period requirements are based on the additional SMTC configuration according to SSB adaptation mapped with SSB periodicity. Issue 1-3-4: Colliding with MG - Proposals - Option 1: If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-1: SCell activation requirement for SSB adaptation – SSB adaptation command related Agreement: - For the conditions for SCell activation with SSB adaptation, SSB adaptation command comes no later than SCell activation command. Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements - Proposals - Option 1: R18 Scell activation with L3 reporting - Option 2: multiple SCells activation scenario with SSB adaptation

Open issues for SSB adaptation are contained in [2] and previous RAN4 agreements are listed in Annex 1.

RAN1 agreements are listed in Annex 2.

  - Scenarios for SSB adaptation

In this section we consider open issues related to scenarios for SSB adaptation.

## Applicable SCell activation scenarios with SSB adaptation

RAN4 #114bis discussed the applicable SCell activation scenarios with SSB adaptation and agreed to specify/study requirements for the following scenarios [4].

Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

The items for further study were treated at RAN4 #115 without conclusion, whether to define RRM core requirements for them, as depicted in section 2.1.

In our view, aside the first three agreed SCell activation scenarios, agreed at RAN4 #114bis, the fourth scenario

- Rel-18 SCell activation with L3 reporting,

which targets to improve the SCell activation delay in case of unknown single SCell, should also be considered for SSB adaptation to reduce the cell detection delay. Thereagainst, multiple SCells activation is a further enhancement on top of single SCell activation. This scenario has not been discussed neither in RAN1 nor RAN2 so far. Furthermore, those SCells may adapt to the same SSB periodicity or to different SSB periodicities, and SCell activation command may be sent subsequently to a particular SCell or in one common message to all SCells. These multiple cases need to be dealt with and the network energy saving gain is left unclear for this scenario. Hence, RAN4 should exclude this scenario for Rel-19 NES enhancements.

Proposal 1:	RAN4 to add the scenario of Rel-18 SCell activation with L3 reporting to the applicable SCell activation scenarios with SSB adaptation.

Proposal 2:	RAN4 to exclude multiple SCells activation scenario with SSB adaptation from Rel-19 NES enhancements.

    - Signaling methods for SSB adaptation

RAN4#115 kept the discussion open on signaling methods for triggering SSB adaptation awaiting further progress in RAN1 and RAN2 [2].

Issue 1-1-2: Scenario clarification-triggering - Proposals - Option 1(Samsung) - For SSB adaptation, RAN4 to define requirements considering the following cases - HF index is same before and after SSB adaptation and SFN offset is same before and after SSB adaptation - HF index is same before and after SSB adaptation and SFN offset is same or different before and after adaptation - Wait for RAN1 conclusions on whether additional RRC signalling for offset of SSB for additional SSB periodicity is needed - Option 2: RAN4 not to define RRM requirements for SSB adaptation for NW triggered MAC-CE based signalling. (Samsung) - Option 3: RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling methods, designed by RAN1 and RAN2. (Nokia) - Recommended WF: - For option 1, wait for RAN1 progress. - RAN4 to specifying RRM requirements for SSB periodicity adaptation triggered by DCI, and discuss whether to specify RRM requirements for RRC triggered SSB adaption.

In fact, as SSB adaptation represents a network-controlled procedure, as agreed under issue 1-1-3 at RAN4 #113, see Annex 1, the UE needs to process the signaling message, depending on the signaling method, which will be defined by RAN1 and RAN2 (e.g., based on group common DCI, MAC-CE or RRC). However, RAN2#129bis decided not to specify a MAC-CE based solution for SSB adaptation aside the group common DCI based solution designed by RAN1. Thus, RAN4 needs to define RRM requirements for SSB adaptation triggered only by group common DCI command and by RRC based signaling, when SSB adaptation command is before or is combined with normal SCell activation or direct SCell activation or PUCCH SCell activation, as agreed for issue 1-4-2, see section 2.2.1.

In our view, the additional delay for L1 and L3 measurements due to SSB adaptation for RRC triggered SSB adaptation is same as for DCI triggered SSB adaptation. This does not include DCI or RRC decoding delay, respectively.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling, the latter in combination with normal SCell activation or direct SCell activation or PUCCH SCell activation or any further agreed SCell activation scenarios.

  - Layer 1 measurement impact

RAN4#115 resolved some issues on L1 measurement requirements as depicted in section 2.1. One issue is identified when comparing definitions in RAN1 and RAN4 for first SSB occasion of the new SSB periodicity, and another issue which was left for further study at RAN4#115 is related to measurement restrictions.

## Definition of TfirstSSB

RAN1#121 achieved following agreement on the first SSB burst of the new SSB periodicity.

Agreement When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication, - Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d. SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

Regarding processing time for SSB adaptation, RAN4 achieved the following agreement at RAN4#115, replicated from section 2.1.

- Sub-topic 1-1 SSB adaptation – General Issue 1-1-1: Processing time for SSB adaption Agreement RAN4 to define the additional delay requirement in TfirstSSB for L1 transition period measurement for SSB adaptation. - FR1: 2ms FR2: 3ms

This means RAN4 agreed that UE needs some additional time of 2 ms in FR1 and 3 ms in FR2 to be ready to receive the first SSB occasion belonging to the new SSB periodicity signalled in the DCI command with DCI format 2_9.

From the above, RAN1 has agreed to specify that UE shall be assuming the first SSB belonging to the new SSB periodicity can be received not earlier than slot m+d given the DCI command indicating SSB adaptation is received in slot m.

SSB adaptation using DCI format 2_9 has been implemented for Rel-19 in clause 11.6 of TS 38.213, which is depicted below.

In our understanding, RAN1 and RAN4 views are inconsistent.

While RAN1 assumes the UE is ready to receive at slot m+d, e.g. after m+3 ms for SCS=15kHz in FR1 or after m+3ms for SCS=60 kHz in FR2, RAN4 assumes the UE is only ready to receive the first SSB after m+d+2 ms in FR1, e.g. m+5 ms for SCS=15kHz in FR1, or after m+d+3ms in FR2, e.g. m+6ms for SCS=60kHz in FR2, to take into account not only message decoding but also required Rx reconfiguration in the UE, since the additional SSB could have different SFN offset and different half frame offset than the default (nominal) SSB periodicity, as used in the PCell. This means that a UE is only required to measure the next SSB occasion of the new SSB periodicity that takes into account the additional processing delay of 2ms in FR1 and 3ms in FR2, respectively.

Thus, it is proposed to include the additional processing delay tFR1 and tFR2 into the last paragraph of clause 11.6 of TS 38.213 replacing

- “beginning of slot m + d” by

- “beginning of slot m + d + tFR1 for FR1 or beginning of slot m + d + tFR2 for FR2 with tFR1 and tFR2 specified in TS 38.133.”

The above leads to consistency of RAN1 and RAN4 specifications.

Proposal 4: 	RAN4 to send liaison to RAN1, asking them to include the additional SSB adaptation delay in the calculation of the earliest possible occurrence of the first SSB occasion belonging to the new SSB periodicity in clause 11.6 of TS 38.213.

## L1 measurement restrictions

RAN4#115 discussed the issue of collision of adapted SSB with CSI-RS.

Issue 1-2-4: L1 requirements – Measurement restriction impact - Proposals - Option 1: RAN4 to discuss whether existing L1 measurement restriction can apply to the case when adapted SSB colliding with CSI-RS. i.e. Whether UE can choose to measure one of them or only to measure SSB. (Huawei) - Option 2: When adapted SSB colliding with CSI-RS, existing L1 measurement restriction can apply. (CTC, Vivo) - Recommended WF: - Discuss above option.

The discussion was not further progressed at RAN4#115.

In our view, option 2 should be selected, as the collision between SSB after SSB adaptation and cell-specific CSI-RS is not different in substance to the collision of an SSB belonging to the normal SSB pattern and cell-specific CSI-RS, since only the SSB periodicity is changed (PRB allocation and Tx power setting are same). Hence legacy requirements should also apply here.

Proposal 5: 	For measurement restrictions, when adapted SSB collides with CSI-RS, existing L1 measurement restriction in clauses 9.5.5.1 and 9.5.5.2 apply.

  - Layer 3 measurement impact

RAN4#115 resolved some issues on L3 measurement requirements as depicted in section 2.1. One issue which was left for further study is related to measurement impact from adapted SSB to deactivated SCells, the other open issue is how to deal with collisions between adapted SSB and a measurement gap occasion.

## L3 measurement impact to deactivated SCell

For deactivated SCell measurements, RAN4 #114bis defined following open issue [4].

Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. - For neighbour cell L3 measurement, legacy requirements apply. Open issue for further discuss: - For SCell with SSB adaptation (deactivated SCell or activated SCell), wait for more RAN2 progress on additional SMTC configuration, and discuss following aspects: - Whether to define L3 measurement requirement at transition. - Any other RRM requirements impacts due to additional SMTC configurations.

RAN4 #115 continued the discussion but did not reach a conclusion.

Issue 1-3-2: L3 requirements for SSB adaptation – after transition for serving cell - Proposals - Option 1: Subsequent to transition, cell identification and measurement period requirements are based on the SMTC mapped with SSB periodicity. (Samsung (for activated SCell), ZTE, Oppo, CTC, CMCC, Ericsson, Xiaomi) - Option 1: UE should restart the L3 measurement and meet the requirements of measurement period based on the SMTC of adapted SSB periodicity after SSB adaptation (Oppo) - Option 2: If RAN4 to define L3 measurement requirements on deactivated SCell with SSB adaptation for DCI 2_9 based SSB adaptation indication. (Samsung) - The same mechanism from OD-SSB case#2 (same frequency but with different periodicities) can be applied by adaptive SMTC - FFS: Additional UE processing/preparation time for reception of SSB after adaptation - Option 3: For deactivated SCell L3 requirements for SSB adaptation, reuse legacy requirements. (LGE) - Option 3a: For deactivated Scell, the requirements of measurement period are still based on meascycleScell, But the interruption due to measurement on deactivated SCC should be updated with the mapped SMTC of SSB periodicity after SSB adaptation. (Oppo) - Option 3b: RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG. (Nokia) - Recommended WF: - For activated SCell, Agree on option 1. - For deactivated SCell, discuss following options: - Option 1: legacy framework based on SMTC and meascycleScell - Option 2: reuse same framework as OD-SSB case#2.

Legacy requirements for L3 measurements of deactivated SCells in RRC_CONNECTED state are contained in clauses 9.2.5 and 9.2.7, being specified under intra-frequency requirements. This includes the case that NCD-SSB based L3 measurements are used in the SCell, if the NW indicates it as reference SSB.

RRM impacts exist for cell identification of unknown cells (PSS/SSS detection, time index detection) and cell measurements for known cells, both for measurements without gaps (from Rel-15) and with NCSG (from Rel-17) as the measurement period is proportional to the measCycleSCell value up to a defined limit.

In all cases, the measurement period is depending on the parameter measCycleSCell, configurable between 160 ms and 1280ms, hence it is greater or equal to the largest existing SSB periodicity (160ms). Thus, there is no misconfiguration possible between SSB periodicity and measCycleSCell. In addition, these measurements will cause interruptions in the serving cell, either unknown (of measured without gaps) or known (if measured with NCSG). Higher number of these interruptions may result in more required retransmissions in the PCell which should be minimized. Hence it is recommended to stay with the legacy UE behavior for deactivated SCell measurements, as throughput in serving cell may be degraded due to caused interruptions and both UE / network energy savings may be compromised in case of lower SSB periodicity after adaptation. Thus, option 1 above is preferred.

Proposal 6: 	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG based on measCycleSCell.

## Collision of SSB measurements with MG occasion

Related to the scenario of additional SMTC configuration colliding with MG occasion, see section 2.1, RAN4 #115 left the conclusion open. This refers to the scenario, when the SMTC occasion of the selected additional SMTC configuration (like the SMTC occasion for the legacy SMTC configuration) collides with MG occasion. In this case, the UE should be allowed to skip measurements in the overlapping SMTC occasion of serving SCell.

Proposal 7: 	If the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

  - SCell activation requirements

For measurements related to SCell activation, RAN4 #114 achieved following agreement [5]:

Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG

RAN4#114bis did not further reach a conclusion on this issue beyond the recommended way forward [6].

- Recommended WF: - RAN4 to discuss the conditions for SCell activation with SSB adaptation (i.e. based SSB periodicity after SSB adaptation), considering following aspects: - SSB adaptation command (e.g. DCI/MAC-CE/RRC), which subject to conclusion from other WG - Time relation between SSB adaptation command and SCell activation command.

RAN4 #115 got the agreement listed under issue 1-4-1 in section 2.1 and continued the discussion without conclusion.

Issue 1-4-3: SCell activation requirement for SSB adaptation – Others - Proposals - Option 1: L1 measurement during SCell activation for DCI 2_9 based SSB adaptation indication (Samsung) - The requirement when DRX is configured. - Based on the additional SSB periodicity after adaptation - Option 2: concurrent SCell activation and SSB adaptation command in same RRC message, i.e. based on (Nokia) - Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms and known SCell in FR1. - Recommended WF: - Discuss above options.

Option 2 above addresses the scenario of concurrent SCell activation and SSB adaptation in same RRC message. The SCell activation delay is investigated in more detail.

For SCell activation in RRC_CONNECTED state, with current requirements specified in TS 38.133, clause 8.3.2, SSB periodicity adaptation to lower SSB periodicity can be used to fasten the serving cell measurements required to perform SCell activation based on lower time of TFirstSSB. This includes the case that NCD-SSB based measurements are used in the SCell, if the NW indicates it as reference SSB.

If the SSB adaptation command comes together with SCell activation command in the RRC message, the SCell activa-tion delay for known SCell in FR1 may be somewhat higher due to additional processing for SSB adaptation as part of Tactivation_time, such as

Tactivation_time = TFirstSSB + X, with X>5ms for measurement period lower or equal to 2400ms, and

Tactivation_time = TFirstSSB_MAX + Trs + X, with X>5ms, if the measurement period of the SCell being activated is larger than 2400 ms.

For instance, if 1ms additional processing delay is assumed, X=6ms for the case of RRC triggered concurrent SCell activation and SSB adaptation command. No separate SSB adaptation delay is needed in this case and the lower SSB periodicity, part of the additional SMTC configuration, is immediately used after SCell activation and will fasten L3 measurements in the activated SCell.

Proposal 8: 	RAN4 to define SCell activation requirements in case of concurrent SCell activation and SSB adaptation command in same RRC message, e.g. for known SCell in FR1, based on

- Tactivation_time = TFirstSSB + X, with X=6ms for measurement period ≤ 2400ms,

- Tactivation_time = TFirstSSB_MAX + Trs + X, with X=6ms for measurement period > 2400ms.

- Conclusion

In this contribution, as part of the objective on “adaptation of common signal/channel transmissions” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed scenarios for investigating the RRM impact from SSB periodicity adaptation in time domain based on RAN1/2 progress and RAN4#115 agreements and presented our views on the impact to L1 and L3 measurement requirements.

Following proposals are made.

Scenarios for SSB adaptation

Proposal 1:	RAN4 to add the scenario of Rel-18 SCell activation with L3 reporting to the applicable SCell activation scenarios with SSB adaptation.

Proposal 2:	RAN4 to exclude multiple SCells activation scenario with SSB adaptation from Rel-19 NES enhancements.

Proposal 3: 	RAN4 to consider specifying RRM requirements for SSB periodicity adaptation for NW triggered group common DCI and RRC signaling, the latter in combination with normal SCell activation or direct SCell activation or PUCCH SCell activation or any further agreed SCell activation scenarios.

Layer 1 measurement transition requirements

Proposal 4: 	RAN4 to send liaison to RAN1, asking them to include the additional SSB adaptation delay in the calculation of the earliest possible occurrence of the first SSB occasion belonging to the new SSB periodicity in clause 11.6 of TS 38.213.

Proposal 5: 	For measurement restrictions, when adapted SSB collides with CSI-RS, existing L1 measurement restriction in clauses 9.5.5.1 and 9.5.5.2 apply.

Layer 3 measurement impact

Proposal 6:	RAN4 to define measurement requirements for deactivated SCell in case of SSB adaptation reusing the legacy requirements for measurements without gaps or with NCSG based on measCycleSCell.

Proposal 7: 	RAN4 to agree, if the SMTC occasion of the selected additional SMTC configuration collides with MG occasion, the UE is allowed to skip measurements in the overlapping SMTC occasion of the serving SCell.

SCell activation requirements with concurrent SSB adaptation

Proposal 8: 	RAN4 to define SCell activation requirements in case of concurrent SCell activation and SSB adaptation command in same RRC message, e.g. for known SCell in FR1, based on

- Tactivation_time = TFirstSSB + X, with X=6ms for measurement period ≤ 2400ms,

- Tactivation_time = TFirstSSB_MAX + Trs + X, with X=6ms for measurement period > 2400ms.

- References

[1] 	RP-251678, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, Apple, RAN#108

[2]	 R4-2505571, “Topic summary for [115][222] Netw_Energy_NR_enh_Part2, Moderator (Huawei)”, Moderator 	 	 	(Huawei), RAN4#115

[3]	R4-2508441, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4#115

[4]	R4-2504907, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4#114bis

[5]	R4-2502689, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4#114

[6]	R4-2503626, “Topic summary for [114bis][226] Netw_Energy_NR_enh_Part2”, Moderator (Huawei),

RAN4#114bis

- Annex 1: Previous RAN4 agreements on SSB adaptation

RAN4 has discussed the SSB adaptation enhancement in their last meetings. At RAN4#114bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - Existing SSB burst periodicity (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - SSB adaptation in SCell for NCD-SSB. - Time domain positions of SSBs within a burst is same before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement - RAN4 FFS how to define Tfirst_SSB considering the processing/preparation time based on further RAN1/2 progress on SSB adaptation command timeline. - No impacts on section 9.5.3 of L1 reporting requirements due to SSB adaptation, unless identified by other WGs. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation Agreement: - RAN4 to define requirements based legacy L3 measurement framework for SSB adaptation. For neighbour cell L3 measurement, legacy requirements apply. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-2: SCell activation requirement for SSB adaptation – applicable SCell activation requirements Agreement: - Normal SCell activation - Direct SCell activation - PUCCH SCell activation - FFS Rel-18 SCell activation with L3 reporting - FFS Multiple SCells activation

At RAN4#114 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adaptation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Further clarification on the scope of RRM impact for SSB adaption: - No RRM impacts for SSB adaptation requirement for PCell: - Frequency location is same before and after SSB adaptation. - No change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation - Note: Above could be further updated based on further RAN1 progress if any. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: Applicable L1 requirements for SSB adaptation Agreement: - No impact of SSB adaptation: - RLM - BFD - Define requirements if SSB adaption happens: - L1-RSRP for serving cell - L1-SINR - CBD - Whether and how to define requirements at transition is discussed in other issues. Issue 1-2-2: Transition requirements applicability & Issue 1-2-3: Transition requirements – details Agreement: - For L1-RSRP/L1-SINR/CBD - The measurement/evaluation period based on SSB periodicity after adaptation apply from Tfirst_SSB. - Tfirst_SSB is the uncertainty time from the timepoint when SSB adaptation occurs till the first SSB after SSB adaptation. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused.

- Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: Deactivated SCell measurement: - Option 1: For SSB adaption, reuse the same conclusion from OD-SSB case#2 (scenario#4) discussion to define RRM requirements. - At least for the case when the SSB periodicity is adapted to be shorter than before - Option 2: Same as legacy - Other options are not precluded. The requirement can be updated based on further progress in other WG or other aspects are identified that the conclusion can not be directly reused. - Sub-topic 1-4 SSB adaptation – Other requirements impacts Issue 1-4-1: SCell activation requirement for SSB adaptation Agreement: For SCell activation: - Define the SCell activation requirements based on the SSB periodicity after SSB adaptation. - For the case SSB adaptation command comes together with SCell activation command. The requirement can be updated based on further progress in other WG Issue 1-4-2: TCI state switching requirements for SSB adaptation Agreement: - No requirements when SSB adaptation happens during TCI state switching. - Sub-topic 2-1 RRM requirements impacts for OD-SIB1 Issue 2-1-1: RRM requirements impacts for OD-SIB1 Agreement: RAN4 to discuss the potential RRM impact for OD-SIB1 from the next meeting.

At RAN4#113 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - No RRM impacts for SSB adaptation requirement for IDLE mode. - No RRM impacts for Cell DTX. Issue 1-1-2: Scenario clarification- SSB periodicity Agreement: - Postpone the discussion and wait for RAN1 progress. Issue 1-1-3: Scenario clarification- SSB adaptation triggering Agreement: - RAN4 to focus on NW controlled SSB adaptation firstly. Issue 1-1-4: Scenario clarification- SSB adaptation mechanism Agreement: - Postpone the discussion and wait for RAN1 progress. - Sub-topic 1-2 SSB adaptation – L1 Requirements impacts Issue 1-2-1: L1 requirements at transition Agreement: - L1 evaluation/measurement period is the maximum of the period corresponding to the SSB before and after adaptation. - FFS the applicable L1 requirements. Issue 1-2-2: L1 requirements subsequent to “transition period” Agreement: - Subsequent to transition period, requirements apply based on the SSB periodicity after transition for SSB adaptation of periodicity. - Note: The transition period is discussed in other issues. - Sub-topic 1-3 SSB adaptation – L3 Requirements impacts Issue 1-3-1: L3 requirements for SSB adaptation – general Agreement: - Wait for LS reply on whether there is impact on SMTC.

At RAN4#112bis meeting, the following has been agreed on RRM requirements for SSB adaptation.

### Sub-topic 1-1 SSB adapation – Scenario Issue 1-1-1: Scenario clarification based on RAN1 progress Agreement: - Per RAN1 agreements, RAN4 to take following as starting point for SSB adaption discussion: - CONNECTED mode, and - SSB adaptation for SCell, and - SSB adaptation for NCD-SSB, and - SSB periodicity adaptation in time-domain Note: Above could be further updated based on further RAN1 progress (e.g. PCell, other adaptation mechanism) Issue 1-2-1: Requirements need updating Agreement: - RAN4 to discuss whether there are any impacts on at least the following RRM requirements due to SSB adaption: - L1-RSRP for serving cell - L1-SINR - BFD - CBD - SCell activation - TCI state switching - NR intra-frequency L3 measurement - NR inter-frequency L3 measurement Note: Above list could be further updated based on RAN4 discussion.

At RAN4#112 meeting, the following has been agreed on RRM requirements for SSB adaptation.

- Sub-topic 3-1: SSB Adaptation requirements Issue 3-1-1: SSB adaptation in IDLE/CONNECTED mode - Agreement - RAN4 to prioritize the SSB adaptation discussion in CONNECTED mode. - Whether to discuss SSB adaptation in IDLE mode depends on RAN1 progress Issue 3-1-2: SSB adaptation impact in RRM requirement - Agreement: SSB adaptation impact in RRM requirement - RAN4 to discuss the L1/L3 measurement requirement impact as starting point. - Wayforward: - RAN4 to discuss SSB periodicity adaptation in time-domain as starting point. - Other SSB adaptation mechanism (s) and requirements are not precluded depending on RAN1 progress.

- Annex 2: RAN1 agreements on SSB adaptation

RAN1 has discussed the SSB adaptation enhancement in their last meetings. At RAN1#121 meeting, the following agreements were achieved.

Agreements (RAN1#121) Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell for the case when cell DTX/DRX is not configured, reuse existing search space configuration parameter for DCI 2_9-based monitoring and existing DCI 2_9 size configuration parameter and update in specification that these are also applicable to SSB burst periodicity adaptation (when configured) Agreement Value d (in the WA from RAN1#120bis) is the number of slots for the SCS of the active DL BWP of the first serving cell in Table 11.5-1 of TS 38.213. Agreement When the SSB burst periodicity is switched from periodicity value P1 to periodicity value P2 based on DCI format 2_9 indication, - Alt 1: SFN offset (relative to SFN0) and half-frame index are configured per additional SSB periodicity value. - the first SSB burst according to the periodicity value P2 is determined as the first SSB burst according to the SSB burst periodicity value P2 and associated SFN offset and half-frame index that is no earlier than slot m+d. SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity.

At RAN1#120bis meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#120bis) Agreement For adaptation of SSB in time-domain, for DCI 2_9-based SSB burst periodicity adaptation for an SCell, - The DCI is scrambled a new RNTI, - Same search space and DCI size as that of cell DTX/DRX DCI if gNB configures both Agreement For DCI 2_9-based SSB burst periodicity adaptation for an SCell - the starting location of the information block for SSB burst periodicity indication for a SCell within the DCI format 2_9 is configured using a new RRC parameter - the length of the information block is given by ceil(log2(1+X)), where UE is configured with X additional SSB burst periodicities for the SCell Agreement For adaptation of SSB in time-domain, UE can be configured with X (<=Xmax) additional SSB burst periodicities for an SCell. - Xmax=2 Working Assumption When a UE receives in slot m on the active DL BWP of a first serving cell a PDCCH providing DCI format 2_9 that indicates a change in SSB burst periodicity of the SSB transmission on a second serving cell, the UE assumes SSB is transmitted on the second serving cell according to the indicated SSB burst periodicity from the beginning of the first slot containing the first [actually] transmitted SSB within the first [possible] SSB burst according to the indicated SSB burst periodicity that is no earlier than the slot m+d of the first serving cell where d is a number of slots for the SCS of the active DL BWP of the first serving cell [in Table 11.5-1 of TS 38.213]. - FFS: how to determine the first [possible] SSB burst

At RAN1#120 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#120) Conclusion There is no RAN1 consensus to support the following in Rel-19 - New SSB burst periodicity values other than the legacy values (i.e., 5 ms, 10 ms, 20 ms, 40 ms, 80 ms, or 160 ms). - New UE trigger - Adapting the transmitted number of SSBs within a SSB burst - Adaptation of the time domain positions of SSBs within a burst Conclusion There is no RAN1 consensus to support time domain adaptation for CD-SSB in Rel-19 for Scell Agreement For adaptation of SSB in time-domain, support the following to adapt SSB burst periodicity for an SCell - UE is configured with SSB burst periodicity using legacy signalling for the SCell - UE is configured with X additional SSB burst periodicity for the SCell - FFS: Value of X - SSB occasions with larger periodicity are subset of the SSB occasions with shorter periodicity - FFS: Whether there is specification impact - Note: This does not impact the discussion on OD-SSB - For switching the periodicity, down-select between - (MAC-CE) - MAC-CE details for SSB burst periodicity adaptation is up to RAN2. - (DCI) - Alt 1-3: DCI based signalling is used Agreement For adaptation of SSB in time-domain, for adapting SSB burst periodicity for an SCell - Support group common DCI signalling for switching the SSB burst periodicity using DCI format 2_9 with [cellDTRX-RNTI] - FFS: which scenario(s) is this applicable for (e.g. as defined in 9.5.1) Note: Above does not prevent RAN2 from designing a MAC CE based on OD-SSB feature and also used for SSB burst adaptation

At RAN1#119 meeting, the following conclusion and agreements were achieved:

Agreements (RAN1#119) Agreement Reply to Q1(What is the relation in terms of time location before and after SSB adaptation?): - RAN1 agreed that at least SSB burst periodicity is adapted. - There are no RAN1 agreements to adapt the time location of the SSB burst other than the periodicity but RAN1 is still discussing other options. Agreement Reply to Q2(What is the relation in terms of frequency location before and after SSB adaptation?): - The frequency location is same before and after SSB adaptation. Agreement Reply to Q3(What is the spatial relation before and after SSB adaptation?): - There is no change to spatial relation (in terms of QCL assumption) for the same SSB index before and after SSB adaptation. - At least the case where there is no change to actually transmitted SSBs within a burst before and after SSB adaptation is supported. Further update to be made based on RAN1#119 progress if any. Conclusion There is no RAN1 consensus to support SSB adaptation in time domain for Rel-19 NES-capable UE’s PCell (connected mode).

At RAN1#118bis, the following conclusion was reached:

Agreements (RAN1#118bis) [Adaptation of SSB in time domain] Conclusion​ There is no consensus on the support of adaptation of SSB for idle mode UEs in Rel-19​. Proposal 2.1.4-rev1​ For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).​ - No spec impact

At RAN1 #118 meeting, the following agreements were reached:

Agreements (RAN1#118) [Adaptation of SSB in time domain] Agreement For adaptation mechanism(s) of SSB in time-domain, - For Rel-19 NES-capable UE’s PCell (Connected mode), adaptation of CD-SSB on sync raster is not supported - FFS: Adaptation for SSB that is not CD-SSB is supported (A2) - FFS: Adaptation for SSB not on sync raster is supported (A3) - For Rel-19 NES-capable UE’s SCell - Adaptation of SSB configured for the SCell is supported for the following cases - FFS: Adaptation for CD-SSB (B1) including UE impact compared to legacy operation where the SSB is configured with periodicity>20msec for SCell - Adaptation for SSB that is not CD-SSB on sync raster (B2’) - Adaptation for SSB that is not CD-SSB not on sync raster (B3’) Agreement For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select from following options - Option 1: One SSB burst periodicity is configured for the UE and UEs assumes SSB transmissions are not present during Cell DTX non-active period - Option 2: UE assumes SSB transmission with different periodicities during Cell DTX non-active period and during Cell DTX active period - Option 3: Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior) – no spec impact

At RAN1#117 meeting, the following agreement was reached:

Agreement (RAN1#117) For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g., two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB