3GPP TSG-RAN WG4 Meeting #116	R4-2509953

Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM core requirements for On-demand-SIB1

Agenda item:	7.23.3.3

Document for:	Discussion

Introduction

In the Rel-19 WID on Enhancements of network energy savings for NR [1], the second objective for core requirements relates to the support for on-demand SIB1 (OD-SIB1). The objective was further discussed at RAN4#115 [2] and agreements were captured in the WF [3]. In this contribution, we continue the discussion on impacts to RRM core requirements for OD-SIB1.

Discussion

  - Progress status in RAN4

At RAN4#115, the following has been agreed on RRM requirements for On-demand SIB1 [3].

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption Agreement: - For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell - Define test case with longer TSI-NR with reasonable value in performance part.

  - Impacts on RRM core requirements for OD-SIB1

In this section we consider impacts on RRM core requirements for OD-SIB1 for idle/inactive RRC states as discussed during RAN4 #115 [2].

## Paging interruption

RAN4#115 concluded that no definite core requirement will be specified for paging interruption due to cell reselection to a NES cell. Rather a clarification will be added that the paging interruption may be longer due to the longer time to acquire SIB1 information in case of OD-SIB1, as SIB1 is not regularly present in the NES cell supporting OD-SIB1, and hence it must be activated in order to identify whether the NES cell is available or is barred for the UE.

The change to current requirements for paging interruption in idle mode, contained in TS 38.133, clause 4.2.2.6, is depicted below.

#### 4.2.2.6 Maximum interruption in paging reception UE shall perform the cell re-selection with minimum interruption in monitoring downlink channels for paging reception. In addition, when the UE is configured with eDRX_IDLE cycle, the UE shall not miss any paging in a PTW provided the paging is sent in at least 2 DRX cycles before the end of that PTW. At intra-frequency and inter-frequency cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels of the target intra-frequency and inter-frequency cell for paging reception. The interruption time shall not exceed TSI-NR + 2*Ttarget_cell_SMTC_period ms. Ttarget_cell_SMTC_period is the periodicity of the SMTC occasions configured for the target NR cell. If the target cell is in the PCI list of smtc2-LP, the SMTC periodicity follows smtc2-LP; otherwise, the SMTC periodicity follows smtc. At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission. At inter-RAT cell re-selection, the UE shall monitor the downlink of serving cell for paging reception until the UE is capable to start monitoring downlink channels for paging reception of the target inter-RAT cell. For NR to E-UTRAN cell re-selection the interruption time must not exceed TSI-EUTRA + 55 ms. TSI-NR is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 38.331 [2] for an NR cell. TSI-EUTRA is the time required for receiving all the relevant system information data according to the reception procedure and the RRC procedure delay of system information blocks defined in TS 36.331 [16] for an E-UTRAN cell. These requirements assume sufficient radio conditions, so that decoding of system information can be made without errors and does not take into account cell re-selection failure.

Changes to current requirements for paging interruption in inactive mode, contained in TS 38.133, clause 5.1.2.6, are not needed as it includes a reference to the clause for idle mode.

#### 5.1.2.6 Maximum interruption in paging reception The requirements in clause 4.2.2.6 shall apply.

Hence paging interruption will be extended in case of OD-SIB1 versus legacy case.

Proposal 1: 	RAN4 to consider following change to paging interruption during cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.” in clause 4.2.2.6 of TS 38.133.

## Cell reselection delay

In idle mode requirement, the time for cell reselection is specified as the delay within which the UE shall be able to evaluate whether a cell meets the reselection criteria, e.g. in TS 38.133 clause 4.2.2.3 for intra-frequency NR cells. This delay is also longer in case of an OD-SIB1 supporting cell, similar as for paging interruption time. Thus, following changes to current requirements for measurement of intra-frequency NR cells in idle mode, contained in TS 38.133, clause 4.2.2.3, are required, as depicted below.

#### 4.2.2.3 Measurements of intra-frequency NR cells The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities. The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS 38.304 [1] within Tdetect,NR_Intra when that Treselection= 0. An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band. The UE shall measure SS-RSRP and SS-RSRQ at least every Tmeasure,NR_Intra (see table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5) for intra-frequency cells that are identified and measured according to the measurement rules. At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission. The UE shall filter SS-RSRP and SS-RSRQ measurements of each measured intra-frequency cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by at least Tmeasure,NR_Intra/2. The UE shall not consider an NR neighbour cell for cell reselection, if it is indicated as not allowed in the measurement control system information of the serving cell.

And likewise, changes are needed to measurements for inter-frequency NR cells in clause 4.2.2.4 and to measurements for inter-RAT E-UTRAN cells in clause 4.2.2.5 of TS 38.133.

In addition, following change to clause 5.1.2.3 is needed for inactive mode.

#### 5.1.2.3 Measurements of intra-frequency NR cells The requirements in this clause apply when UE is configured with eDRX_IDLE, otherwise the requirements in clause 4.2.2.3 shall apply. The requirements in clause 4.2.2.9 apply for UE configured with relaxed measurement criterion when UE is not configured with eDRX_IDLE. When UE is configured with eDRX_IDLE and UE is not configured with eDRX by ran-ExtendedPagingCycleConfig-r18 or eDRX-AllowedInactive-r18 is not signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in table 5.1.2.3-1 and table 5.1.2.3-2. When UE is configured with eDRX by ran-ExtendedPagingCycleConfig-r18 and eDRX-AllowedInactive-r18 is signalled in SIB1, the requirements defined in section 4.2.2.3 shall apply with Tdetect,NR_Intra, Tmeasure,NR_Intra and Tevaluate,NR_Intra defined in table 5.1.2.3-3 and table 5.1.2.3-4. At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.

And likewise, changes are needed to measurements for inter-frequency NR cells in clause 5.1.2.4 and to measurements for inter-RAT EUTRAN cells in clause 5.1.2.5 of TS 38.133.

Proposal 2: 	RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells.

## UL-WUS Msg1 retransmission attempts

Another RRM impact which was raised during RAN4 #115 [2] is whether a minimum of UL-WUS Msg1 retransmissions per paging cycle should be considered. In such case, UL-WUS Msg1 retransmission(s) would be performed in case UE does not receive a RAR. RAN1 has sent a list of RRC parameters for UL-WUS transmission which comprise a maximum number of UL-WUS retransmissions preambleTransMax [4]. Thereagainst, UL-WUS repetitions were already excluded by RAN1.

On one side, this makes the UL-WUS transmission more robust in the NES cell, on the other side it also increases the paging interruption time discussed in section 2.2.1. In our view, priority may be given either to better coverage or to lower delay, which is then up to network decision. Hence RAN4 should investigate RRM impacts for both scenarios.

Proposal 3: 	RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured.

- Conclusion

In this contribution, as part of the second objective to “Specify support for on-demand SIB1 for UEs in idle/inactive mode” of the Rel-19 WID [1] to further enhance network energy savings for NR, we discussed impacts to RRM core requirements for OD-SIB1.

Following proposals are made.

Paging interruption

Proposal 1: 	RAN4 to consider following change to paging interruption during cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency and inter-frequency cell re-selection, the interruption time may be longer if the target cell is a NES cell supporting on-demand SIB1 transmission.” in clause 4.2.2.6 of TS 38.133.

Cell reselection delay

Proposal 2: 	RAN4 to consider following change to cell reselection delay in idle mode, when UE performs cell reselection to an OD-SIB1 cell: Add the phrase “At intra-frequency cell re-selection, Tdetect,NR_Intra , Tmeasure,NR_Intra and Tevaluate,NR_Intra may be longer if the target Cell is a NES Cell supporting on-demand SIB1 transmission.” for intra-frequency NR cells in clause 4.2.2.3 of TS 38.133 and the same phrase in clause 5.1.2.3 for inactive mode. Add similar statements in clauses 4.2.2.4 and 5.1.2.4 for inter-frequency NR cells and in clauses 4.2.2.5 and 5.1.2.5 for inter-RAT E-UTRAN cells.

UL-WUS Msg1 retransmission attempts

Proposal 3: 	RAN4 to consider the impact on RRM core requirements for one or more UL-WUS Msg1 retransmission attempts in the NES cell, if configured.

Changes to TS 38.133 related to Proposals 1 and 2 are submitted in the accompanying CR [5].

- References

[1] 	RP-251678, “Revised WID: Enhancements of network energy savings for NR”, Ericsson, Apple, RAN#108

[2]	 R4-2505571, “Topic summary for [115][222] Netw_Energy_NR_enh_Part2, Moderator (Huawei)”, Moderator 	 	 (Huawei), RAN4#115

[3]	R4-2508441, “WF on RRM requirements for Netw_Energy_NR_enh_Part2”, Huawei, RAN4#115

[4]	R1-2501644, “LS on Rel-19 higher layers parameters list Post RAN1#120”, RAN1

[5] 	R4-2509954, “Draft CR38.133 Introduction of RRM core requirements for OD-SIB1”, Nokia, Nokia Shanghai Bell