3GPP TSG-RAN WG4 Meeting #116	R4-2509958

# Bengaluru, India, August 25th – 29th, 2025

Source:	Nokia

Title:	Discussion on XR ph3 RRM requirements

Agenda item:	7.26.3

Document for:	Discussion

Introduction

In this paper we present Nokia’s views on open issues after RAN4 #115 on RRM requirements for XR enhancements [1] and provide respective observations and proposals. Note, agreements reached in RAN4#115 are contained in [2].

Discussion

Discussion Highlights

In particular, the following aspects are treated in this contribution and respective observations and proposals are made.

- Section 2.2: Scenarios for measurement gap occasion cancelation

  - Measurement gap cancelation applies for all power classes, except for PC6 (HST).

- Section 2.3: Types of measurement gaps

  - For concurrent gaps, no procedure modification is needed, as TS 38.213 is clear on the UE behaviour for 2 non-colliding concurrent gaps separated by less than X ms.

- Section 2.4: Formulas for measurement delay extension

  - Measurement delay extension is determined with cancelled MG occasions not included in the CSSF scaling calculation.

  - This applies both for non-DRX mode and DRX mode.

- Section 2.5: Inter-RAT measurements

  - Measurement delay extension is determined with cancelled MG occasions for E-UTRAN cells

- Section 2.6: Measurement delay extension with concurrent gaps

  - Measurement delay extension is determined with cancelled MG occasions not included in the CSSF scaling calculation for concurrent Rel-17 gaps.

- Section 2.7: Conditions for measurement gap occasion cancelation

  - No conditions are needed for measurement gap occasion cancelation per measurement period or related to MG configuration.

- Section 2.8: L1/L2 triggered mobility measurements and procedures

  - Measurement delay extension is determined with cancelled MG occasions not included in the CSSF scaling calculation for LTM mobility measurements

- Section 2.9: MG cancelation indication impact on frequency domain

  - The applicability of the MG cancelation indication to the frequency range for per-UE / per-FR gaps is clarified related to normative work.

Scenarios for measurement gap occasion cancelation

In the last RAN4 meeting the following issue was left for discussion [2]:

## Issue 1-1: UE power class <Way forward> - Option 1: The requirements for inter-frequency measurement gap cancellation apply for UE power class 3.

With regard to the UE power class, it is not clear why XR measurement gap occasion cancelation should be limited to power class 3. We understand that this should be the most common case for XR measurement gap occasion cancelation, but nothing precludes that this method is used for other power classes.

The only exception for that is regarding high-speed train. When high speed train is considered, there has been more extensive studies on how to optimize HST measurement delays, and simulation results were provided to analyse mobility performance. Since HST is a special case, we understand that it would require more studies to guarantee the mobility performance.

Measurement gap cancelation applies for all power classes, except for PC6 (HST).

Types of measurement gaps

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 2-1: Type 2 gaps <Agreement> - MG cancelling DCI applies to concurrent MGs and cancels the MG surviving the collision handling rules. - The timeline X ms for the MG cancelling the colliding MGs is based on the one that survives collision handling rules. - DCI skipping cannot be applied if any gap is configured with gapAssociationPRS - FFS the applicability of the gap cancellation if two non colliding concurrent gaps are separated by less than X ms

The FFS in the agreement of RAN4 #115 was included because some companies state that the UE behaviour is unclear if two non-overlapping gaps are configured in close proximity. That is possible to happen if 2 concurrent gaps are configured with a 4 ms separation, and the UE supports XR gap cancelation with 5 ms distance between the DCI command and the start of the gap to be cancelled.

In our view the RAN1 specification is already very clear on that aspect. We included the specification on Annex B for the reader’s reference. In 38.213 it is stated that the cancelling command is associated to the earliest gap that starts after X ms after the DCI. In Figure 1 we show one example of such behaviour, where Gap 1 and Gap 2 are spaced by 5 ms – MGTA > 4ms. In this example, if the network wants to cancel one occasion of gap 2, it must send the DCI before gap 1, but on the 5 ms window before the start of Gap 1.

RAN1 spec is already clear on the UE behaviour of 2 non-colliding gaps separated by less than X ms.

Follow RAN1 specification, i.e.

  - the DCI command with gap cancelation applies for the first gap occasion happening at least X ms after the DCI command,

  - if 2 non-colliding concurrent gaps are separated by less than X ms, the second gap can be cancelled if a DCI is received during the time window of X ms before the first gap occasion.

| a) | b) |
| --- | --- |

Figure 1 Example of behaviour described in RAN1 regarding gap occasions in close proximity where a) the DCI is sent 5 ms before Gap 1, and therefore gap 1 is cancelled, and b) the DCI is sent after the 5ms requirement for cancelling Gap 1, and as a result, gap 2 is cancelled.

Formulas for measurement delay extension

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 3-1: Formulas for measurement delay extension without DRX <Way forward> - RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Option 1: Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Nskip × Max(MGRP , SMTC period)), - Option 2: Max( 200 ms, ( 8 + Nskip ) ´ Max(MGRP , SMTC period) ) ´ CSSFinter - Option 3: - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - where Tskip = Nskip ´ Max(MGRP , SMTC period) - Other options are not precluded - where Nskip is the number of skipped MG occasions in the measurement period. - FFS: CSSFinter

The main difference between option 1 and option 2 is how the multiple layers are considered with the carrier specific scaling factor (CSSF) factor. In option 2, the delay extension of 1 measurement gap cancelation is multiplied by CSSF.

Going back to the formulas in R4-2508285, and considering that CSSF is the number of layers, we calculate the measurement delay for Option 1 and Option 2 for a given cancelling ratio. The cancelling ratio is calculated based on the number of cancelled occasions Ncancel as

- Ratio = (Ntotal-Ncancel) / Ntotal

Where Ntotal is the total number of gap occasions in the measurement cycle, and is calculated based on the formulas for Option 1 and Option 2 as

- Option 1: Ntotal = Max(200 ms × CSSFinter, 8 × Max(MGRP , SMTC period) × CSSFinter + Ncancel × Max(MGRP , SMTC period)) Max(MGRP , SMTC period)

- Option 2: Ntotal = Max( 200 ms, ( 8 + Ncancel ) × Max(MGRP , SMTC period) ) × CSSFinterMax(MGRP , SMTC period)

- Option 3: Ntotal = TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ TcancelMax(MGRP , SMTC period)

Figure 3 show the measurement delay calculation versus the cancellation ratio for the requirements in formulas of Option 1, 2, and 3.

When CSSF = 2 is analysed, for Options 1 and 3 the measurement delay for cancellation ratio of 50% is increased from 400ms to 640ms and 800ms for Options 1 and 3 in Figure 3 a). On the other hand, the measurement delay for cancellation ratio of 50 % is not determined for the formula of Option 2 as shown in Figure 3 b), i.e. delay approaches infinity. Also, it can be observed in Figure 3 a) that with 40% cancellation ratio, the delay with Option 2 is 1.7 s, whereas it is 520ms and 680ms for Option 1 and Option 3, respectively.

a); b). c); d).

Figure 2 Calculated measurement delay requirement versus cancellation ratio considering CSSF = 2 for the formulas in a) Option 1, Option 2 and Option 3, and b) Option 2; and CSSF = 3 for a) the formulas in Option 1, Option 2 and Option 3, and d) the formula in Option 2

When CSSF = 3 is analysed, for Options 1 and 3 the measurement delay for cancellation ratio of 33.3% is increased from 400ms to 720ms and 900ms for Options 1 and 3 in Figure 3 c). On the other hand, the measurement delay for cancellation ratio of 33.3 % is not determined for the formula of Option 2 as shown in Figure 3 d).

The results from Figure 3 show that when using the measurement delay extension formula of Option 2, there is an excessive extension whenever CSSF is 2 or larger. The resulting measurement delay of Option 2 approaches infinity whenever the gap cancellation ratio approaches 100/CSSF.

Formula in Option 2 results in measurement delay of infinity (no requirements) when CSSF = 2 and the measurement gap cancelation ratio approaches 50%.

Formula in Option 2 results in measurement delay of infinity (no requirements) when the measurement gap cancelation ratio approaches 100/CSSF %.

We have also provided simulation results showing the feasibility of the formula in Option 1 in Annex A. As part of this study, we reached the following conclusions:

Even with complex configuration of overlapping between frequency layers, it is possible to schedule measurements according to Option 1 of Issue 3-1.

The requirements according to Option 1 of Issue 3-1 provide a margin of 1 to 4 gap periods even in complex overlapping scenarios with CSSF>1.

RAN4 to define measurement delay extension due to measurement gap occasion cancellation for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, 8   ´ Max(MGRP , SMTC period) ´ CSSFinter + Ncancel ´ Max(MGRP , SMTC period)), where Ncancel is the number of cancelled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

As an alternative solution, we can also consider the formula as adding the cancelation term an additional term to the total measurement delay.

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tcancel) ms

  - Where

    - Tcancel = Ncancel ´ Max(MGRP , SMTC period)

    - Ncancel is the number of  canceled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Additionally, the same issue is open for the case with DRX configured [2]:

## Issue 3-2: Formulas for measurement delay extension with DRX <Way forward>: - Option 1: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - DRX cycle ≤ 320 ms - Max(200 ms ´ CSSFinter, Ceil(8 ´1.5) ´ Max(MGRP, SMTC period, DRX cycle) ´ CSSFinter + Nskip × Max(MGRP, SMTC period, DRX cycle) ) - DRX cycle > 320 ms - 8 ´ DRX cycle ´ CSSFinter + Nskip ´ DRX cycle - Option 2: Inter-frequency cell detection period requirements with MG cancellation due to XR can be as in the table below for FR1 and FR2-1, respectively: - DRX cycle ≤ 320 ms - Max(200 ms, Ceil((8+XPSS/SSS,gaps) ´1.5) ´ Max(MGRP, SMTC period, DRX cycle)) ´ CSSFinter - DRX cycle > 320 ms - Ceil(8+XPSS/SSS,gaps) ´ DRX cycle ´ CSSFinter - Option 3: RAN4 to define measurement delay extension due to measurement skipping for inter-frequency measurements with gap as - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tskip) ms - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tskip) ms - Where - DRX cycle ≤ 320 ms, Tskip = Nskip ´ Max(MGRP , SMTC period, DRX cycle) - DRX cycle > 320 ms, Tskip = Nskip ´ DRX cycle - Other options are not precluded

For the case with DRX we propose to extend our previous proposals to consider DRX. The first alternative proposal is the following.

Alternative 1: RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5)  ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Lcancel ´ Max(MGRP , SMTC period, DRX cycle))

  - DRX cycle > 320 ms

    - 8 ´ DRX cycle ´ CSSFinter +  Lcancel ´ DRX cycle

  - Use this formula as example and follow similar approach for the other cases.

And the 2nd alternative proposal is the following.

Alternative 2:

  - RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + Tcancel) ms

  - Where

    - DRX cycle ≤ 320 ms, Tcancel = Lcancel ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tcancel = Lcancel ´ DRX cycle

Inter-RAT measurements

In the last RAN4 meeting the following issue was left for discussion [2]:

## Issue 3-3: Formulas for measurement delay extension inter-RAT <Way forward>: - Discuss how to extend the formulas in 3-1 and 3-2 for inter-RAT

In the case of inter-RAT E-UTRA measurements, one simple way to extend the measurement delay is to add a term proportional to the number of cancelled gap occasions, i.e. ceil( LCancel×Kgap_EUTRA)×MGRP, where Kgap_EUTRA relates to the concurrent gaps. If a single gap is configured, Kgap_EUTRA=1, and Kgap_EUTRA>1 in the case of concurrent overlapping gap patterns. This is in line with the proposals that we brought for the NR measurements (with and without DRX) and the proposals related to concurrent gaps in the next section of this paper.

Extend the measurement delay for Inter-RAT E-UTRAN as

  - TIdentify,  E-UTRAN FDD, cancel=TIdentify,  E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA)×MGRP

  - TMeasure,  E-UTRAN FDD, cancel=TMeasure,  E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA )×MGRP

  - TIdentify,  E-UTRAN TDD, cancel=TIdentify,  E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP

  - TMeasure,  E-UTRAN TDD, cancel=TMeasure,  E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP

Measurement delay extension with concurrent gaps

In the case of concurrent gaps, the extension of the measurement delay must consider Kgap. The Rel-17 measurement delay for concurrent gaps considers the factor Kgap in order to extend the measurement delay in the case of partially overlapping measurement gaps. In one example, we evaluate the case where 2 gap patterns are configured with

- Gap 1 with MGRP=20, and low priority

- Gap 2 with MGRP=40 and high priority

In this example Kgap = 2 for Gap 1, and Kgap = 1 for Gap 2.

Figure 3 shows one example of gap cancellation with the concurrent gap configuration described above. In Figure 3 a) the cancellation indication is received before one non-dropped occasion of Gap 2. In this case the next available occasion for MO2 is non-dropped. And since Gap 2 has Kgap=1, the measurement delay extension due to cancelling an occasion of Gap 2 can be MGRP = 40 ms. The cancelling of Gap 2 should also have no impact on the measurement delay of Gap 1.

In the example of Figure 3 b) a non-dropped occasion of Gap 1 is cancelled. Since Gap 1 has lower priority, and half of its occasions are dropped due to the collision with Gap 2, the extension of Gap 1 should consider that only the second next occasion is non-dropped and available for measurement of MO1. Therefore, the extension of Gap 1 should be MGRP * Kgap = 20 * 2 = 40 ms.

a)

b)

Figure 3 Examples of gap cancellation with partially overlapped concurrent gaps; a) cancelling high priority gap, b) cancelling low priority gap

Another aspect to consider is that Kgap may be non-integer. If 2 partially overlapping gaps have MGRP=160, and MGRP=40 ms, the value of Kgap for the 40ms gap will be 4/3. This is already considered in Rel-17 requirements with aceil operation.

Considering the aspects above, the formulas for measurement delay requirements can be extended for intra-frequency measurement period as:

- Max(200 ms ´ CSSFinter, ceil( 8 * Kgap)   ´ Max(MGRP , SMTC period) ´ CSSFinter + ceil( Lcancel ´ Kgap ) ´ Max(MGRP , SMTC period)), where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period.

The resulting measurement delay versus cancellation ratio with the proposed formula can be visualised with Figure 4, where only the non-dropped measurement occasions are used to calculate the cancellation ratio. As it can be observed in Figure 4, the resulting measurement delay requirement for 50% cancellation ratio is twice the delay for the legacy non-cancellation behaviour (cancellation ratio=0%) even for all the analysed cases.

Figure 4 Measurement delay versus cancellation ratio for the proposed measurement delay requirement formula

RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, ceil( 8 * Kgap)   ´ Max(MGRP , SMTC period) ´ CSSFinter + ceil( Lcancel ´ Kgap ) ´ Max(MGRP , SMTC period)), where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Cancelation of measurement gap occasions related to a certain gap pattern does not affect the measurement delay for another configured gap pattern.

Conditions for measurement gap occasion cancelation

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 3-5: Conditions for measurement skipping <Way forward>: - Option 1: No conditions are defined - Option 2: Define the maximum number of gaps skipped during the measurement period. - Option 3: Define a maximum separation between two non-skipped gaps for measurement

In this issue, few companies proposed that a limitation condition is added where measurement gap occasion cancelation may be used. These included maximum number of occasions to be canceled or measurement gap configurations.

Regarding Option 1 of issue 3-3, we understand that some companies have concerns on potential impacts if measurement gap occasion cancelation exceeds a certain limit. However, it is not very clear how to define a limit to measurement gap occasion cancelation that is general enough to account for all possible scenarios.

In our companion paper we provided analysis of the need for measurement gap occasion cancelation and mobility impacts [7]. The results in our paper show that it can be beneficial in some situations to increase the measurement gap occasion cancelation to about 60%. Additionally, at least in one scenario we could observe that 75% cancellation could be used without significant performance impact.

Our simulation results show the benefit of using about 60% cancellation ratio.

In at least some scenarios cancellation ratio can be increased to about 75% without significant mobility degradation.

In other scenarios the UE will be performing mobility mostly on intra-frequency without gaps. In that case, if gaps are used for inter-frequency there should be no mobility performance impact of cancelling inter-frequency measurements.

Additionally, in other cases the network may configure gaps for the UE to report measurements on potential candidates for CA. In that case, the extension of the measurement delay will not have any damaging impact on mobility performance.

Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates.

Therefore, we think that it is natural that in the network deployment there is a limitation on the maximum number of cancellations. However, this is an optimization that can be determined case by case, for example considering the typical mobility of XR UEs in a given location, the ISD, or layers to monitor. If we define a limit in the specification, we will simply limit available options on the deployment and the benefit of the feature will be impacted.

Maximum number of cancellations depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance.

When considering Option 3, it is not clear, what is the relationship between the measurement gap length and the impact to measurement gap occasion cancellation. We understand that the measurement gap occasion cancellation will have a direct impact on measurement delay requirements, however those requirements do not depend on the measurement gap length.

No conditions are needed for measurement gap occasion cancellation regarding maximum number of cancelled occasions per measurement period or related to MG configuration.

L1/L2 triggered mobility measurements and procedures

For L1/L2 triggered mobility, a similar approach as the one used for defining the requirements for L3 measurements can be adopted. When extending that approach which we proposed in the sections above, we get the following proposal:

The measurement delay for L1/L2 triggered mobility is determined as follows: Max(Treport, Ceil(M * Kgap)  Max(MGRP, SSB period))  CSSFinter + Ceil(Lcancel * Kgap)  Max(MGRP, SSB period))

MG cancellation indication impact on frequency domain

In the last RAN4 meeting we have agreed on the following [2]:

## Issue 5-2: Skipping indication on frequency domain <Agreement>: - It is RAN4 common understanding that: If UE is configured with per-FR gap, the MG skipping only applies to the same FR and has no impacts on UE behavior on the other FR. If UE is configured with per-UE gap, the MG skipping applies to both FRs. - Further discuss whether it has already been specified in RAN1 CR, and whether/how to capture in RAN4 spec.

The application of the skipping or cancelation indication to the frequency range depending on the use of per-UE or per-FR gaps is taken into account in the accompanying CR on measurement gap cancelation [17] , as depicted below in red.

#### 9.1.y.3 Applicability The requirements with measurement gap cancelation in this clause apply under the following conditions. 1> if the indication of measurement gap occasion cancelation is configured by higher layers; and 1> if an explicit indication is received via DCI according to TS 38.213 [3] to cancel a measurement gap occasion; and 1> if the UE is configured with SA NR (with single carrier or NR CA) in FR1 or FR2-1; and 1> if the UE is not configured to do measurements for SRVCC in clause 9.4.6; and 1> if the UE is not configured with any MUSIM measurement gap pattern via MUSIM-GapConfig [2]; and 1> if the UE is not configured with positioning measurements requiring measurement gaps; and 1> if the UE is configured with one of the measurement gap configurations listed in clause 9.1.y.2: 2> if the UE is configured with per-UE measurement gap pattern(s), the measurement gap occasion cancelation applies to both FRs and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply; 2> Otherwise, if the UE is configured with per-FR measurement gap pattern(s), the measurement gap occasion cancellation only applies to the same FR and has no impacts on UE measurement behaviour in the other FR and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply;

In our view this covers well the previous agreement from RAN4 #115.

RAN4 to agree the applicability of the MG cancellation indication to the frequency range for per-UE / per-FR gaps as provided in the Draft CR38.133, clause 9.1.y.3 in R4-2509959.

Conclusion

The following Observations and Proposals were made:

- Measurement gap cancelation applies for all power classes, except for PC6 (HST).

- RAN1 spec is already clear on the UE behaviour of 2 non-colliding gaps separated by less than X ms.

Follow RAN1 specification, i.e.

  - the DCI command with gap cancelation applies for the first gap occasion happening at least X ms after the DCI command,

  - if 2 non-colliding concurrent gaps are separated by less than X ms, the second gap can be cancelled if a DCI is received during the time window of X ms before the first gap occasion.

Formula in Option 2 results in measurement delay of infinity (no requirements) when CSSF = 2 and the measurement gap cancelation ratio approaches 50%.

Formula in Option 2 results in measurement delay of infinity (no requirements) when the measurement gap cancelation ratio approaches 100/CSSF %.

Even with complex configuration of overlapping between frequency layers, it is possible to schedule measurements according to Option 1 of Issue 3-1.

The requirements according to Option 1 of Issue 3-1 provide a margin of 1 to 4 gap periods even in complex overlapping scenarios with CSSF>1.

RAN4 to define measurement delay extension due to measurement gap occasion cancellation for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, 8   ´ Max(MGRP , SMTC period) ´ CSSFinter + Ncancel ´ Max(MGRP , SMTC period)), where Ncancel is the number of  canceled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Alternative proposal

  - RAN4 to define measurement delay extension due to measurement cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter+ Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter+ Tcancel) ms

  - Where

    - Tcancel = Ncancel ´ Max(MGRP , SMTC period)

    - Ncancel is the number of  canceled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Alternative 1: RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

  - DRX cycle ≤ 320 ms

    - Max(200 ms ´ CSSFinter, ceil(8 ´ 1,5)  ´ Max(MGRP , SMTC period, DRX cycle) ´ CSSFinter + Lcancel ´ Max(MGRP , SMTC period, DRX cycle))

  - DRX cycle > 320 ms

    - 8 ´ DRX cycle ´ CSSFinter +  Lcancel ´ DRX cycle

  - Use this formula as example and follow similar approach for the other cases.

Alternative 2:

  - RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

    - Tidentify_inter_without_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + Tcancel) ms

    - Tidentify_inter_with_index = (TPSS/SSS_sync_inter + TSSB_measurement_period_inter + TSSB_time_index_inter + Tcancel) ms

  - Where

    - DRX cycle ≤ 320 ms, Tcancel = Lcancel ´ Max(MGRP , SMTC period, DRX cycle)

    - DRX cycle > 320 ms, Tcancel = Lcancel ´ DRX cycle

Extend the measurement delay for Inter-RAT E-UTRAN as

  - TIdentify,  E-UTRAN FDD, cancel=TIdentify,  E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA)×MGRP

  - TMeasure,  E-UTRAN FDD, cancel=TMeasure,  E-UTRAN FDD+ceil( LCancel×Kgap_EUTRA )×MGRP

  - TIdentify,  E-UTRAN TDD, cancel=TIdentify,  E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP

  - TMeasure,  E-UTRAN TDD, cancel=TMeasure,  E-UTRAN TDD+ceil( LCancel×Kgap_EUTRA)×MGRP

RAN4 to define measurement delay extension due to measurement gap occasion cancelation for inter-frequency measurements with gap as

  - Max(200 ms ´ CSSFinter, ceil( 8 * Kgap)   ´ Max(MGRP , SMTC period) ´ CSSFinter + ceil( Lcancel ´ Kgap ) ´ Max(MGRP , SMTC period)), where Lcancel is the number of non-dropped cancelled MG occasions in the measurement period.

  - Use this formula as example and follow similar approach for the other cases.

Cancelation of measurement gap occasions related to a certain gap pattern does not affect the measurement delay for another configured gap pattern.

Our simulation results show the benefit of using about 60% cancellation ratio.

In at least some scenarios cancellation ratio can be increased to about 75% without significant mobility degradation.

Measurement gaps can be configured by the network to receive reports on potential carrier aggregation candidates.

Maximum number of cancellations depends on the deployment, e.g. number of layers, measurement gap configuration, deployment inter-site distance.

No conditions are needed for measurement gap occasion cancellation regarding maximum number of cancelled occasions per measurement period or related to MG configuration.

The measurement delay for L1/L2 triggered mobility is determined as follows: Max(Treport, Ceil(M * Kgap)  Max(MGRP, SSB period))  CSSFinter + Ceil(Lcancel * Kgap)  Max(MGRP, SSB period))

- RAN4 to agree the applicability of the MG cancellation indication to the frequency range for per-UE / per-FR gaps as provided in the Draft CR38.133, clause 9.1.y.3 in R4-2509959.

- References

- RP-250107, Revised WID: XR (eXtended Reality) for NR Phase 3, Nokia, Qualcomm

- R4-2508285, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2508312, LS on UE assistance information, Nokia

- R4-2504911, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2504972, Reply LS on UE assistance information, Nokia

- R4-2502700, WF on RRM requirements for NR_XR_Ph3, Nokia

- R4-2500542, Topic summary for [113][228] NR_XR_Ph3, Moderator (Nokia)

- R4-2502241, General aspects for XR enhancements, Nokia

- R4-2502229, RRM requirements for XR enhancements, Qualcomm Incorporated

- R4-2502150, Discussion on XR ph3 RRM requirements, Nokia

- R4-2504505, Analysis of skip ratio on XR capacity and mobility performance, Nokia

- R4-2504508, Discussion on XR ph3 RRM requirements, Nokia

- R1-2502977, Updated RAN1 UE features list for Rel-19 NR after RAN1 #120bis, AT&T, NTT DOCOMO, INC.

- R4-2420198, Response LS on gaps/restrictions that are caused by RRM measurements, To: RAN1, cc: RAN2, Source: Qualcomm

- R4-2504972, Reply LS on UE assistance information, Nokia

- 38.213, NR; Physical layer procedures for control, V19.0.0

- R4-2509959, “DraftCR38.133 Introduction of measurement gap cancelation”, Nokia

- Annex A: Measurement scheduling analysis

In this section we show results that show the feasibility of the requirement in the formula of Option 1 in Issue 3-1. In order to achieve this purpose, we studied the scenario of Figure 2, with 3 frequency layers, where the SMTC configuration is determined as

- For frequency layer F1: TSMTC = 40 ms, offset = 0 ms

- For frequency layer F2: TSMTC = 80 ms, offset = 0 ms

- For frequency layer F3: TSMTC = 80 ms, offset = 40 ms

- MGRP = 40 ms, and offset = 0 ms

In this scenario, the even SMTC occasions overlap with F1 and F2, whereas the odd SMTC occasions overlap with F1 and F3.  That results in CSSF = 2 for all frequency layers, since there is always 2 layers overlapping for each gap occasion. It was stated in RAN4 #115 that the problem with the scenario of Figure 2, is that it is not trivial for the UE to schedule a new measurement if one gap occasion is dropped due to XR gap cancellation. We show in this paper, that a simple Round Robin approach is enough to guarantee that the measurement delay is not larger than Option 1 in Issue 3-1.

Figure 5 Measurement delay simulation scenario with 3 frequency layers, and CSSF = 2.

In this paper we evaluated a simple method, where the even gap occasions are selected into 2 groups: The first gap group contains the gap occasions overlapping with F1 and F2. The second gap group contains the gap occasions overlapping with F1 and F3. The scheduling of the measurement is performed in a round-robin fashion within one measurement gap group, i.e. if one gap occasion is cancelled for one gap group, the same measurement decision is reused for the next gap occasion belonging to that group.

Using the configuration of Figure 2, we can give an example of scheduling as

- For gap occasions on group 1: [1 X 2 1 2 1 2]

- For gap occasions on group 2: [1 3 X 1 3 1 3]

That results in the scheduling decision as in the example of Figure 3.

Figure 6 Example of measurement decision based on Round Robin approach.

Using the approach above, we have developed simulations that show for each measurement cycle what is the measurement delay requirement of Option 1, and what is the achieved measurement delay. In all the cases, it was assumed the UE completes 1 measurement after collecting 5 samples (i.e. assuming intra frequency measurement period). The lower bond for measurement delay was not considered, and therefore we can expect that for low number of measurement gap cancelation the requirement would be more relaxed as the one shown in the results. For that purpose, we simulated 4 cases:

- Case 1: No gap cancellation, as control variable

- Case 2: 50% random gap cancellation

- Case 3: gap cancellation with a first regular pattern (110011001100), where 1 indicates gap cancellation

- Case 4: gap cancellation with a second regular pattern (11001110000).

Figure 4 shows the CDF for the measurement delay and the measurement delay requirement according to the formula in Option 1 in Issue 3-1. The horizontal axis is presents in gap occasions, or measurement delay divided by MGRP, for MO1, MO2, and MO3.

In the reference case of Figure 4 a), without gap cancelation, it can be noticed that the UE is able to complete the measurements few samples ahead of the requirements. It is also noticed that the delay is smaller with MO1, since the SMTC period is half for MO1 in comparison to MO2 and MO3.

In the cases with measurement gap cancelation of Figure 4 b), c, and d) similar trend as in the case without gap cancelation in Figure 4 a) can be observed, but now with a random delay extension. In all the cases, the CDF of the requirement is always to the right of the CDF of the actual measurement delay, which indicates that the UE is able to achieve the measurement delay within the requirements of Option 1 in Issue 3-1. In order to confirm that this is always the case, we also present the results in Figure 5, with statistics of the difference between the requirement of Option 1 in Issue 3-1 and the measurement delay.

| a) No gap cancellation | b) 50% random gap cancellation |
| --- | --- |
| c) gap cancellation with regular pattern 1 (110011001100) | d) gap cancellation with regular pattern 2 (11001110000) |

Figure 7 Simulation results for Round Robin scheduling of measurements for a) measurement delay without XR gap cancellation b) 50 % random gap cancellation c) first regular cancellation pattern (110011001100 with 1= cancel); d) second regular cancellation pattern (11001110000 with 1= cancel);

Figure 5 complements the results in Figure 4, with the indication of how tight the requirements of Option 1 in Issue 3-1 are in comparison to what the UE can really achieve. In this figure we analyse the difference between the extended measurement delay requirements and the actual measurement delay. Negative values in this figure indicate that the UE was not able to complete a measurement on time, i.e. requirements are not met. Values larger than 1 indicate that the UE has some margin to complete the measurements.

Figure 5 a) shows the results without measurement cancelation as a reference. In this case we can notice that the UE can always conclude the measurements on time, i.e. the difference is always larger or equal to 0. For some of the measurement samples margin for the measurements is of 1 to 4 measurement gap periods.

When cancelation of measurement gaps is considered in Figure 5 b), c) and d), similar margins as for the case without measurement cancelation in Figure 5 a) is observed. For MO1, the UE concludes the measurement by the deadline on about 15 to 20% of the cases, i.e. difference is equal to 0. In about 70% of the cases the UE concludes the measurements with 1 or 2 samples margin. For the case of MO2 and MO3, bigger margin is observed, since the SMTC for MO2 and MO3 is 2x larger than for MO1. For both MO2 and MO3 the margin in relation to the measurement requirement is of 1 to 4 measurement gap periods.

Even with complex configuration of overlapping between frequency layers, it is possible to schedule measurements according to Option 1 of Issue 3-1.

The requirements according to Option 1 of Issue 3-1 provide a margin of 1 to 4 gap periods even in complex overlapping scenarios with CSSF>1.

| a) No gap cancellation | b) 50% random gap cancellation |
| --- | --- |
| c) gap cancellation with regular pattern 1 (110011001100) | d) gap cancellation with regular pattern 2 (11001110000) |

Figure 8 Simulation results for Round Robin scheduling of measurements for a) measurement delay without XR gap cancellation b) 50 % random gap cancellation c) first regular cancellation pattern (110011001100 with 1= cancel); d) second regular cancellation pattern (11001110000 with 1= cancel)

- Annex B: RAN1 text in 38.213 for reference

RAN1 description of the measurement cap cancellation procedure in 38.213 [16]:

## 10.6 Indication for cancelation of RRM measurement gaps/restrictions A measurement gap cancellation field in a DCI format [5, TS 38.212] provided by a PDCCH reception is associated to an earliest RRM measurement gap occasion that starts at least 3 msec or 5 msec, as determined by a reported UE capability, after the end of the PDCCH reception. The indication by the measurement gap cancellation field is applied to the associated RRM measurement gap occasion on all cells in the applicable range of the measurement gap occasion that include the one or more scheduled cells associated with the DCI format [10, TS 38.133]. A UE can be indicated by a value ‘1’ of a measurement gap cancellation field in a DCI format provided by a PDCCH reception to cancel an associated RRM measurement gap occasion. If a UE indicates “Option 1” for XYZ, the UE ignores a value ‘0’ of the measurement gap cancellation field in a DCI format. If a UE indicates “Option 2” for XYZ, a value ‘0’ of the measurement gap cancellation field in a DCI format provided by a PDCCH reception indicates to the UE that the UE behavior for the associated RRM measurement gap occasion is as described in [10, TS 38.133]. When the UE detects more than one DCI formats that include the measurement gap cancellation field and are associated with a same associated RRM measurement gap occasion, the UE applies the indication provided by the measurement gap cancellation field of a last DCI format, from the more than one DCI formats.