3GPP TSG-RAN4 Meeting #116	R4-2509959

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DRAFT | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| x |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Title: | Draft CR38.133 Introduction of measurement gap cancelation |  |  |  |  |  |  |  |  |  |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_XR_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-15 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Measurement gap cancelation is part of the Rel-19 work item NR XR Phase 3. Delay requirements for cancelation of a particular measurement gap occasion are missing in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduction of requirements for measurement gap occasion cancelation used for XR. Clause 9.1.y - Specifies which gap types are subject to measurement gap skipping - Specifies applicability of the measurement gap occasion cancelation enhancement - Specifies requirements for the measurement gap occasion cancelation delay. This CR is drafted using RAN2 pseudo code approach to serve as one example for specification improvement as agreed in RAN4 #113 in TDoc R4-2420107. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing delay requirements and applicability for measurement gap occasion cancelation. The enhancement cannot be used for XR. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.y (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | None. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

--- Start of change 1 ---

#### 9.1.13.3	Collision involving NCSGs

Collisions between occasions of concurrent measurement gap and NCSG or of two NCSGs may occur as specified in this clause if the two occasions are

-	two per-UE NCSGs, or

-	two per-FR NCSGs in the same FR, or

-	one per-UE NCSG and one per-UE measurement gap, or

-	one per-FR NCSG and one per-UE measurement gap, or

-	one per-UE NCSG and one per-FR measurement gap, or

-	one per-FR NCSG and one per-FR measurement gap in the same FR.

and if the gap collision condition specified in clause 9.1.8.3 is met then the gap collision rule applies.

When the first occasion is NCSG, the ending point of the first occasion is the end of VIL2 and/or when the second occasion is NCSG, the starting point of the second occasion is the start of VIL1.The requirements with concurrentMeasGapsNCSG-r18 apply provided that two gaps (at least one of the gaps is NCSG) colliding with each other are configured with different priorities.

### 9.1.y	Measurement gap occasion cancelation

#### 9.1.y.1	Introduction

This clause specifies the general requirements and applicability for UE supporting indication for cancelation of measurement gap occasions as described in 38.213 [3].

#### 9.1.y.2	Applicable measurement gap configurations

Measurement gaps configured by one of the below configurations are applicable for measurement gap occasion cancelation:

-	GapConfig without suffix;

-	GapConfig-r17 with gapPriority-r17 and without ncsgInd-r17 or preConfigInd-r17 or gapAssociationPRS;

#### 9.1.y.3	Applicability

The requirements with measurement gap cancelation in this clause apply under the following conditions.

1>	if the indication of measurement gap occasion cancelation is configured by higher layers; and

1>	if an explicit indication is received via DCI according to TS 38.213 [3] to cancel a measurement gap occasion; and

1>	if the UE is configured with SA NR (with single carrier or NR CA) in FR1 or FR2-1; and

1>	if the UE is not configured to do measurements for SRVCC in clause 9.4.6; and

1>	if the UE is not configured with any MUSIM measurement gap pattern via MUSIM-GapConfig [2]; and

1>	if the UE is not configured with positioning measurements requiring measurement gaps; and

1>	if the UE is configured with one of the measurement gap configurations listed in clause 9.1.y.2:

2>  if the UE is configured with per-UE measurement gap pattern(s), the measurement gap occasion cancelation applies to both FRs and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply;

2> Otherwise, if the UE is configured with per-FR measurement gap pattern(s), the measurement gap occasion cancellation only applies to the same FR and has no impacts on UE measurement behaviour in the other FR and the requirements with measurement gap occasion cancelation in clause 9.1.y.4 apply;

#### 9.1.y.4	Requirements for canceling measurement gap occasions

For the measurement gap patterns configured via gapPriority-r17 and without ncsgInd-r17 or preConfigInd-r17 or gapAssociationPRS, the measurement gap cancelation applies to measurement gap occasions after applying collision rules as defined in clause 9.1.8.3.

UE behaviour during cancelled measurement gap occasions is the following.

1>	if the indication of measurement gap occasion cancelation is configured by higher layers; and

1>	if the measurement gap occasion is of one of the applicable gap configurations listed in clause 9.1.y.2; and

1>	if a DCI indicating gap cancelation (value ‘1’) is received at n, where n is the last symbol of the PDCCH containing DCI, according to TS 38.213 [3]:

2>	if the UE supports [“Option 1” for XYZ ]

3>	Define T as the actual start of the first non-dropped measurement gap including MGTA where T ≥ n + X ms, and X is defined in Table 9.1.y.4-1;

3>	If the UE receives a DCI with value ‘0’ at n1 > n and T ≥ n + X ms

4) 	It shall ignore this DCI with value ‘0’.

3>	The UE shall cancel the first non-dropped measurement gap occasion starting at T;

3>	The UE shall receive and transmit without scheduling restrictions on all the cells applicable to the cancelled measurement gap occasion starting at T;

2>	if the UE supports [“Option 2” for XYZ ]

3>	Define T as the actual start of the first non-dropped measurement gap including MGTA where T ≥ n + X ms, and X is defined in Table 9.1.y.4-1;

3>	If the UE receives a DCI with value ‘0’ at n1 > n and T ≥ n + X ms

4>	The first non-dropped measurement gap occasion starting at T is not canceled;

3>	Otherwise, if no DCI with 0 is received before n + X,

4>	The UE shall cancel the first non-dropped measurement gap occasion starting at T;

4>	The UE shall receive and transmit without scheduling restrictions on all the cells applicable to the cancelled measurement gap occasion starting at T;

Table 9.1.y.4-1 – Time offset X between the indication of cancelation of measurement gap and the start of measurement gap occasions including MGTA

| [Component 4 of FG 64-1] | Time offset |
| --- | --- |
| 5 | 5 ms |
| 3 | 3 ms |

--- End of change 1 ---