3GPP RAN WG4 Meeting #116																R4-2509964

Bangalore, India, 25th – 29th August 2025

Agenda item:	7.6.4.2

Source:	Apple

Title:	Power back-off values for the 3MHz channel in band n256

WI/SI:	NR_IoT_NTN_req_test_enh

Release:	Rel-19

Document for:	Decision

# 1	Introduction

After the RAN#86 meeting, the Rel-17 WI was approved that aimed at enabling the 5G/NR radio access technology for non-terrestrial satellite deployments [1]. One of the WI objectives was to define and introduce the corresponding bands into the 3GPP specifications. The outcome of these discussions was two NTN bands – L-band and S-band – which were added as bands n255 and n256, respectively, to the Rel-17 core specifications.

After the RAN#104 meeting a new WI was agreed [2], one of the aims of which is to enable the 3MHz channel for the NTN bands including the band n256. During the previous RAN WG4 meetings initial power back-off simulation results were submitted to for technical discussions and were captured in [3]; however, there was no final conclusion on which A-MPR regions should be defined with associated power back-off values. After the RAN4#115 meeting the preliminary A-MPR values were captured in WF [5], but after further detailed checking we spotted some logical inconsistencies across several cases. Thus, in this discussion paper we present UE power back-off simulation results for the 3MHz channel in the NTN band n256 frequency range.

# 2	Power back-off for band n256

## 2.1	Simulation assumptions and scenarios

The satellite S-band (n256) was standardised already in Rel-17, whereupon the smaller considered channel bandwidth was 5MHz. Here we present power back-off results for the PC3 power class and the 3MHz channel with the following common assumptions:

-	Tx power: PC3 (+23dBm)

-	LO placement: always in the centre of the carrier

-	Regulations: -50dBm/MHz to protect the 2010  ≤ f ≤ 2025MHz frequency range

The power back-off simulations were conducted for the following center frequencies:

-	the right-most channel with the 5MHz offset, as "guard" band, from the upper edge of the band (Fc=2003.5MHz)

-	the upper channel with the 6.5MHz offset from the upper edge (Fc=2002MHz)

-	the upper channel with the 8MHz offset from the upper edge (Fc=2000.5MHz)

-	the upper channel with the 9.5MHz offset from the upper edge (Fc=1999MHz)

## 2.2	Power back-off simulations

Figures 2.2-1, 2.2-2 and 2.2-3 below present the power back-off simulation results for the 3MHz channel at different locations as mentioned in the section 2.1. As anticipated, the upper most channel has the highest power back-off values. For instance, the full channel allocation of 15RB might require 6dB power back-off. And as the channel shifts to the left from the upper edge of the band, the power back-off decreases as well.

Figure 2.2-1: Power back-off results for NS_24 (Fc=2003.5MHz).

Figure 2.2-2: Power back-off results for NS_24 (Fc=2002MHz).

Figure 2.2-3: Power back-off results for NS_24 (Fc=2000.5MHz).

Figure 2.2-4: Power back-off results for NS_24 (Fc=1999MHz)

## 2.3	Proposed A-MPR regions and further discussions

After the RAN4#115 meeting the following preliminary A-MPR regions and values were agreed, see Table 2.3-1 and 2.3-2 below.  However, after further checking of the proposed values the following minor inconsistencies were spotted as summarized below:

-	DFT-s-OFDM A-MPR values are not consistent across different higher order modulation schemes. Usually, we adopt extra 0.5dB for every next higher modulation, but this is not consistent in A1 and A4.

-	DFT-s-OFDM A-MPR values for PI/2 BPSK and QPSK are not consistent either. Usually, we observe 0.5..1dB improvement for PI/2 BPSK, but A4 has 1.5dB difference.

Table 2.3-1: A-MPR regions for NS_24N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 2000.5 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  | [>= 2.52] | [<= 1.08 > 0.72] | [A2] |
|  | 1997.5 < fc <= 2000.5 |  | >= 1.8 | A4 |

Table 2.3-2: A-MPR for NS_24N

| Modulation/Waveform | A1 | A2 | A4 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | 6.0 | [1.0] | 2.0 |
| DFT-s-OFDM QPSK | 7.0 | [1.5] | 3.5 |
| DFT-s-OFDM 16 QAM | 8.0 |  | 3.5 |
| DFT-s-OFDM 64 QAM | 8.5 |  | 3.5 |
| CP-OFDM QPSK | 9.0 |  | 5.0 |
| CP-OFDM 16 QAM | 9.0 |  | 5.0 |
| CP-OFDM 64 QAM | 9.0 |  | 5.0 |

Based on the presented considerations and additional simulations done for PI/2 BPSK, we suggest adopting the following changes.

Table 2.3-3: A-MPR regions for NS_24N

| Channel BW | Carrier Frequency, Fc, MHz | RBStart*12*SCS (MHz) | LCRB*12*SCS (MHz) | A-MPR |
| --- | --- | --- | --- | --- |
| 3MHz | 2000.5 < fc <= 2003.5 |  | > 1.08 | A1 |
|  |  | [>= 2.52] | [<= 1.08 > 0.72] | [A2] |
|  | 1997.5 < fc <= 2000.5 |  | >= 1.8 | A4 |

Table 2.3-4: A-MPR for NS_24N

| Modulation/Waveform | A1 | A2 | A4 |
| --- | --- | --- | --- |
| DFT-s-OFDM PI/2 BPSK | 6.0 | [1.0] | 2.0 |
| DFT-s-OFDM QPSK | 7.0 | [1.5] | 3.0 |
| DFT-s-OFDM 16 QAM | 7.5 |  | 3.5 |
| DFT-s-OFDM 64 QAM | 8.0 |  | 4.0 |
| CP-OFDM QPSK | 9.0 |  | 5.0 |
| CP-OFDM 16 QAM | 9.0 |  | 5.0 |
| CP-OFDM 64 QAM | 9.0 |  | 5.0 |

Proposal:	Define A-MPR for the 3MHz channels as presented in Table 2.3-3 and 2.3-4.

# 3	Conclusions

In this discussion paper we have presented our further results for the power back-off values for the 3MHz channel operating in the NTN band n256 frequency range, for which we did additional simulations involving PI/2 BPSK. Since according to the presented simulations PI/2 BPSK can improve power back-off by only 0.5..1dB, we suggest adopting slightly revised numbers as presented in Table 2.3-3 and 2.3-4.

Proposal:	Define A-MPR for the 3MHz channels as presented in Table 2.3-3 and 2.3-4.

# 4	References

- RP-193234, "Solutions for NR to support non-terrestrial networks (NTN)", Thales

- RP-240857, "New WID: Enhanced requirements and test methodology for NR and IoT NTN"

- R4-2502282, "WF for [113][311] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi

- R4-2504713, "WF for [114bis][310] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi

- R4-2508629, "WF for [115][310] NR_IoT_NTN_less_than_5MHz_UERF", Xiaomi (moderator)