3GPP TSG RAN WG4 #116	R4-2509989

Bengaluru, India, Aug 25th – 29th, 2025

Agenda item:		7.3.4

Source:				Ofinno

Title:					Analysis of RRM Requirements for Low Band CA via Switching

Document for:		Discussion

# Introduction

The core part of Rel-19 WI on Low Band Carrier Aggregation via Switching is planned to be completed at RAN#109 (September) [1]. In the last RAN4 meeting a way forward containing agreements related to the RRM core requirements and draft CR work split among companies was approved [2]. The RAN1 specification work on the physical layer aspects of the Low Band Carrier Aggregation via Switching was completed at RAN1#121. According to the RAN1 agreements the low band CA switching pattern (LBCA-SwitchingPattern) is a bit map with a periodicity of 40 slots (i.e., 40 ms since SCS=15 KHz) [3]. In the 40-bit bitmap ‘0’ indicates FDD carrier and ‘1’ indicates SDL carrier [3].

In this contribution we provide our views on some of the RRM core requirements for the low band CA via switching.

# Active TCI state switching requirements

In the last RAN4 meeting (RAN4#115), RAN4 identified the impact of the low band CA via switching on the active TCI state switching requirements defined in clause 8.10 of TS 38.133. According to the RAN4 WF the following high level agreement was reached [2].

Issue 1-3-11: TCI state switch delay requirement for SDL SCell

Agreement:

- The L1-RSRP measurement and T/F tracking for TCI switching on SDL carrier is performed on the overlapped occasions between SSB/CSI-RS and SDL duration in switch pattern.

The above-mentioned agreement is related to the active TCI state switching delay requirements only on SDL SCell. However, in our view the active TCI state switching delay requirements on the PCell shall also be impacted. The analysis below applies to both PCell and SCell.

## MAC-CE based TCI state switching delay

MAC-CE based TCI state switching delay for known TCI state:

Clause 8.10.3 of TS 38.133 defines requirements on the MAC-CE based TCI state switching delay for the case when the target TCI state is known to the UE as follows:

If the target TCI state is known, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc) / NR slot length.

In the above equation:

- TSSB-proc being the processing time (2 ms) will not be impacted by the low band CA switching pattern.

- Tfirst-SSB is the time to first SSB transmission after MAC CE command is decoded by the UE. However, after sending the HARQ ACK, the SSB of the serving cell on which the active TCI state is to be switched may or may not be available at the UE on that serving cell due to the low band CA switching pattern. Therefore, TCI state switching delay should account for an extra delay due to unavailability of the SSB.

The availability of the SSB on the serving cell whose TCI state is to be activated depends on:

- Contents of the bitmap of the low band CA switching pattern, and

- Periodicity of the SSB

- Since both are configurable parmeters therefore the active TCI state switching delay (TMAC_known_TCI) for a known TCI state on a serving cell can be expressed as follows:

TMAC_known_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length      (1)

Where:

- TSSB the periodicity of the SSB occasion configured on the serving cell,

- LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_known_TCI.

- Other parameters are the same as described in clause 8.10.3.

MAC-CE based TCI state switching delay for unknown TCI state:

Clause 8.10.3 of TS 38.133 also defines requirements on the MAC-CE based TCI state switching delay for the case when the target TCI state is unknown to the UE as follows:

If the target TCI state is unknown, upon receiving PDSCH carrying MAC-CE for indication of UE-specific PDCCH TCI state as defined in clause 6.1.3.15 of TS 38.321 [7] in slot n, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ THARQ +3Nslotsubframe,µ + TL1-RSRP +TOuk*(Tfirst-SSB+ TSSB-proc) / NR slot length.

The active TCI state switching delay for the case when target TCI state is unknown incorporates an additional variable of TL1-RSRP, which is the measurement period of the L1-RSRP. The L1-RSRP can be measured on the SSB or on the CSI-RS, whichever is configured by the network. However, according to clause 8.10.3 of TS 38.133, TL1-RSRP = 0 in FR1. As the low band CA via switching is applicable to FR1 bands, therefore, the active TCI state switching delay (TMAC_unknown_TCI) for an unknown TCI state on a serving cell can be expressed similar to Equation (1) as follows:

TMAC_unknown_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length     (2)

Where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.3.

- Observation 1: During the MAC-CE based active TCI state switching procedure on a serving cell, SSB may not be available on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 1: The MAC-CE based active TCI state switching delay on a serving cell should account for an extra delay caused due to unavailability of the SSB on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 2: The MAC-CE based active TCI state switching delay (TMAC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TMAC_known_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_known_TCI.

  - Other parameters are the same as described in clause 8.10.3 of TS 38.133.

- Proposal 3: The MAC-CE based active TCI state switching delay (TMAC_unknown_TCI) on a serving cell when the target TCI state is unknown is expressed as follows:

TMAC_unknown_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.3 of TS 38.133.

## RRC based TCI state switching delay

RRC based TCI state switching delay for known TCI state:

Clause 8.10.5 of TS 38.133 defines requirements on the RRC based TCI state switching delay for the case when the target TCI state is known to the UE as follows:

If the target TCI state is known, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TOk*(Tfirst-SSB + TSSB-proc)) / NR slot length

In the above equation:

- TRRC_processing being the RRC processing delay, and TSSB-proc being the processing time (2 ms), will not be impacted by the low band CA switching pattern.

- Tfirst-SSB is the time to first SSB transmission after the RRC processing by the UE. However, after the RRC processing the SSB of the serving cell on which the active TCI state is to be switched may or may not be available at the UE on that serving cell due to the low band CA switching pattern. Therefore, TCI state switching delay should account for an extra delay due to unavailability of the SSB.

- As described above (section 2.2), the SSB periodicity and bit map contents are configurable parmeters. Therefore the active TCI state switching delay (TRRC_known_TCI) for a known TCI state on a serving cell can be expressed as follows:

TRRC_known_TCI = n+ (TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP)) / NR slot length    (3)

Where:

- TSSB the periodicity of the SSB occasion configured on the serving cell,

- LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_known_TCI.

- Other parameters are the same as described in clause 8.10.5.

RRC based TCI state switching delay for unknown TCI state:

Clause 8.10.5 of TS 38.133 defines requirements on the RRC based TCI state switching delay for the case when the target TCI state is unknown to the UE as follows:

If the target TCI state is unknown, UE shall be able to receive PDCCH with target TCI state of the serving cell on which TCI state switch occurs at the first slot that is after slot n+ (TRRC_processing +TL1-RSRP +TOuk*(Tfirst-SSB + TSSB-proc)) / NR slot length

Also, for the RRC based active TCI state switching delay when target TCI state is unknown incorporates an additional variable of TL1-RSRP, which is the measurement period of the L1-RSRP. The L1-RSRP can be measured on the SSB or on the CSI-RS, whichever is configured by the network. However, according to clause 8.10.5 of TS 38.133, TL1-RSRP = 0 in FR1.

As the low band CA via switching is applicable to FR1 bands, therefore, the active TCI state switching delay (TRRC_unknown_TCI) for an unknown TCI state on a serving cell can be expressed similar to Equation (3) as follows:

TRRC_unknown_TCI = n+ (TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP)) / NR slot length    (4)

Where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.5.

- Observation 2: During the RRC based active TCI state switching procedure on a serving cell, SSB may not be available on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 4: The RRC based active TCI state switching delay on a serving cell should account for an extra delay caused due to unavailability of the SSB on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 5: The RRC based active TCI state switching delay (TRRC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TRRC_known_TCI = n+ (TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP))/NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_known_TCI.

  - Other parameters are the same as described in clause 8.10.5 of TS 38.133.

- Proposal 6: The RRC based active TCI state switching delay (TRRC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TRRC_unknown_TCI =n+(TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP))/NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.5 of TS 38.133.

# Active BWP switching delay requirements

The requirements on the active BWP switching delay for single CC and multiple CCs are specified in clause 8.6 of TS 38.133. The RAN4 WF [2] has not identified any impact on these requirements on the active BWP switching delay.

The requirements on the DCI and timer-based BWP switching delay in clause 8.6.2 and RRC based BWP switch delay in clause 8.6.3 are also applicable to the UE configured in carrier aggregation (CA) as described in the introductory clause 8.6.1. Therefore, the requirements in clause 8.6.2 and clause 8.6.3 are also applicable to the UE configured with a low band CA switching pattern (LBCA-SwitchingPattern) in the CA operation.

However, as analysed below some clarification is needed in the active BWP switching delay requirements to account for the unavailability of the slots due to low band CA switching pattern.

## DCI and timer-based BWP switching delay

DCI-based BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For DCI-based BWP switch, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay + Y which starts from the beginning of DL slot n.

In response to the DCI command to switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

DCI-based dormant BWP switching delay:

Clause 8.6.2 of TS 38.133 defines requirements on the DCI based dormant BWP switching delay when the BWP switch (to a dormant BWP) is performed on a single CC (e.g., SDL SCell) as follows:

For DCI-based BWP switch, if the new BWP is a dormant BWP, after the UE receives BWP switching request at DL slot n on a serving cell, UE shall be able to receive CSI-RS (for DL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL slot occurs right after a time duration of TdormantBWPswitchDelay which starts from the beginning of DL slot n.

In response to the DCI command to switch to the dormant BWP on the SDL SCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the FDD PCell during the remaining part of the dormant BWP switch delay (TdormantBWPswitchDelay).

Timer-based BWP switching delay:

Clause 8.6.2 of TS 38.133 also defines requirements on the timer-based active BWP switching delay when the BWP switch is performed on a single CC (e.g., on PCell, or SCell) as follows:

For timer-based BWP switch, the UE shall start BWP switch at DL slot n, where slot n is the first slot of a DL subframe (FR1) or DL half-subframe (FR2) immediately after a BWP-inactivity timer bwp-InactivityTimer [2] expires on a serving cell, and the UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot occurs right after a time duration of TBWPswitchDelay which starts from the beginning of DL slot n.

In response to bwp-InactivityTimer expiry triggering the switch BWP on a serving cell (e.g., FDD PCell), the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the other serving cell (e.g., SDL SCell) during the remaining part of the active BWP switch delay (TBWPswitchDelay).

However, in all the above BWP switching scenario, the UE can only receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on a serving (e.g., PCell) cell provided that the slots are also assigned to that serving cell. When configured with LBCA-SwitchingPattern, a situation may arise that the “first slot” after the completion of the BWP in a serving cell is not available in that serving cell and is instead assigned/allocated to the other serving cell. In this case the UE may not be capable of transmitting PDSCH or receiving PUSCH in the first slot on the serving cell whose BWP is switched. To clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.2:

“If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the DCI or timer-based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which:

is available after the completion of the BWP switching and

is assigned to the same serving cell on which the active BWP switching occurs.”

- Observation 3: Due to low band CA switching pattern (i.e., LBCA-SwitchingPattern), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 4: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., LBCA-SwitchingPattern), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 7: Clarify the UE behavior by adding the following text in clause 8.6.2:

  - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching and

    - is assigned to the same serving cell on which the active BWP switching occurs.

## RRC based BWP switching delay

Clause 8.6.3 of TS 38.133 defines requirements on the RRC based BWP switching delay when the BWP switch is performed on a single CC (applicable to only SpCell) as follows:

For RRC-based BWP switch, after the UE receives RRC reconfiguration involving active BWP switching or parameter change of its active BWP, UE shall be able to receive PDSCH/PDCCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch occurs on the first DL or UL slot right after a time duration of TRRCprocessingDelay+TBWPswitchDelayRRCNR Slot length slots which begins from the beginning of DL slot n

In response to RRC message to switch BWP on the FDD PCell, the UE can perform baseband processing part of the BWP switching (e.g., SCS change, etc.) even if the slots are assigned to the SDL SCell during the remaining part of the RRC-based active BWP switch delay.

When the UE is configured with LBCA-SwitchingPattern, then a situation similar to the one described in section 3.1 (for DCI and timer-based BWP switching) may arise in that the “first slot” after the completion of the BWP in the FDD PCell is not available in the FDD PCell, i.e., available in the SDL SCell.

Therefore, to clarify the UE behavior and avoid misinterpretation of the requirements we propose to include the following text in clause 8.6.3:

“If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.”

- Proposal 8: Clarify the UE behavior by adding the following text in clause 8.6.3:

  - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.

# Conclusion

In this contribution we have analyzed RRM requirements related to active TCI state switching delay and active BWP switching delay when the UE is configured with low band CA switching pattern (LBCA-SwitchingPattern). Based on this analysis our observations and proposals are as follows:

MAC-CE based TCI switching:

- Observation 1: During the MAC-CE based active TCI state switching procedure on a serving cell, SSB may not be available on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 1: The MAC-CE based active TCI state switching delay on a serving cell should account for an extra delay caused due to unavailability of the SSB on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 2: The MAC-CE based active TCI state switching delay (TMAC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TMAC_known_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_known_TCI.

  - Other parameters are the same as described in clause 8.10.3 of TS 38.133.

- Proposal 3: The MAC-CE based active TCI state switching delay (TMAC_unknown_TCI) on a serving cell when the target TCI state is unknown is expressed as follows:

TMAC_unknown_TCI = n+ THARQ + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP) / NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TMAC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.3 of TS 38.133.

RRC based TCI switching:

- Observation 2: During the RRC based active TCI state switching procedure on a serving cell, SSB may not be available on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 4: The RRC based active TCI state switching delay on a serving cell should account for an extra delay caused due to unavailability of the SSB on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern).

- Proposal 5: The RRC based active TCI state switching delay (TRRC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TRRC_known_TCI = n+ (TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP))/NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_known_TCI.

  - Other parameters are the same as described in clause 8.10.5 of TS 38.133.

- Proposal 6: The RRC based active TCI state switching delay (TRRC_known_TCI) on a serving cell when the target TCI state is known is expressed as follows:

TRRC_unknown_TCI =n+(TRRC_processing + 3Nslotsubframe,µ+ TOk*(Tfirst-SSB + TSSB-proc +TSSB*LSP))/NR slot length, where:

  - TSSB the periodicity of the SSB occasion configured on the serving cell,

  - LSP is the number of SSB occasions not available at the UE on the serving cell due to the low band CA switching pattern (LBCA-SwitchingPattern) during TRRC_unknown_TCI.

  - Other parameters are the same as described in clause 8.10.5 of TS 38.133.

DCI or timer-based BWP switching:

- Observation 3: Due to low band CA switching pattern (i.e., LBCA-SwitchingPattern), after the BWP switching delay on a serving cell, a first DL or UL slot may not be available in that serving cell.

- Observation 4: If the first DL or UL slot is not available Due to low band CA switching pattern (i.e., LBCA-SwitchingPattern), the UE will not be able to transmit PDSCH or receive PUSCH in the first slot on the serving cell whose BWP is switched.

- Proposal 7: Clarify the UE behavior by adding the following text in clause 8.6.2:

  - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot refers to the first DL or UL slot, which:

    - is available after the completion of the BWP switching and

    - is assigned to the same serving cell on which the active BWP switching occurs.

RRC based BWP switching:

- Proposal 8: Clarify the UE behavior by adding the following text in clause 8.6.3:

  - If the UE is configured with LBCA-SwitchingPattern then the first DL or UL slot in the RRC based BWP switching delay requirements defined in this clause refers to the first DL or UL slot, which is available after the completion of the BWP switching and is assigned to the FDD PCell.

# References

- RP-251315, “Revised WID on Low NR band carrier aggregation via switching”, TELUS (Rapporteur).

- R4-2508259, “WF on RRM requirements for NR_LBCA_Sw”, Moderator (Apple).

- R1-2504869, “LS on Low NR band carrier aggregation via switching”, RAN1 LS to RAN2/RAN4.

- R4-2505556, “Topic summary for [115][207] NR_LBCA_Sw”, Moderator (Apple).