3GPP TSG-RAN WG4 Meeting #116	   	R4-250xxxx

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.8.5

Source: 	CMCC

Title:			  Discussion on RRM core requirements for R19 ATG

Document for:	Discussion

- Introductions

In last meeting, most of requirements have been discussed and defined for Air-to-ground network enhancement, the WF has been approved [1]. In this contribution, we discuss all remaining issues and target to close the core part of this WI.

- Discussion

Sharing factor between L1 measurement and RRM measurement - CSI-RS based L3 measurement

As we known, the intra-frequency CSI-RS based L3 measurement requirement including PSS/SSS detection time of associatedSSB, the time period used to acquire the SFN information and CSI-RS based measurement period without gap. Therefore, we should consider the overlapping between L1-RSs and both SMTC and CSI-RSs of intra-frequency neighbour cells.

Therefore, the Klayer1_measurement shall be updated to:

- For UE capable of [antennaArrayType-r18] on the measured carrier

Klayer1_measurement=1,

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions or CSI-RSs occasions of same serving cell

- Otherwise,

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions.

Proposal 1: For CSI-RS based L3 measurement, the Klayer1_measurement shall be defined as:

- For UE capable of [antennaArrayType-r18] on the measured carrier

Klayer1_measurement=1,

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions or CSI-RSs occasions of same serving cell

- Otherwise,

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions.

Sharing factor between L1 measurement and RRM measurement - P sharing factor

As analyzed above, we should consider the overlapping between L1-RSs and both SMTC and CSI-RSs of intra-frequency neighbour cells. Besides, the L3 neighbour cell measurement skipping shall also be considered.

Therefore, the Navailable of P sharing factor shall be also updated as:

Navailable for RLM, BFD and CBD of PCell is

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- otherwise,

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

Navailable for CBD of SCell, L1-RSRP and L1-SINR is

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

if inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of [two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap

otherwise,

- -      the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of other serving cell within the window W

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- Otherwise,

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

Proposal 2: For RLM, BFD and CBD of PCell, the Navailable in Psharingfactort shall be defined as:

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- otherwise,

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

- Proposal 3: For CBD of SCell, L1-RSRP and L1-SINR, the Navailable in Psharingfactort shall be defined as:

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap

otherwise,

- -      the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of other serving cell within the window W

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- Otherwise,

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

Issue 1-3-2: Sharing factor between L1 measurement and RRM measurement - effective non-overlapping duration

Proposals

- Proposal 1: For the ‘SSB-ToMeasure and SS-RSSI-Measurement are configured’, use MRTD as the reference when calculate the margin for non-overlapping duration

  - Proposal 1-1:

    - Consider impact on Klayer1_measurement due to 33usec MRTD for Case 4 antenna type, e.g.,

      - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbol before each consecutive SSB symbols and the RSSI symbols, and 2 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

  - Proposal 1-2:

    - For UE capable of [antennaArrayType-r18] and [intra-band or inter-band carrier aggregation is configured],

      - Klayer1_measurement=1,

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in the same band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

        - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency in different band outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 2 symbols before each consecutive SSB symbols and the RSSI symbols, and 2 symbols after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured and the UE is capable of [common Rx beam between PCC band and SCC band], where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement, or;

- Proposal 2:

  - Firstly align the understanding about the reference timing of SSB symbols and the RSSI symbols in the definition of Klayer1_measurement.

    - If the serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then extend the margin considering the maximum propagation delay difference between serving cell and neighbour cells.

      - If the neighbour cell timing is used as the reference timing of SSB symbols and the RSSI symbols, then reuse the legacy 1 symbol margin

- Proposal 3:

  - reuse the legacy specification, i.e.

    - if all of the reference signal configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap and fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols of serving cells when inter-band carrier aggregation within FR1 is configured and UE supports [capability of case 4], given that SSB-ToMeasure and SS-RSSI-Measurement are configured, where SSB symbols are indicated by the union set of SSB-ToMeasure from all the configured measurement objects on the same serving carrier which can be merged. and RSSI symbols are indicated by SS-RSSI-Measurement

In legacy FR2, Klayer1_measurement =1 when the L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the SSB symbols and the RSSI symbols, and 1 symbol before each consecutive SSB symbols and the RSSI symbols, and 1 symbol after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured. This was introduced from R15, and only FR2-1 SCS up to 240kHz was considered at that time.

In our understanding, the location of SSB symbols and the RSSI symbols of intra-frequency neighbour cells are determined by serving cell timing and the configured SSB-ToMeasure and SS-RSSI-Measurement. Thus, 1 symbol margin is considered including the maximum propagation delay difference（3µs for 1km cell coverage）and sync margin (3µs for FDD, 1.5µs for TDD) between serving cell and neighbour cells.

Observation 1: The serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols indicated by SSB-ToMeasure and SS-RSSI-Measurement.

The same logic shall be followed for FR1 ATG, that the margin shall consider the maximum propagation delay difference and sync margin between serving cell and neighbour cells. For the ATG scenario, the maximum propagation delay difference is about 1ms if we consider 300km ISD.

As a consequence, if we still want to introduce this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case, the margin shall be extend to at least 1ms, and the modification shall be done from R18, due to this is not the CA specific feature.

However, if companies think this case is not valid anymore with 2ms margin in total, then this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case shall be removed, the removal shall be done from R18, a CAT F CR is needed.

Proposal 4: Two ways for ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case

- Way 1: Introduce this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case.

  - Modify the effective non-overlapping duration to 1ms before each consecutive SSB symbols and the RSSI symbols, and 1ms after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured. The modification shall be done from R18.

- Way 2: Remove this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

For now we prefer Way 2, to make it align with scheduling restriction, that if deriveSSB-IndexFromCell is not enabled, the UE is not expected to receive PDCCH/PDSCH/TRS/CSI-RS for CQI on all symbols within SMTC window duration.

Proposal 5: Way 2 is preferred that remove ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

All the contents are stable except there is a [TBD] in the Note column which depends on the documentation of RF spec. Based on the endorsed draft CR R4-2507992 [3], the operating band for ATG inter-band CA will be captured in Table 5.2J.1A.2 in TS 38.101-1. Hence, we update the UE feature table accordingly.

Proposal 6: Update the Note column of Rel-19 ATG UE feature ‘Rx beam Type’ to Rel-19 UE feature list.:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_ATG_enh | 48-1 | Rx beam Type | Indicates whether the UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table 5.2J.1A.2 in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

- Conclusion

In this contribution, we further discuss the remaining issues about the definition of the sharing factor between L1 and L3 measurements. The observation and proposals are:

Proposal 1: For CSI-RS based L3 measurement, the Klayer1_measurement shall be defined as:

- For UE capable of [antennaArrayType-r18] on the measured carrier

Klayer1_measurement=1,

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions or CSI-RSs occasions of same serving cell

- Otherwise,

- -   if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting outside measurement gap are not fully overlapped by intra-frequency SMTC occasions and CSI-RSs occasions.

Proposal 2: For RLM, BFD and CBD of PCell, the Psharingfactort shall be defined as:

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- otherwise,

- -   the number of RLM/BFD-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

- Proposal 3: For CBD of SCell, L1-RSRP and L1-SINR, the Navailable in Psharingfactort shall be defined as:

If the measured carrier is the SCC with servingcellMO configured, and the network indication [skippingSCCneighbourCellMeas] is set to [‘enable’] to UE,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap

otherwise,

- -      the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of other serving cell within the window W

Otherwise,

If inter-band carrier aggregation within FR1 is configured, and

UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and [UE support two simultaneous separate Rx beams]

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions of same serving cell within the window W,

- Otherwise,

- -   the number of CBD/SSB/CSI-RS resource occasions that are not overlapped with any measurement gap occasion nor any SMTC occasion nor CSI-RSs occasions within the window W.

Observation 1: The serving cell timing is used as the reference timing of SSB symbols and the RSSI symbols indicated by SSB-ToMeasure and SS-RSSI-Measurement.

Proposal 4: Two ways for ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case

- Way 1: Introduce this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case.

  - Modify the effective non-overlapping duration to 1ms before each consecutive SSB symbols and the RSSI symbols, and 1ms after each consecutive SSB symbols and the RSSI symbols, given that SSB-ToMeasure and SS-RSSI-Measurement are configured. The modification shall be done from R18.

- Way 2: Remove this ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

Proposal 5: Way 2 is preferred that remove ‘L1-RSs fully-overlapped by intra-frequency SMTC occasions are not overlapped with any of the effective L3-RSs symbols’ case from R18.

Proposal 6: Update the Note column of Rel-19 ATG UE feature ‘Rx beam Type’ to Rel-19 UE feature list.:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 48. NR_ATG_enh | 48-1 | Rx beam Type | Indicates whether the UE supports one common Rx beam or two simultaneous separate Rx beams when UE capable of antennaArrayType-r18 on both PCC and SCC. |  | yes | N/A | If UE does not support the capability, network does not know ATG UE’s reception capability. | Per BC | No | FR1 only | N/A | This UE feature is applicable only for inter-band CA band combination(s) in Table 5.2J.1A.2 in TS 38.101-1. | Conditional mandatory for UEs supporting antennaArrayType-r18 on each band of the supported Band combination |

- Reference

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC