3GPP TSG-RAN WG4 Meeting #116	    	R4-2510023

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.13.4

Source: 	CMCC

Title: 		     Discussion on BS demodulation requirements for MMSE-IRC receiver

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed simulation assumptions for BS with MMSE-IRC receiver, the WF has been approved in [1]. In this contribution, we further discuss the test applicability and the interference covariance matrix estimation methodology.

- Discussion

MMSE-IRC receiver assumption - estimation of interference covariance matrix

- RAN4 agree that MMSE-IRC receiver assumption is not going to be captured in the RAN4 spec

- There is no consensus whether to modify the previously agreed MMSE-IRC receiver assumption.

The previous agreed MMSE-IRC receiver assumption is to use the DMRS REs to derive the interference covariance matrix. Considering the power boosting between DMRS RE and PUSCH RE, the SNR will be misaligned between DMRS and PUSCH.

Considering that whether the power boosting exists and how to configure the CDM group are all decided by gNB. Therefore, we think it is reasonable for gNB to use both all REs on DMRS symbol to derive the interference covariance matrix when CDM group 2 and -3dB power ratio of PUSCH EPRE to DM-RS EPRE are configured. We will further update our simulation results based on new MMSE-IRC algorithm.

Whether to consider additional applicability rule on how the test is performed

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

- Online agreement from RAN4#114bis:

  - Follow the LTE approach – Only specifies that HomNet requirements are not applicable for Local Area BS.

- FFS whether to consider additional applicability rule on how the test is performed:

- Option 1: If a BS declared to support Wide Area BS, Medium Range BS and pass the tests for HomNet, the test for HetNet can be skipped

- Option 2: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

For Wide Area BS and Medium Range BS, it may be deployed in both HomNet and HetNet scenario, therefore, we don’t support to further consider additional test applicability rule.

Follow the LTE approach,

For Wide Area BS and Medium Range BS, both HomNet and HetNet requirements are applicable.

For Local Area BS, only HetNet requirement is applicable.

Proposal 1: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

- Conclusion

In this contribution, we provides our opinion about test applicability. The proposal is:

Proposal 1: Not to consider additional applicability rule on how the test is performed for different test scenarios or BS classes.

- Reference

[1] R4-2508618, Way Forward for [115][322] NR_demod_Ph5_Part1_General_BS, China Telecom