3GPP TSG-RAN WG4 Meeting #116		R4-2510029

Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.23.4

Source: 	CMCC

Title: 			 Discussion on test cases for R19 NES

Document for:	Discussion

- Introduction

In last meeting, the discussion of performance part was started, and the WF has been agreed at[1]. According to the work plan [2], in this meeting, RAN4 targets to initially discuss test case list

- 3GPP RAN4 #116 meeting (Aug, 2025, 0.5TU, Perf part)

    - On-demand SSB(OD-SSB) SCell operation [RAN4]

  - Discussion on test scenarios

  - Initial discussion the related test case list

- Common signal/channel adaptation [RAN4]

  - Discussion on test scenarios

  - Initial discussion the related test case list

- On-demand SIB1 [RAN4]

  - Discussion the related test case list if has

In this contribution, we will provide our views on the test case list for R19 NES.

- Discussion

  - Test scope for OD-SSB

Issue 3-1-1: Scenario

RAN4 to discuss whether the following test scenarios can be agreed.

- RAN4 to define the OD-SSB based test case for both Case 1 and Case 2.

- RAN4 to test OD-SSB based Scell activation for scenario 2 and 2a.

- For Case 1, RAN4 should define the test cases considering scenario #2, scenario #2A and scenario #3B.

- For Case 1, RAN4 to define the OD-SSB based SCell activation test case for known SCell and unknown SCell.

- RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement.

- RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

- No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB.

- RAN4 to verify the EMP requirement in case 2 Alt Time-C1.

Issue 3-1-3: Test case list

RAN4 to discuss whether the following OD-SSB test cases can be agreed.

- SCell activation in Case 1

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based multiple SCell activation

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- Fast SCell Activation with OD-SSB;

- OD-SSB based SCell Activation with the L3 reporting during activation

- Intra-frequency measurement(deactivated SCell) in Case 1

- Intra-frequency measurement(deactivated SCell) in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

OD-SSB based deactivated SCell measurement

The fast measurement in FMW is the new measurement procedure introduced for OD-SSB based measurement. The measurement delay requirement and the interruption from FMW and OD-SSB activation/deactivation requirement shall be verified by test cases.

To verify the UE functionality with minimum test effort, we propose to test the Case 1 which can verify the OD-SSB periodicity based fast measurement and related interruption requirement, and the Case 2 Alt Time-C1 which can verify the EMP based fast measurement.

Proposal 1: Introduce OD-SSB based deactivated SCell measurement test case to verify the fast measurement requirement and interruption requirement due to FMW and OD-SSB activation/deactivation. The test case list shall at least including:

- OD-SSB based deactivated SCell measurement in Case 1 (interruption requirements are tested together).

- OD-SSB based deactivated SCell measurement in Case 2 Alt Time-C1.

OD-SSB based SCell activation

For MAC-CE triggered SCell activation, the OD-SSB based activation procedure, delay requirements and interruption requirements shall be tested. Both known and unknown case, measCycleSCell=160ms and 640ms shall be verified. Besides, The combination of Case#1, Case#2 Alt Time-C1, Case#2 Alt Time-C2, scenario 2, scenario 2A shall be covered.

To verify the UE functionality with minimum test effort, we propose to at least test the cases in the following list:

OD-SSB based unknown SCell activation in Case#1 Scenario#2A

OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2A (measCycleSCell=160ms)

OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2A (measCycleSCell=640ms)

OD-SSB based unknown SCell activation in Case#2 Alt Time-C1 Scenario#2A

OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2

OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2

Proposal 2: Introduce OD-SSB based SCell activation test case to verify the SCell activation delay requirements and interruption requirements, the test case list shall at least including:

- OD-SSB based unknown SCell activation in Case#1 Scenario#2A

- OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2A (measCycleSCell=160ms)

- OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2A (measCycleSCell=640ms)

- OD-SSB based unknown SCell activation in Case#2 Alt Time-C1 Scenario#2A

- OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2

- OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2

OD-SSB based Direct SCell activation

For Direct SCell activation, both Case#1 and Case2, known and unknown case shall be covered. we propose to at least test the cases in the following list:

OD-SSB based unknown Direct SCell activation in Case#1

OD-SSB based known Direct SCell activation in Case#2 Alt Time-C1 (measCycleSCell=640ms)

OD-SSB based known Direct SCell activation in Case#2 Alt Time-C2 (measCycleSCell=160ms)

Proposal 3: Introduce OD-SSB based Direct SCell activation test case to verify the SCell activation delay requirements, the test case list shall at least including:

- OD-SSB based unknown Direct SCell activation in Case#1

- OD-SSB based known Direct SCell activation in Case#2 Alt Time-C1 (measCycleSCell=640ms)

- OD-SSB based known Direct SCell activation in Case#2 Alt Time-C2 (measCycleSCell=160ms)

  - Test scope for OD-SIB1

Issue 3-2-1: Test case

- Proposals

  - Option 1: RAN4 not to define OD-SIB1 RRM performance test case.

  - Option 2: RAN4 to define OD-SIB1 RRM performance test case.

Issue 2-1-1: RRM requirements impacts for OD-SIB1 – paging interruption

Agreement:

- For core requirements: Clarify that the paging interruption may be longer when the target Cell is NES Cell

- Define test case with longer TSI-NR with reasonable value in performance part.

In last meeting’s core part discussion, RAN4 has agreed to introduce OD-SIB1 test case with longer TSI-NR

As we know, the UL-WUS and OD-SIB1 reception procedure will be contained in TSI-NR, we think the value of TSI-NR can be extended by one RAR window and one OD-SIB1 monitoring window. For RAR window, 10ms is the largest value for msg2 on licensed spectrum. For OD-SIB1 monitoring window, as designed by RAN2 signaling, the candidate value for od-sib1-WindowDuration-r19 contains 20ms, 40ms, 80ms, 160ms, and 320ms. Here, we propose to use 80ms as the reasonable value. After the extension, the TSI-NR shall be 1370ms =(1280+10+80)ms

Proposal 4: Introduce cell re-selection test case with UL-WUS for OD-SIB1. For the embedded paging interruption requirements, extend the TSI-NR to 1370ms = 1280+10+80ms.

  - Test scope for Signal adaptation

Issue 3-3-1: Test case list

RAN4 to discuss whether the following SSB adaptation test case list can be agreed.

- L1-RSRP

- L1-SINR

- CBD

- L3 measurement

- Deactivated SCell measurement

- normal SCell activation

- direct SCell activation

- PUCCH SCell activation

In high level, the test cases for L1 measurement requirements, L3 measurement requirements and normal SCell activation requirements when UE receives DCI 2-9 no later than SCell activation MAC-CE shall be introduced.

For L1 measurement, the L1-RSRP measurement serving cell testing, L1-SINR measurement testing and CBD for SCell testing shall be covered.

For L3 measurement, the intra-frequency measurement testing shall be covered.

Proposal 5: For SSB periodicity adaptation, at least introduce following testing

- L1-RSRP measurement serving cell testing

- L1-SINR measurement testing

- CBD for SCell testing

- Intra-frequency measurement testing, different test requirements before and after SSB adaptation

- Normal SCell activation testing when UE receives DCI2-9 no later than SCell activation MAC-CE

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: Introduce OD-SSB based deactivated SCell measurement test case to verify the fast measurement requirement and interruption requirement due to FMW and OD-SSB activation/deactivation. The test case list shall at least including:

- OD-SSB based deactivated SCell measurement in Case 1 (interruption requirements are tested together).

- OD-SSB based deactivated SCell measurement in Case 2 Alt Time-C1.

Proposal 2: Introduce OD-SSB based SCell activation test case to verify the SCell activation delay requirements and interruption requirements, the test case list shall at least including:

- OD-SSB based unknown SCell activation in Case#1 Scenario#2A

- OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2A (measCycleSCell=160ms)

- OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2A (measCycleSCell=640ms)

- OD-SSB based unknown SCell activation in Case#2 Alt Time-C1 Scenario#2A

- OD-SSB based known SCell activation in Case#2 Alt Time-C1 Scenario#2

- OD-SSB based known SCell activation in Case#2 Alt Time-C2 Scenario#2

Proposal 3: Introduce OD-SSB based Direct SCell activation test case to verify the SCell activation delay requirements, the test case list shall at least including:

- OD-SSB based unknown Direct SCell activation in Case#1

- OD-SSB based known Direct SCell activation in Case#2 Alt Time-C1 (measCycleSCell=640ms)

- OD-SSB based known Direct SCell activation in Case#2 Alt Time-C2 (measCycleSCell=160ms)

Proposal 4: Introduce cell re-selection test case with UL-WUS for OD-SIB1. For the embedded paging interruption requirements, extend the TSI-NR to 1370ms = 1280+10+80ms.

Proposal 5: For SSB periodicity adaptation, at least introduce following testing

- L1-RSRP measurement serving cell testing

- L1-SINR measurement testing

- CBD for SCell testing

- Intra-frequency measurement testing, different test requirements before and after SSB adaptation

- Normal SCell activation testing when UE receives DCI2-9 no later than SCell activation MAC-CE

- Reference

- R4-2508277, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson

- R4-2506909, Work plan for R19 NES enhancement performance part, Ericsson