3GPP TSG-RAN4 Meeting #116	R4-2510035

# Bengaluru, India, August 25th – 29th, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | Draft | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_Ph3-Core) draftCR on Introduce MDT requirements for RedCap UEs over NTN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-08-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - RAN4 is now discussing whether MDT requirements are needed to be introduced for RedCap UEs over NTN. If agreed to introduce, the corresponding requirements need to be captured. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Introduce MDT requirements for (e)Redcap UE over NR NTN |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec for RedCap UE over NR NTN will not be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3X |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# <Start of change#1>

4.3X	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access

4.3X.1	Introduction

The requirements in clause 4.3C.1 shall apply except that:

-	clause 4.3C is replaced with clause 4.3X, and

-	clause 4.3C.2 is replaced with clause 4.3X.2, and

-	clause 4.3C.3 is replaced with clause 4.3X.3, and

-	clause 4.3C.4 is replaced with clause 4.3X.4.

4.3X.2	Measurement Requirements

The requirements in clause 4.3C.2 shall apply except that:

The measurement values that are used to meet

- serving cell and reselection requirements as specified in clauses 4.2E.2.24.2E.2.11.

shall also apply to values logged for MDT measurements in RRC_IDLE state.

4.3X.3	Requirements for Relative Time Stamp Accuracy

The requirements in clause 4.3C.3 shall apply.

4.3X.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting

The requirements in clause 4.3C.4 shall apply.

4.3X.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting

The requirements in clause 4.3C.5 shall apply.

# <End of change#1>

# <Start of change#2>

5.3X	Minimization of Drive Tests (MDT) for NR RedCap UE with Satellite Access

5.3X.1	Introduction

The requirements in clause 5.3C.1 shall apply except that:

-	clause 5.3C is replaced with clause 5.3X, and

-	clause 5.3C.2 is replaced with clause 5.3X.2, and

-	clause 5.3C.3 is replaced with clause 5.3X.3, and

-	clause 5.3C.4 is replaced with clause 5.3X.4.

5.3X.2	Measurement Requirements

The measurements and measurement requirements applicable for MDT in RRC_INACTIVE are the same as specified for MDT in RRC_IDLE in clause 4.3X.2.

5.3X.3	Requirements for Relative Time Stamp Accuracy

The requirements for relative time stamp accuracy applicable for MDT in RRC_INACTIVE are the same as specified for MDT in RRC_IDLE in clause 4.3X.3.

5.3X.4	Requirements for Relative Time Stamp Accuracy for RRC Connection Establishment Failure Log Reporting

The requirements for relative time stamp accuracy for RRC connection establishment failure applicable for MDT in RRC_INACTIVE are the same as specified for MDT in RRC_IDLE in clause 4.3X.4.

5.3X.5	Requirements for Relative Time Stamp Accuracy for Radio Link Failure and Handover Failure Log Reporting

The requirements for relative time stamp accuracy for RRC link failure and handover failure applicable for MDT in RRC_INACTIVE are the same as specified for MDT in RRC_IDLE in clause 4.3X.5.

5.3X.6	Requirements for Relative Time Stamp Accuracy for RRC Resume Failure Log Reporting

The requirements for relative time stamp accuracy for RRC resume failure applicable for MDT in RRC_INACTIVE are the same as specified for MDT in RRC_IDLE in clause 4.3X.4.

# <End of change#2>