3GPP TSG-RAN WG4 Meeting #116			R4-2510036

# Bengaluru, India, August 25th – 29th, 2025

Agenda Item:	7.27.5.2

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3 DL CE

Document for:	Discussion

- Introduction

In last meeting, RAN4 mainly discussed the extension of SMTC number and SMTC periodicity and some other issues. The WF has been approved in [1]. In this contribution, we further discuss the open issues.

- Downlink coverage enhancements

## Issue 0: LS from RAN2 on multiple SMTCs with different periodicities

Agreement: (Ad-hoc)

From RAN4 perspective, it is feasible to support configuring more than 4 SMTCs per frequency (i.e. 6) for idle/inactive UEs.

- In the reply LS (to be prepared by Xiaomi), the following information highlighted in yellow for both intra- and inter-frequency will be added for reference:

- FFS on the exact RRM requirement change.

- FFS whether to limit the number of SMTCs to no more than 4 is desirable in an earth-moving cell deployment

Agreement: online

From RAN4 perspective, it is feasible to support SMTCs with up to 2 periodicities and different offsets but requires some clarifications/modifications to the existing RRM requirements.

- RAN4 assumes that the configured SMTC periodicity is equal to or greater than the largest SSB periodicity among the cells associated with the relevant SMTC on the frequency layer.

Agreement: online

The progress of RRM requirement definition for multiple SMTCs with up to two periodicities does not block the completion of RAN4 RRM core part of this WI.

After examining the current spec, we identify some requirements which may be impacted by SMTC configuration, we give a list below for quick view:

| Cell re-selection requirement | M1/M2/M3/M4 value |
| --- | --- |
| Handover | Trs |
| RRC Re-establishment | TSMTC/TSMTC,i |
| RRC Connection Release with Redirection | Trs |
| Connected intra/inter-frequency measurement | Kmulti_SMTC K_satellite Kp Klayer1_measurement Kgap |

Then we look the related requirements in detail.

In RRC_IDLE and Inactive mode, maximum 6 SMTCs per frequency can be configured, and two different SSB periodicities with different offsets are supported among the configured SMTCs.

Cell re-selection - serving cell measurement and evaluation

For DRX cycle based cell selection requirement, only scaling factor M1/M2/M3/M4 will be impacted by SMTC number and periodicity.

M1 is the scaling factor for serving cell measurement and evaluation to guarantee the UE power saving gain in short DRX cycle. In current spec,

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  4,

-	otherwise M1=1.

With NSMTC>4, and up to 2 SMTC periodicities, RAN4 shall discuss whether the M1 value shall be further relaxed.

As we know, UE will not use more SMTCs/Gaps in parallel than in previous releases, which means even though network configure 6 SMTCs, the SMTC number used at UE side will not be changed, no additional power consumption due to the increased SMTC number configuration. Therefore, M1 will not be further relaxed due to the number of SMTCs configured by SAN.

Regarding up to 2 SMTC periodicities, when determine the M1 scaling factor, the maximum periodicity shall be considered. Therefore, we propose following:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  6,

-	otherwise M1=1.

Proposal 1: For serving cell measurement and evaluation requirement, M1 parameter will be impacted by the SMTC enhancement scheme introduced by RAN2, which should be updated as:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  6,

-	otherwise M1=1.

Cell re-selection - inter/intra-frequency measurement and evaluation

For DRX cycle based cell re-selection requirement, scaling factor M2/M3/M4 will be impacted by SMTC number and periodicity.

M2/M3/M4 is the scaling factor to guarantee UE power saving gain in longer SMTC period. In current spec,

For basic measurement

-	M2 = 2 if SMTC periodicity of measured intra-frequency cell > 20 ms and 1<NSMTC  4 upon more than 1 SMTC configured at the UE; M2 = 1.5 if SMTC periodicity of measured intra-frequency cell > 20 ms and NSMTC=1 upon 1 SMTC configured at the UE; otherwise M2=1. Where, NSMTC is the number of SMTCs configured by SAN. If different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified. During PSS/SSS detection, the periodicity of the SMTC configured for the intra-frequency carrier is assumed, and if the actual SSB transmission periodicity is greater than the SMTC configured for the intra-frequency carrier, longer Tdetect, NR_intra is expected.

For high-speed measurement

-	When SMTC < = 40 ms, M2 = M3 = M4 = 1; and when SMTC > 40 ms, M2 = 1.5, M3 = M4 = 2

For M2 factor in basic measurement, when network configure the number of SMTC larger than 4, measurement period longer than the corresponding measurement period specified is expected. Therefore, no need to further extend the M2 due to the number of SMTCs configured by SAN. Meanwhile, we can see that the M2 value has already related to SMTC periodicity per cell, so no need to extend the M2 due to up to 2 SMTC periodicities.

For M2/M3/M4 factor in high-speed measurement, we think the same logic as basic measurement can be used, that no need to extend the due to the number of SMTCs configured by SAN, but the SMTC periodicity shall related to SMTC periodicity used by cell being identified.

Proposal 2: Update the Note of M2/M3/M4 in high-speed measurement requirements that if different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

Handover & RRC Connection Release with Redirection

For the Handover interruption and RRC Connection Release with Redirection delay requirements, the definition of Trs is as follows:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (The remaining unrelated parts are skipped)

However, in RRC Connected mode, RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for neighbour cell measurement, which means when UE has not been provided with an SMTC configuration for target cell, there may be two SMTC periodicities in the MO which having the same SSB frequency and subcarrier spacing.

Therefore, the definition of Trs shall be updated. In the aforementioned case, we think the larger SMTC periodicity shall be used as default for target cell search.

Proposal 3: For handover interruption and RRC Connection Release with Redirection delay requirements, update the definition of Trs as:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (Remaining unrelated parts are skipped)

RRC Re-establishment

For the RRC Re-establishment delay requirements, the definition of TSMTC/TSMTC,i is as follows:

-	TSMTC: It is the periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

However, in RRC Connected mode, RAN2 considers to support configuring two different SMTC periodicities (with different offsets) in one frequency layer for neighbour cell measurement, which means for intra-frequency carrier and inter-frequency carrier, there may be two SMTC periodicities from the SMTC configuration

Therefore, the definition of TSMTC/TSMTC,i shall be updated. In the aforementioned case, we think the larger SMTC periodicity shall be used as default for conducting measurement.

Proposal 4: For RRC Re-establishment delay requirements, update the definition of TSMTC/TSMTC,i as:

-	TSMTC: It is the maximum periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the maximum periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

Intra/inter frequency measurement

Regarding the scaling factor Kmulti_SMTC/K_satellite for measurement of multiple SMTCs or multiple satellites, currently, the calculation method doesn’t consider the multi SMTC periodicities scenario. For example, 2 SMTC with different periodicity configuration may not always be collided during a single Tidentify or TSSB_measure.

We think same methodology from RRC_IDLE mode can be reused, to add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Proposal 5: For the the scaling factor Kmulti_SMTC/K_satellite for intra/inter frequency measurement, add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Regarding the scaling factor Kp, Klayer1_measurement, and Kgap, we think the definition shall be updated to per target cell level. Existing method as in Table 9.2C.5.2-1 Note 1 can be reused, that add a note to give following further clarification: For calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 6: For the the scaling factor Kp, Klayer1_measurement, and Kgap for intra/inter frequency measurement, add a note that for calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

## Issue 1-2-A: RRC Re-establishment (default TSMTC,i)

Agreement from RAN4#114

- In RRC re-establishment requirements, update is needed to the TSMTC,i value for case where the SMTC for the inter-frequency carrier is not provided.

FFS from RAN4#114

- In RRC re-establishment requirements, update the value of TSMTC,i to 160ms for case where the SMTC for the inter-frequency carrier is not provided.

  - FFS: whether the above will apply only for UEs capable of coverage enhancement feature, if such a UE capability is defined.

Agreement from RAN4#114bis (online):

- In RRC re-establishment requirements, for case where the SMTC for the inter-frequency carrier is not provided, the value of TSMTC,i is assumed to be 160ms:

  - Discuss the applicability of the requirement for with the updated value of TSMTC,i to 160ms after the related RAN1/2 agreement.

FFS from RAN4#114bis: Wait for RAN2 progress and discuss the following

- The requirements for intra-frequency and inter-frequency RRC re-establishment are applicable provided that the configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell.

No discussion in RAN4#115:

- Due to the potential enhancement of the SMTC configuration in Issue #0, companies' views on this matter appeared to be diverse.

In RAN4#114bis, companies brought the applicability rule that the requirements are applicable provided that the  configured SMTC period is not shorter than the periodicity of the actual SSB transmission of the target cell. This come from one SMTC period may be mapping to two SSB periodicity, e.g., SMTC period 20ms, and SSB periodicity 20ms and 160ms. However, after RAN4#115, due to RAN2 has design the SMTC enhancement methodology, up to 2 SMTC periodicities can be configured to UE, the mismatch issue under DL-CE scenario can be addressed. We think there is no need to further discuss the FFS above.

Proposal 7: No need to define the applicability rule about the relationship between configured SMTC period and the periodicity of the actual SSB transmission of the target cell.

Currently, RAN1 and RAN2 hasn’t achieve any UE capability about DL CE feature. Therefore, RAN4 may consider that apply default TSMTC,i to 160ms for all UEs or sent a LS to ask the necessity of the UE capability.

Proposal 8: Regarding the applicable range of default TSMTC,i = 160ms, RAN4 should consider either apply the default value for all UEs, or sent a LS to ask the necessity of the UE capability.

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: For serving cell measurement and evaluation requirement, M1 parameter will be impacted by the SMTC enhancement scheme introduced by RAN2, which should be updated as:

-	M1=2 if SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and NSMTC =1, upon one SMTC configured at the UE,

-	M1=2.5 if at least one configured SMTC periodicity (TSMTC) > 20 ms and DRX cycle ≤ 0.64 second and 1<NSMTC  6,

-	otherwise M1=1.

Proposal 2: Update the Note of M2/M3/M4 in high-speed measurement requirements that if different SMTC periodicities are configured for different cells, the SMTC periodicity in this note is the one used by the cell being identified.

Proposal 3: For handover interruption and RRC Connection Release with Redirection delay requirements, update the definition of Trs as:

-	Trs is the SMTC periodicity of the target NR SAN cell if the UE has been provided with an SMTC configuration for the target cell in the handover/redirection command, otherwise Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. (Remaining unrelated parts are skipped)

Proposal 4: For RRC Re-establishment delay requirements, update the definition of TSMTC/TSMTC,i as:

-	TSMTC: It is the maximum periodicity of the SMTC occasion configured for the intra-frequency carrier. If the UE has been provided with higher layer in TS 38.331 [2] signalling of smtc2, Tsmtc follows smtc1 or smtc2 according to the physical cell ID of the target cell.

-	TSMTC,i: It is the maximum periodicity of the SMTC occasion configured for the inter-frequency carrier i. If it is not configured, the UE may assume that the target SSB periodicity is no larger than 20 ms.

Proposal 5: For the the scaling factor Kmulti_SMTC/K_satellite for intra/inter frequency measurement, add a note that for deriving Kmulti_SMTC /K_satellite for Tidentify and TSSB_measure, two SMTCs are considered as overlapping if they overlap in one or more occasions during a single Tidentify or TSSB_measure.

Proposal 6: For the the scaling factor Kp, Klayer1_measurement, and Kgap for intra/inter frequency measurement, add a note that for calculation of Kp, Klayer1_measurement, and Kgap, if different SMTC periodicities are configured for different cells, the SMTC period in the requirement is the one used by the cell being identified.

Proposal 7: No need to define the applicability rule about the relationship between configured SMTC period and the periodicity of the actual SSB transmission of the target cell.

Proposal 8: Regarding the applicable range of default TSMTC,i = 160ms, RAN4 should consider either apply the default value for all UEs, or sent a LS to ask the necessity of the UE capability.

- Reference

- R4-2508288, WF on RRM requirements for NR_NTN_Ph3_Part2, Moderator (Qualcomm Incorporated)