# 3GPP TSG-RAN WG4 Meeting #116	R4-2510084

# Bengaluru, India, August 25th – 29th, 2025

Agenda item:	7.14.6

Source:	China Telecom

Title:	Discussion on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR

Document for:	Discussion

- Introduction

In RAN4 #115 meeting, RRM performance requirements of fast SCell activation for UE supporting Rel-18 EMR were discussed and the WF was approved [1].

In this paper, we present our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR captured in the WF [1] and summary [2].

- Discussion

Status in the WF [1] and summary [2]:

### Issue 2-2-2: Details of test case Further discuss this issue. - Proposals - Option 1: (OPPO) - The test can consist of four successive time periods, with duration T1, T2, T3 and T4, respectively. - T1 for IDLE mode measurement, T2 for transition to connected mode, T3 for SCell configuration and T4 for SCell activation. - For direct SCell activation, T3 and T4 can be merged into one. - The test configurations of T1 and T2 can use the unknown case in legacy cell reselection (A.6.1.1/A.7.1.1) or IDLE mode CA/DC measurement (A.6.6.9/A.7.6.23) as baseline. - The test configurations of T3 and T4 can use the unknown case in legacy SCell activation (A.6.5.3) as baseline. - Option 2: (Apple) - Basic principle is to reuse the legacy test cases as much as possible. For instance, RAN4 can add SCell configuration and activation procedure on top of R18 eEMR test cases. - Option 3: (CATT) - Reuse the contents in the legacy test cases for Rel-18 eEMR measurement with valid result reporting and SCell activation. - Time gap between early measurement report transmission and SCell activation command reception needs to be larger than the time duration specified in known conditions: - For target SCell in FR1, time gap needs to be larger than max(5*measCycleSCell, 5*DRX cycles); - For target SCell in FR2, time gap needs to be larger than4 s for UE supporting power class 1/5 and 3 s for UE supporting power class 2/3/4. - Option 4: (Ericsson) - The performance of Rel-19 fast SCell activation for UEs supporting Rel-18 EMR can be evaluated by combining the existing SCell activation test case with the Idle state fast CA/DC reporting test case such as take table.2 as test configuration. - Recommended WF - Discuss the options.

The requirements on Fast SCell activation for UE supporting Rel-18 EMR are based on the requirements on Rel-18 EMR and legacy SCell activation, thus the test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

Proposal 1: The test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

- Conclusion

The contribution presents our views on RRM performance requirements of Fast SCell activation for UE supporting Rel-18 EMR, with the following proposals:

Proposal 1: The test cases for Rel-18 EMR measurement with valid result reporting and SCell activation can be the baseline to design the test cases for Fast SCell activation for UE supporting Rel-18 EMR.

- References

- R4-2508266, WF on RRM requirements for NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #115, 19 – 23 May, 2025.

- R4-2505562, Topic summary for [115][213] NR_RRM_Ph5_Part2, CATT, RAN WG4 Meeting #115, 19 – 23 May, 2025.