3GPP TSG-RAN WG4 Meeting # 116                                        R4-2510099

Bengaluru, India, Aug. 25 – 29, 2025

Agenda Item: 7.25.3.1

Source: MediaTek Inc.

Title: Discussion on event triggered L1 report for LTM

Document for: Discussion

# Introduction

RAN#107 agreed on the revised WID “NR mobility enhancements Phase 4” [1]. According to WID, RAN4 is supposed to define requirements for the following measurement enhancements:

- Event triggered L1 report,

- CSI-RS L1 measurements,

- CSI acquisition on candidate cell(s) based on CSI-RS before or during LTM cell switch.

Last meeting, there were some discussions [2]. In the next, we would like to provide our views on the open issues of event triggered L1 report.

# Discussion

## 2.1 Capabilities for Support of Event Triggering L1 Reporting

#### Issue 2-3: whether to introduce/update requirements for reporting criteria per measurement category Candidate options: - Option 1: Define a separate capability for supported number of events triggered L1 measurement reporting. (CTC, Nokia, Xiaomi, CATT, vivo, MTK, ZTE, CMCC, E///) - Details: - Option 1a:(vivo) - RAN4 to specify the reporting capabilities for SSB-based L1 measurements for LTM by considering intra-frequency measurement and inter-frequency measurement. For CSI-RS based L1 measurements for LTM, consider 1 layer of measurement for the reporting capability. - Reporting criteria for LTM event triggered reporting is 4 per frequency layer. - The total number of reporting criteria for LTM event triggered reporting is 4*n, where n is the total number frequency layers for L1-RSRP measurement, including intra-frequency layer(s), inter-frequency layer(s) for SSB-based L1-RSRP measurement and 1 frequency layer for CSI-RS based L1-RSRP measurement if configured. - Option 1b: E/// - RAN4 to support a common LTM Ecat capability to be shared between intra- and inter-frequency candidate cells. - RAN4 to agree that Ecat capability for LTM L1 measurements includes both PCell event, and candidate cells’ events. The detail number of Ecat is FFS - Option 2: update the existing requirements to cover L1 event triggered reporting criteria per measurement category. (OPPO) - Option 3: assuming new UE capabilities regarding support of event triggering needs to be introduced, RAN4 shall discuss whether it is still necessary to update/introduce new Event Triggering and Reporting Criteria for LTM. (Apple)

In our understanding, the goal of event triggered L1 measurement reporting is to replace L3 report and used to trigger LTM. With this understanding, for the frequency layers configured with event triggered L1 measurement reporting, it is not necessary to have event triggered L3 measurement reporting.

But on the other hand, NW may configure and adjust event triggered L1 measurement reporting based on L3 report. Then there will be both L3 event and L1 event for the same frequency layer. We think it is reasonable to introduce extra UE capability to support event triggered L1 measurement reporting.

Regarding Option 3, checking the capabilities agreed or in discussion for event triggered L1 measurement reporting, it seems there is only one capability related in discussion as shown below.

RAN1#120bis Agreement Regarding the LS from RAN2 (R1-2501686) the following contents are included in the reply LS. - RAN1 assumes at maximum 16 beams can be reported by a single event triggered reporting - Regardless whether or not the report includes the current beam - Note: The maximum number of beams that can be reported by a UE subject to UE capability is under discussion in RAN1

For L3 measurement, the events for intra-frequency and inter-frequency are viewed as different events. It is straightforward for L3 measurement, as each report configuration is linked to a measObject which is for a single frequency layer. Take UE configured with SA operation mode as an example, the total number of NR reporting criteria is , where n is the number of configured NR serving frequencies, including PCell, and SCells carrier frequencies. In FR2, for each frequency layer, the UE shall be capable of performing L3 measurements for at least 6 identified cells. So UE is supposed to be able to compare 6 x (10 + 9 x n) values with the configured thresholds.

But for L1 measurement and report, according to RAN1/2 conclusion, L1 events will be configured under “LTM-CSI-ReportConfig-r18”. Each LTM-CSI-ReportConfig is linked to a LTM-CSI-SSB-ResourceSet. The maximum number of LTM-CSI-ReportConfig is maxNrofLTM-CSI-ReportConfigurations-r18, which is 48. At most 512 resources (maxNrofLTM-CSI-SSB-ResourcesPerSet-r18=512) can be linked with each LTM-CSI-ReportConfig-r18. If there is no additional limit on the number of L1 events UE supports, the total number of values that UE is supposed to compare with the configured thresholds will be “maxNrofLTM-CSI-ReportConfigurations” * “maximum number of beams that can be reported can be reported by a single event triggered reporting”, i.e., 48 * the above capability may be introduced by RAN1. If UE wants to report 16 beams per event triggered reporting, then the complexity is roughly 48*16/(6*(10+9n)) times of L3 events, which is about 14 times for non-CA case. In one L1 event configuration, the SSB/CSI-RS to measure can be from any LTM candidate cell (intra-f or inter-f) and NW can configure different offsets for each candidate (using the IE candidateSpecificOffset-r19). It means that the signalling design is flexible enough to configure a particular LTM event for all candidates in a single L1 event configuration. There are only four types of L1 events (Event LTM2,3,4,5) in total. We don’t think whether NW would configure 48 (maxNrofLTM-CSI-ReportConfigurations) different events. So it is better to introduce a limit on the total number of L1 events that NW can configure. We propose the candidate values as {4, 8, 12, 16}.

For information: maxNrofLTM-CSI-ReportConfigurations-r18 INTEGER ::= 48 -- Maximum number of LTM CSI reporting configurations maxNrofLTM-CSI-SSB-ResourcesPerSet-r18 INTEGER ::= 512 -- Maximum number of LTM CSI SSB resource per set

Proposal 1: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. The candidate values are {4, 8, 12, 16}.

## 2.2 Impact on other LTM requirements

Agreement: - For FR1: - UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, - Case 1: leaving condition of the event is configured and corresponding condition is not met, - the related cell switch delay and TCI state activation requirements are applicable - Case 2: leaving condition of the event is NOT configured, FFS - For FR2: FFS

In the past two meetings, RAN4 discussed the impact of event-triggered L1 report on other LTM requirements and reached the above agreement.

From UE’s point of view, as long as the channel condition is not bad and UE can keep measuring the cell or beam, the cell and TCI state can be known to UE. For NW controlled LTM, it is also important to let NW know whether the cell/TCI state is known. The way to let NW know is through reporting or not reporting (leaving condition is configured and corresponding condition is not met). For Case 2, we don’t know how to let NW know whether the cell/TCI state is known or not. So, we prefer not to do further optimization for the case that leaving condition of the event is NOT configured.

In FR2, when NW activates some TCI state(s), UE will prioritize L1-RSRP measurement on those TCI state(s) and may never measure the beam which is not activated. So UE may stop measuring the beam if NW activates some other TCI state(s) after event-triggered L1 report. In this case, the above optimization seems not workable.

Proposal 2: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases.

# Summary

In this paper, we provide some views on event triggered L1 report in R19 mobility. We have the following proposals:

Proposal 1: Introduce reported UE capabilities for supported number of events triggered L1 measurement reporting. The candidate values are {4, 8, 12, 16}.

Proposal 2: For FR1, if UE has sent an event-triggered L1 report and the reporting condition continues to be met at the time of receiving the cell switch command or TCI state activation command, leaving condition of the event is configured and corresponding condition is not met, the related cell switch delay and TCI state activation requirements are applicable. Not to extend the optimization to other cases.

# 4 Reference

RP-250339, Revised Work Item: NR mobility enhancements Phase 4, Apple Inc, China Telecom.

R4-2508282, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple, RAN4#115.