3GPP TSG-RAN WG4 Meeting #116																			R4-2510117

Bengaluru, India, 25 -29 August 2025

Agenda item:	8

Source: 	Qualcomm

Title: 	Proposal for Rel-19 FR1 intra-band UL CA with single CC activated cell to support Rel-18 power boosting

Document for:	Approval

# 1. Introduction

This contribution includes several proposals for RAN4 UE features for Rel-19 NR.

For a detailed description of the following changes please refer to R4-2509199

- NR_ENDC_RF_Ph4

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 46. NR_ENDC_RF_Ph4 | 46-1 | MPR enhancement for activated carrier | Indicates whether UE supports reduced MPR for single CC if single CC is activated for intra-band UL contiguous CA. If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applicable to the activated CC |  | Yes | N/A | The FR1 UE shall support MPR requirement based on UL CA configuration even for single activated CC. | Per FS | No | FR1 only | N/A |  | Optional with capability signaling |
|  | 46-4 | MPR reduction for single carrier with single value | Indicates whether UE supports MPR reduction for single UL carrier with 1/2*[UE CBW] extension for dual-sided symmetric and single-sided case If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |
|  | 46-5 | MPR reduction for single carrier with multiple values | [Indicates whether UE supports MPR reduction for single UL carrier with 1/X*UE CBW extension for dual-sided symmetric and single-sided case, where 1/X is a set of ratios equal and smaller than 1/2 [{1/4, …}].] If the UE also supports powerBoosting-pi2BPSK-QPSK-r18 and/or powerBoosting-pi2BPSK-QPSK-Modified-r18, then these capabilities are applied to the MPR constructs defined for the Rel-19 capability |  | Yes | N/A | The FR1 UE shall support MPR requirement in existing spec without enhancement. | Per FS | No | FR1 only | N/A | This capability is applicable for single UL CC case | Optional with capability signaling |