3GPP TSG-RAN WG4 Meeting#116		R4-2510118

Bengaluru, India, Aug 25th -29th, 2025

Agenda item:	7.22.2

Source: 	Spreadtrum, UNISOC

Title: 	Discussion on system parameters for ambient IoT

Document for:	Discussion

Introduction

In RAN4#115 meeting, a WF [1] was agreed. This contribution continues to discuss the remaining open issues as follows.

- Topic 2-2: D2R bandwidth Issue 2-2-1: D2R bandwidth Agreement in RAN4#115: - Define D2R CBW based on all Tb, Tc, R combinations as finally approved in RAN1. - Assumed 10% SFO for device - For Reader, [90%] filter spectrum utility ([10%] guard band) is considered. - The value of transmission BW and channel BW will be explicitly listed. - Using following equation for BS as starting point: D2R CBW for BS (KHz)=(2SB Transmission BW*(1/2)+2* Small frequency shift)/0.9 =(2000*(1+R)/Tb)* (1+∣SFO∣)/0.9 =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣)/0.9 - Define [200kHz]，[3.54MHz] D2R CBW for AIoT BS. Other D2R CBWs for BS are FFS - Using following equation for device as starting point: D2R CBW for device (KHz)=2SB Transmission BW*(1/2)+2* Small frequency shift =(2000*(1+R)/Tb)* (1+∣SFO∣) =(1000*(R+1)/(Tc*R)) * (1+∣SFO∣) - The following are for information: - Figure 1-1 Small frequency shift diagram of FDMA without SFO - Figure 1-2. SFO impact of transmission bandwidth and small frequency shift of FDMA - As the figures above indicate, SFO not only affects the D2R transmission bandwidth but also scales the small frequency shift value . - The 2SB transmission BW and small frequency shift _both measured in kHz——with and without SFO are as follows: - Note : BW and Small frequency shift are in kHz;Tc and Tb are in μs 2SB Transmission BW (kHz); Small frequency shift (kHz). Without SFO; With SFO; Without SFO; With SFO. Btx,D2R_ without SFO= 4*1000/Tb=2*1000/(R*Tc); Btx,D2R_ with SFO =Btx,D2R_ without SFO×(1±∣SFO∣); SFSwithout SFO=±1000*R/Tb= ±1000/(2*Tc); SFSwith SFO =SFSwithout SFO×(1±∣SFO∣).. - Topic 2-3: Channel raster Issue 2-3-1: R2D channel raster Agreement in RAN4#115: - Define 10KHz channel raster - FFS on channel raster offse

Discussion

R2D channel raster

The transmission bandwidth configuration of R2D is 1PRB, 2PRB, 3PRB and 4PRB with 15 kHz SCS. According to the last meeting agreement, the R2D channel raster is 10 kHz. Whether to define channel raster offset value or not needs to be further discussed.

In our understanding, we need to define the location of A-IoT carrier frequency first. Based on TR 37.824, when NB-IoT is operating in NR in-band, the NB-IoT carrier frequency is referring to a frequency located in between 2 NB-IoT tones. NR RF reference frequency is referring to a frequency located in the middle of a tone. To align sub-carriers, considering NR 15 kHz SCS, a 7.5 kHz offset shall therefore be considered in between reference frequencies. From our perspective, the purpose of the location of NB-IoT carrier location is to avoid the LO leakage. However, the transmission of R2D comes from A-IoT BS, maybe we don’t need to consider the LO leakage. In addition, to simplify the operation and the definition of A-IoT, we can define the location of A-IoT carrier frequency as similar as NR.

Figrue1: NB-IoT carrier location

Figure2: NR RF reference frequency position

Proposal 1: Define the location of carrier frequency of A-IoT as same as NR in Table 1.

Table 1: Channel raster to resource element mapping

|  | NRBmod2 = 0 | NRBmod2 = 1 |
| --- | --- | --- |
| Resource element index | 0 | 6 |
| Physical resource block number |  |  |

To align sub-carriers, considering NR 15 kHz SCS, we define the R2D channel raster as 10kHz, and reuse the same definition of carrier frequency for A-IoT, there is no need to define R2D channel raster offset value.

Proposal 1: Define R2D channel raster as 10 kHz, there is no need to define R2D channel raster offset value.

D2R bandwidth

In the last meeting, an agreement was agreed to define D2R channel bandwidth for A-IoT device based on all Tb, Tc, R combinations as finally approved in RAN1. According to RAN1’s agreement as follow.

Agreement For D2R transmission with small frequency shift +/- R/Tb Hz, where time duration Tb corresponding to a bit (that is after FEC if applied) and R = Tb/(2 × D2R chip length): - the transmitted 2R chips for bit 1 and bit 0 are [0 1 0 1 …] and [1 0 1 0 …], respectively, for OOK modulation - the transmitted 2R chips for bit 1 and bit 0 are [-1 +1 -1 +1 …] and [+1 -1 +1 -1 …], respectively, for BPSK modulation. - Note: The case of R=1 is equivalent to using a Manchester line code without repeating each Manchester codeword.

Agreement For D2R transmission, - The minimum Tb is 1.39μs. - Support at least the following values of Tb, Tchip, and R from the table agreed in RAN1#120bis. Tchip (μs). Tb (μs). 133.33; 66.67; 33.33; 16.67; 11.11; 8.33; 5.56; 4.17; 2.78; 2.08; 1.39; 1.04; 0.69; 0.52. 266.67; R=1; R=2; R=4; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128; R=256. 133.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64; R=96; R=128. 66.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32; R=48; R=64. 33.33; R=1; R=2; R=4; R=6; R=8; R=12; R=16; R=24; R=32. 22.22; R=1; R=2; R=4; R=8; R=16. 16.67; R=1; R=2; R=4; R=6; R=8; R=12; R=16. 11.11; R=1; R=2; R=4; R=8. 8.33; R=1; R=2; R=4; R=6; R=8. 5.56; R=1; R=2; R=4. 4.17; R=1; R=2; R=4. 2.78; R=1; R=2. 2.08; R=1; R=2. 1.39; R=1. 1.04; R=1.

Based on the above agreement and information, transmission bandwidth (2SB) is equal to 4/Tb, if we consider SFO impact, the transmission bandwidth is 4/Tb*(1± SFO); small frequency shift is +/- R/Tb Hz, similarly, if we consider SFO impact, small frequency shift is +/- R/Tb* (1± SFO). SFO is 10 %.

When R=1, Channel bandwidth=4/Tb*(1+ SFO) =1.1*4/ Tb

When R>1, Channel bandwidth = 2*[(R/Tb)*(1+SFO)+ 1/2*1.1*4/ Tb]=2*(1.1*R/Tb+2*1.1/Tb)=2*1.1(R/Tb+2/Tb)

The possible channel bandwidth values for D2R are calculated in Table 1.

Table 2. The channel bandwidth of D2R

| Tb (μs) | R value | DSB (kHz) | CBW (kHz) | Tb (μs) | R value | DSB (kHz) | CBW (kHz) | Tb (μs) | R value | DSB (kHz) | CBW (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | 1 | 15 | 16.5 | 133.33 | 1 | 30 | 33 | 66.67 | 1 | 60 | 66 |
| 266.67 | 2 | 15 | 33 | 133.33 | 2 | 30 | 66 | 66.67 | 2 | 60 | 132 |
| 266.67 | 4 | 15 | 49.5 | 133.33 | 4 | 30 | 99 | 66.67 | 4 | 60 | 198 |
| 266.67 | 8 | 15 | 82.5 | 133.33 | 8 | 30 | 165 | 66.67 | 8 | 60 | 330 |
| 266.67 | 16 | 15 | 148.5 | 133.33 | 16 | 30 | 297 | 66.67 | 16 | 60 | 594 |
| 266.67 | 32 | 15 | 280.5 | 133.33 | 32 | 30 | 561 | 66.67 | 32 | 60 | 1121.9 |
| 266.67 | 64 | 15 | 544.5 | 133.33 | 64 | 30 | 1089 | 66.67 | 64 | 60 | 2177.9 |
| 266.67 | 128 | 15 | 1072.5 | 16.67 | 1 | 240 | 263.9 | 8.33 | 1 | 480 | 528.2 |
| 33.33 | 1 | 120 | 132 | 16.67 | 2 | 240 | 527.9 | 8.33 | 2 | 480 | 1056.4 |
| 33.33 | 2 | 120 | 264 | 16.67 | 4 | 240 | 1055.8 | 8.33 | 4 | 480 | 1584.6 |
| 33.33 | 4 | 120 | 396 | 16.67 | 8 | 240 | 1319.7 | 1.39 | 1 | 2880 | 3165.5 |
| 33.33 | 8 | 120 | 660.1 | 4.17 | 1 | 960 | 1055.2 |  |  |  |  |
| 33.33 | 16 | 120 | 1188.1 | 4.17 | 2 | 960 | 2110.3 |  |  |  |  |

Proposal 2: Use the following equations to define the CBW for D2R

When R=1, Channel bandwidth=4/Tb*(1+ SFO) =1.1*4/ Tb

When R>1, Channel bandwidth = 2*[(R/Tb)*(1+SFO)+ 1/2*1.1*4/ Tb]=2*(1.1*R/Tb+2*1.1/Tb)=2*1.1(R/Tb+2/Tb)

Proposal 3: Take the Table 1 to define the specific CBW for D2R.

Table 2. The channel bandwidth of D2R

| Tb (μs) | R value | CBW(kHz) | Tb (μs) | R value | CBW(kHz) | Tb (μs) | R value | CBW(kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | 1 | 16.5 | 133.33 | 1 | 33 | 66.67 | 1 | 66 |
| 266.67 | 2 | 33 | 133.33 | 2 | 66 | 66.67 | 2 | 132 |
| 266.67 | 4 | 49.5 | 133.33 | 4 | 99 | 66.67 | 4 | 198 |
| 266.67 | 8 | 82.5 | 133.33 | 8 | 165 | 66.67 | 8 | 330 |
| 266.67 | 16 | 148.5 | 133.33 | 16 | 297 | 66.67 | 16 | 594 |
| 266.67 | 32 | 280.5 | 133.33 | 32 | 561 | 66.67 | 32 | 1121.9 |
| 266.67 | 64 | 544.5 | 133.33 | 64 | 1089 | 66.67 | 64 | 2177.9 |
| 266.67 | 128 | 1072.5 | 16.67 | 1 | 263.9 | 8.33 | 1 | 528.2 |
| 33.33 | 1 | 132 | 16.67 | 2 | 527.9 | 8.33 | 2 | 1056.4 |
| 33.33 | 2 | 264 | 16.67 | 4 | 1055.8 | 8.33 | 4 | 1584.6 |
| 33.33 | 4 | 396 | 16.67 | 8 | 1319.7 | 1.39 | 1 | 3165.5 |
| 33.33 | 8 | 660.1 | 4.17 | 1 | 1055.2 |  |  |  |
| 33.33 | 16 | 1188.1 | 4.17 | 2 | 2110.3 |  |  |  |

Conclusions

In this paper, we provide our views on system parameters for ambient IoT, we have made the following proposals.

Proposal 1: Define R2D channel raster as 10 kHz, there is no need to define R2D channel raster offset value.

Proposal 2: Use the following equations to define the CBW for D2R

When R=1, Channel bandwidth=4/Tb*(1+ SFO) =1.1*4/ Tb

When R>1, Channel bandwidth = 2*[(R/Tb)*(1+SFO)+ 1/2*1.1*4/ Tb]=2*(1.1*R/Tb+2*1.1/Tb)=2*1.1(R/Tb+2/Tb)

Proposal 3: Take the Table 2 to define the specific CBW for D2R.

Table 2. The channel bandwidth of D2R

| Tb (μs) | R value | DSB (kHz) | CBW (kHz) | Tb (μs) | R value | DSB (kHz) | CBW (kHz) | Tb (μs) | R value | DSB (kHz) | CBW (kHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 266.67 | 1 | 15 | 16.5 | 133.33 | 1 | 30 | 33 | 66.67 | 1 | 60 | 66 |
| 266.67 | 2 | 15 | 33 | 133.33 | 2 | 30 | 66 | 66.67 | 2 | 60 | 132 |
| 266.67 | 4 | 15 | 49.5 | 133.33 | 4 | 30 | 99 | 66.67 | 4 | 60 | 198 |
| 266.67 | 8 | 15 | 82.5 | 133.33 | 8 | 30 | 165 | 66.67 | 8 | 60 | 330 |
| 266.67 | 16 | 15 | 148.5 | 133.33 | 16 | 30 | 297 | 66.67 | 16 | 60 | 594 |
| 266.67 | 32 | 15 | 280.5 | 133.33 | 32 | 30 | 561 | 66.67 | 32 | 60 | 1121.9 |
| 266.67 | 64 | 15 | 544.5 | 133.33 | 64 | 30 | 1089 | 66.67 | 64 | 60 | 2177.9 |
| 266.67 | 128 | 15 | 1072.5 | 16.67 | 1 | 240 | 263.9 | 8.33 | 1 | 480 | 528.2 |
| 33.33 | 1 | 120 | 132 | 16.67 | 2 | 240 | 527.9 | 8.33 | 2 | 480 | 1056.4 |
| 33.33 | 2 | 120 | 264 | 16.67 | 4 | 240 | 1055.8 | 8.33 | 4 | 480 | 1584.6 |
| 33.33 | 4 | 120 | 396 | 16.67 | 8 | 240 | 1319.7 | 1.39 | 1 | 2880 | 3165.5 |
| 33.33 | 8 | 120 | 660.1 | 4.17 | 1 | 960 | 1055.2 |  |  |  |  |
| 33.33 | 16 | 120 | 1188.1 | 4.17 | 2 | 960 | 2110.3 |  |  |  |  |

Reference

- R4-2508116, WF on requirements for system parameters and A-IoT device, CMCC, TSG RAN4 meeting #115